Table A

Candidate Technology –  Propoent 3GPP RIT (IMT2020/14)

# A. Candidate Technology – 3GPP RIT (IMT2020/14)

Sources

| 5GIA | ATIS | ChEG | CEG | WWRF | TCOE (INDIA) | 5GMF | TTA SPG33 | TPCEG | 5GIF | AEG | Bnrist | CIRAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5D/50 | 5D/55 | 5D/69 | 5D/90 | -NA- | 5D/121 | 5D/95 | 5D/49 | 5D/94 | 5D/27 | 5D/123 | -NA- | -NA- |

Note:  For the 5GIF column in the tables, information in [ ] is based on invited verbal interventions from 5GIF on the technical discussions related to the evaluation reports during  the WP 5D meeting #34 and is therefore indicated in [ ] format.   WP 5D will review the information and the [ ] status in Meeting #35.

#### 5.2.4.1 	Compliance template for services

TABLE A.1

|  | Service capability requirements | 5GIA | ATIS | ChEG | CEG | WWRF | TCOE (INDIA) | 5GMF | TTA SPG33 | TPCEG | 5GIF | AEG | Bnrist | CIRAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.2.4.1.1 | Support for wide range of services Is the proposal able to support a range of services across different usage scenarios (eMBB, URLLC, and mMTC)?: YES / NO Specify which usage scenarios (eMBB, URLLC, and mMTC) the candidate RIT or candidate SRIT can support.(1) | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes |  |  |  |

#### 5.2.4.2 	Compliance template for spectrum

TABLE A.2

|  | Spectrum capability requirements | 5GIA | ATIS | ChEG | CEG | WWRF | TCOE (INDIA) | 5GMF | TTA SPG33 | TPCEG | 5GIF | AEG | Bnrist | CIRAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.2.4.2.1 | Frequency bands identified for IMT Is the proposal able to utilize at least one frequency band identified for IMT in the ITU Radio Regulations?: YES / NO Specify in which band(s) the candidate RIT or candidate SRIT can be deployed. | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes |  |  |  |
| 5.2.4.2.2 | Higher Frequency range/band(s) Is the proposal able to utilize the higher frequency range/band(s) above 24.25 GHz?: YES / NO Specify in which band(s) the candidate RIT or candidate SRIT can be deployed. NOTE 1 – In the case of the candidate SRIT, at least one of the component RITs need to fulfil this requirement. | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes |  |  |  |

#### 5.2.4.3 	Compliance template for technical performance

TABLE A.3

| Minimum technical performance requirements item (5.2.4.3.x), units, and Report ITU-R M.2410-0 section reference(1) |  |  | Category |  |  |  |  |  |  | Required value |  |  | 5GIA |  |  | ATIS |  | ChEG |  | CEG |  | WWRF |  | TCOE (INDIA) |  | 5GMF |  | TTA SPG33 |  | TPCEG |  | 5GIF |  | AEG |  | Bnrist |  | CIRAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Usage scenario |  | Test environment |  |  | Downlink or uplink |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.1 Peak data rate (Gbit/s) (4.1) |  |  | eMBB |  | Not applicable |  |  | Downlink |  | 20 |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 10 |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
| 5.2.4.3.2 Peak spectral efficiency (bit/s/Hz) (4.2) |  |  | eMBB |  | Not applicable |  |  | Downlink |  | 30 |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 15 |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
| 5.2.4.3.3 User experienced data rate (Mbit/s) (4.3) |  |  | eMBB |  | Dense Urban – eMBB |  |  | Downlink |  | 100 |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | [Yes] |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 50 |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | [Yes] |  |  |  |  |  |  |
| 5.2.4.3.4 5th percentile user spectral efficiency (bit/s/Hz) (4.4) |  |  | eMBB |  | Indoor Hotspot – eMBB |  |  | Downlink |  | 0.3 |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | [Yes] |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 0.21 |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | [Yes] |  |  |  |  |  |  |
|  |  |  | eMBB |  | Dense Urban – eMBB |  |  | Downlink |  | 0.225 |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | [Yes] |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 0.15 |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | [Yes] |  |  |  |  |  |  |
|  |  |  | eMBB |  | Rural – eMBB |  |  | Downlink |  | 0.12 |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | [Yes] |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 0.045 |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | [Yes] |  |  |  |  |  |  |
| 5.2.4.3.5 Average spectral efficiency (bit/s/Hz/ TRxP) (4.5) |  |  | eMBB |  | Indoor Hotspot – eMBB |  |  | Downlink |  | 9 |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | [Yes] |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 6.75 |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | [Yes] |  |  |  |  |  |  |
|  |  |  | eMBB |  | Dense Urban – eMBB |  |  | Downlink |  | 7.8 |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | [Yes] |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 5.4 |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | [Yes] |  |  |  |  |  |  |
|  |  |  | eMBB |  | Rural – eMBB |  |  | Downlink |  | 3.3 |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | [Yes] |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | [Yes] |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 1.6 |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | [Yes] |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | [Yes] |  |  |  |  |  |  |
| 5.2.4.3.6 Area traffic capacity (Mbit/s/m2) (4.6) |  |  | eMBB |  | Indoor-Hotspot – eMBB |  |  | Downlink |  | 10 |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | [Yes] |  |  |  |  |  |  |
| 5.2.4.3.7 User plane latency (ms) (4.7.1) |  |  | eMBB |  | Not applicable |  |  | Uplink and Downlink |  | 4 |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | URLLC |  | Not applicable |  |  | Uplink and Downlink |  | 1 |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
| 5.2.4.3.8 Control plane latency (ms) (4.7.2) |  |  | eMBB |  | Not applicable |  |  | Not applicable |  | 20 |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | URLLC |  | Not applicable |  |  | Not applicable |  | 20 |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
| 5.2.4.3.9 Connection density (devices/km2) (4.8) |  |  | mMTC |  | Urban Macro – mMTC |  |  | Uplink |  | 1 000 000 |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
| 5.2.4.3.10 Energy efficiency (4.9) |  |  | eMBB |  | Not applicable |  |  | Not applicable |  | Capability to support a high sleep ratio and long sleep duration |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
| 5.2.4.3.11 Reliability (4.10) |  |  | URLLC |  | Urban Macro –URLLC |  |  | Uplink or Downlink |  | 1-10−5 success probability of transmitting a layer 2 PDU (protocol data unit) of size 32 bytes within 1 ms in channel quality of coverage edge |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
| 5.2.4.3.12 Mobility classes (4.11) |  |  | eMBB |  | Indoor Hotspot – eMBB |  |  | Uplink |  | Stationary, Pedestrian |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | eMBB |  | Dense Urban – eMBB |  |  | Uplink |  | Stationary, Pedestrian, Vehicular (up to 30 km/h) |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | eMBB |  | Rural – eMBB |  |  | Uplink |  | Pedestrian, Vehicular, High speed vehicular |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
| 5.2.4.3.13 Mobility Traffic channel link data rates (bit/s/Hz) (4.11) |  |  | eMBB |  | Indoor Hotspot – eMBB |  |  | Uplink |  | 1.5 (10 km/h) |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | eMBB |  | Dense Urban – eMBB |  |  | Uplink |  | 1.12 (30 km/h) |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | eMBB |  | Rural – eMBB |  |  | Uplink |  | 0.8 (120 km/h) |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 0.45 (500 km/h) |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
| 5.2.4.3.14 Mobility interruption time (ms) (4.12) |  |  | eMBB and URLLC |  | Not applicable |  |  | Not applicable |  | 0 |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
| 5.2.4.3.15 Bandwidth and Scalability (4.13) |  |  | Not applicable |  | Not applicable |  |  | Not applicable |  | At least 100 MHz |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Up to 1 GHz |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Support of multiple different bandwidth values(4) |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |

Table B

Candidate Technology – Propoent 3GPP SRIT (IMT2020/13)

# Candidate Technology – 3GPP SRIT (IMT2020/13)

Sources

| 5GIA | ATIS | ChEG | CEG | WWRF | TCOE (INDIA) | 5GMF | TTA SPG33 | TPCEG | 5GIF | AEG | Bnrist | CIRAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5D/50 | 5D/55 | 5D/69 | 5D/90 | -NA- | -NA- | 5D/96 | -NA- | 5D/94 | 5D/27 | 5D/123 | -NA- | -NA- |

#### 5.2.4.1 	Compliance template for services (both RIT components)

TABLE B.1

|  | Service capability requirements | 5GIA | ATIS | ChEG | CEG | WWRF | TCOE (INDIA) | 5GMF | TTA SPG33 | TPCEG | 5GIF | AEG | Bnrist | CIRAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.2.4.1.1 | Support for wide range of services Is the proposal able to support a range of services across different usage scenarios (eMBB, URLLC, and mMTC)?: YES / NO Specify which usage scenarios (eMBB, URLLC, and mMTC) the candidate RIT or candidate SRIT can support.(1) | Yes | Yes | Yes | Yes |  |  | Yes |  |  | Yes |  |  |  |

#### 5.2.4.2 	Compliance template for spectrum  (both RIT components)

TABLE B.2

|  | Spectrum capability requirements | 5GIA | ATIS | ChEG | CEG | WWRF | TCOE (INDIA) | 5GMF | TTA SPG33 | TPCEG | 5GIF | AEG | Bnrist | CIRAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.2.4.2.1 | Frequency bands identified for IMT Is the proposal able to utilize at least one frequency band identified for IMT in the ITU Radio Regulations?: YES / NO Specify in which band(s) the candidate RIT or candidate SRIT can be deployed. | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |
| 5.2.4.2.2 | Higher Frequency range/band(s) Is the proposal able to utilize the higher frequency range/band(s) above 24.25 GHz?: YES / NO Specify in which band(s) the candidate RIT or candidate SRIT can be deployed. NOTE 1 – In the case of the candidate SRIT, at least one of the component RITs need to fulfil this requirement. | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |

#### 5.2.4.3 	Compliance template for technical performance (SRIT both RIT components)

TABLE B.3a

| Minimum technical performance requirements item (5.2.4.3.x), units, and Report ITU-R M.2410-0 section reference(1) |  |  | Category |  |  |  |  |  |  | Required value |  |  | 5GIA |  |  | ATIS |  | ChEG |  | CEG |  | WWRF |  | TCOE (INDIA) |  | 5GMF |  | TTA SPG33 |  | TPCEG |  | 5GIF |  | AEG |  | Bnrist |  | CIRAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Usage scenario |  | Test environment |  |  | Downlink or uplink |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.1 Peak data rate (Gbit/s) (4.1) |  |  | eMBB |  | Not applicable |  |  | Downlink |  | 20 |  |  | Yes |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 10 |  |  | Yes |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.2 Peak spectral efficiency (bit/s/Hz) (4.2) |  |  | eMBB |  | Not applicable |  |  | Downlink |  | 30 |  |  | Yes |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 15 |  |  | Yes |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.3 User experienced data rate (Mbit/s) (4.3) |  |  | eMBB |  | Dense Urban – eMBB |  |  | Downlink |  | 100 |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 50 |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.4 5th percentile user spectral efficiency (bit/s/Hz) (4.4) |  |  | eMBB |  | Indoor Hotspot – eMBB |  |  | Downlink |  | 0.3 |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 0.21 |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Dense Urban – eMBB |  |  | Downlink |  | 0.225 |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 0.15 |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Rural – eMBB |  |  | Downlink |  | 0.12 |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 0.045 |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.5 Average spectral efficiency (bit/s/Hz/ TRxP) (4.5) |  |  | eMBB |  | Indoor Hotspot – eMBB |  |  | Downlink |  | 9 |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 6.75 |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Dense Urban – eMBB |  |  | Downlink |  | 7.8 |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 5.4 |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Rural – eMBB |  |  | Downlink |  | 3.3 |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 1.6 |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.6 Area traffic capacity (Mbit/s/m2) (4.6) |  |  | eMBB |  | Indoor-Hotspot – eMBB |  |  | Downlink |  | 10 |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.7 User plane latency (ms) (4.7.1) |  |  | eMBB |  | Not applicable |  |  | Uplink and Downlink |  | 4 |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | URLLC |  | Not applicable |  |  | Uplink and Downlink |  | 1 |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.8 Control plane latency (ms) (4.7.2) |  |  | eMBB |  | Not applicable |  |  | Not applicable |  | 20 |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | URLLC |  | Not applicable |  |  | Not applicable |  | 20 |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.9 Connection density (devices/km2) (4.8) |  |  | mMTC |  | Urban Macro – mMTC |  |  | Uplink |  | 1 000 000 |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |
| 5.2.4.3.10 Energy efficiency (4.9) |  |  | eMBB |  | Not applicable |  |  | Not applicable |  | Capability to support a high sleep ratio and long sleep duration |  |  | Yes |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.11 Reliability (4.10) |  |  | URLLC |  | Urban Macro –URLLC |  |  | Uplink or Downlink |  | 1-10−5 success probability of transmitting a layer 2 PDU (protocol data unit) of size 32 bytes within 1 ms in channel quality of coverage edge |  |  | Yes |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.12 Mobility classes (4.11) |  |  | eMBB |  | Indoor Hotspot – eMBB |  |  | Uplink |  | Stationary, Pedestrian |  |  | Yes |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Dense Urban – eMBB |  |  | Uplink |  | Stationary, Pedestrian, Vehicular (up to 30 km/h) |  |  | Yes |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Rural – eMBB |  |  | Uplink |  | Pedestrian, Vehicular, High speed vehicular |  |  | Yes |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.13 Mobility Traffic channel link data rates (bit/s/Hz) (4.11) |  |  | eMBB |  | Indoor Hotspot – eMBB |  |  | Uplink |  | 1.5 (10 km/h) |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Dense Urban – eMBB |  |  | Uplink |  | 1.12 (30 km/h) |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Rural – eMBB |  |  | Uplink |  | 0.8 (120 km/h) |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 0.45 (500 km/h) |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.14 Mobility interruption time (ms) (4.12) |  |  | eMBB and URLLC |  | Not applicable |  |  | Not applicable |  | 0 |  |  | Yes |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.15 Bandwidth and Scalability (4.13) |  |  | Not applicable |  | Not applicable |  |  | Not applicable |  | At least 100 MHz |  |  | Yes |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Up to 1 GHz |  |  | Yes |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Support of multiple different bandwidth values(4) |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 5.2.4.3 	Compliance template for technical performance (SRIT – NR Component)

TABLE B.3b

Refer to Table A.3  for candidate technology – 3GPP RIT (IMT 2020/14)

#### 5.2.4.3 	Compliance template for technical performance (SRIT – LTE Component)

TABLE B.3c

| Minimum technical performance requirements item (5.2.4.3.x), units, and Report ITU-R M.2410-0 section reference(1) |  |  | Category |  |  |  |  |  |  | Required value |  |  | 5GIA |  |  | ATIS |  | ChEG |  | CEG |  | WWRF |  | TCOE (INDIA) |  | 5GMF |  | TTA SPG33 |  | TPCEG |  | 5GIF |  | AEG |  | Bnrist |  | CIRAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Usage scenario |  | Test environment |  |  | Downlink or uplink |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.1 Peak data rate (Gbit/s) (4.1) |  |  | eMBB |  | Not applicable |  |  | Downlink |  | 20 |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 10 |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 5.2.4.3.2 Peak spectral efficiency (bit/s/Hz) (4.2) |  |  | eMBB |  | Not applicable |  |  | Downlink |  | 30 |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 15 |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 5.2.4.3.3 User experienced data rate (Mbit/s) (4.3) |  |  | eMBB |  | Dense Urban – eMBB |  |  | Downlink |  | 100 |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 50 |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 5.2.4.3.4 5th percentile user spectral efficiency (bit/s/Hz) (4.4) |  |  | eMBB |  | Indoor Hotspot – eMBB |  |  | Downlink |  | 0.3 |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 0.21 |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Dense Urban – eMBB |  |  | Downlink |  | 0.225 |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 0.15 |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Rural – eMBB |  |  | Downlink |  | 0.12 |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 0.045 |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 5.2.4.3.5 Average spectral efficiency (bit/s/Hz/ TRxP) (4.5) |  |  | eMBB |  | Indoor Hotspot – eMBB |  |  | Downlink |  | 9 |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 6.75 |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Dense Urban – eMBB |  |  | Downlink |  | 7.8 |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 5.4 |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Rural – eMBB |  |  | Downlink |  | 3.3 |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 1.6 |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 5.2.4.3.6 Area traffic capacity (Mbit/s/m2) (4.6) |  |  | eMBB |  | Indoor-Hotspot – eMBB |  |  | Downlink |  | 10 |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 5.2.4.3.7 User plane latency (ms) (4.7.1) |  |  | eMBB |  | Not applicable |  |  | Uplink and Downlink |  | 4 |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | URLLC |  | Not applicable |  |  | Uplink and Downlink |  | 1 |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 5.2.4.3.8 Control plane latency (ms) (4.7.2) |  |  | eMBB |  | Not applicable |  |  | Not applicable |  | 20 |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | URLLC |  | Not applicable |  |  | Not applicable |  | 20 |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 5.2.4.3.9 Connection density (devices/km2) (4.8) |  |  | mMTC |  | Urban Macro – mMTC |  |  | Uplink |  | 1 000 000 |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 5.2.4.3.10 Energy efficiency (4.9) |  |  | eMBB |  | Not applicable |  |  | Not applicable |  | Capability to support a high sleep ratio and long sleep duration |  |  | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 5.2.4.3.11 Reliability (4.10) |  |  | URLLC |  | Urban Macro –URLLC |  |  | Uplink or Downlink |  | 1-10−5 success probability of transmitting a layer 2 PDU (protocol data unit) of size 32 bytes within 1 ms in channel quality of coverage edge |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.12 Mobility classes (4.11) |  |  | eMBB |  | Indoor Hotspot – eMBB |  |  | Uplink |  | Stationary, Pedestrian |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Dense Urban – eMBB |  |  | Uplink |  | Stationary, Pedestrian, Vehicular (up to 30 km/h) |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Rural – eMBB |  |  | Uplink |  | Pedestrian, Vehicular, High speed vehicular |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 5.2.4.3.13 Mobility Traffic channel link data rates (bit/s/Hz) (4.11) |  |  | eMBB |  | Indoor Hotspot – eMBB |  |  | Uplink |  | 1.5 (10 km/h) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Dense Urban – eMBB |  |  | Uplink |  | 1.12 (30 km/h) |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Rural – eMBB |  |  | Uplink |  | 0.8 (120 km/h) |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 0.45 (500 km/h) |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 5.2.4.3.14 Mobility interruption time (ms) (4.12) |  |  | eMBB and URLLC |  | Not applicable |  |  | Not applicable |  | 0 |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |
| 5.2.4.3.15 Bandwidth and Scalability (4.13) |  |  | Not applicable |  | Not applicable |  |  | Not applicable |  | At least 100 MHz |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Up to 1 GHz |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Support of multiple different bandwidth values(4) |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |

Table C

Candidate Technology –  Proponent China RIT (IMT2020/15)

C. Candidate Technology – China RIT (IMT2020/15)

Refer to compliance template in TABLE A.3 and TABLE B.3c, noting that the followings (see ATTACHMENT 1 to IMT-2020/24);

- WP 5D has concluded that NR part in Document IMT-2020/15 and Document IMT-2020/14 are technically identical submissions, therefore a single evaluation is applicable to both.

- WP 5D has concluded that NB-IoT part of Document IMT-2020/15 and NB-IoT feature in Document IMT-2020/13 are technically identical, therefore a single evaluation on NB-IoT is applicable to both.

Table D

Candidate Technology –  Proponent Korea RIT (IMT2020/16)

# D. Candidate Technology – Korea RIT (IMT2020/16)

Refer to compliance template in TABLE A.3, noting that WP 5D has concluded that Documents IMT-2020/16 and IMT-2020/14 are technically identical submissions,therefore a single evaluation is applicable to both (see ATTACHMENT 1 to IMT-2020/25).

Table E

Candidate Technology –  Proponent ETSI (TC DECT) & DECT Forum SRIT (IMT2020/17(Rev.1))

# E. Candidate Technology – DECT SRIT (IMT2020/17)

Sources

| 5GIA | ATIS | ChEG | CEG | WWRF | TCOE (INDIA) | 5GMF | TTA SPG33 | TPCEG | 5GIF | AEG | Bnrist | CIRAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5D/50 | 5D/60 | 5D/69 | 5D/90 | -NA- | -NA- | -NA- | -NA- | -NA- | 5D/27 | -NA- | -NA- | -NA- |

Note:  For the 5GIF column in the tables, information in [ ] is based on invited verbal interventions from 5GIF on the technical discussions related to the evaluation reports during  the WP 5D meeting #34 and is therefore indicated in [ ] format.   WP 5D will review the information and the [ ] status in Meeting #35.

#### 5.2.4.1 	Compliance template for services (both RIT components)

TABLE E.1

|  | Service capability requirements | 5GIA | ATIS | ChEG | CEG | WWRF | TCOE (INDIA) | 5GMF | TTA SPG33 | TPCEG | 5GIF | AEG | Bnrist | CIRAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.2.4.1.1 | Support for wide range of services Is the proposal able to support a range of services across different usage scenarios (eMBB, URLLC, and mMTC)?: YES / NO Specify which usage scenarios (eMBB, URLLC, and mMTC) the candidate RIT or candidate SRIT can support.(1) | Inconclusive (DECT 2020 NR RIT component) |  | Inconclusive (DECT 2020 NR RIT component) | Inconclusive (DECT 2020 NR RIT component) |  |  |  |  |  | [Inconclusive (DECT2020 NR component)] |  |  |  |

#### 5.2.4.2 	Compliance template for spectrum  (both RIT components)

TABLE E.2

|  | Spectrum capability requirements | 5GIA | ATIS | ChEG | CEG | WWRF | TCOE (INDIA) | 5GMF | TTA SPG33 | TPCEG | 5GIF | AEG | Bnrist | CIRAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.2.4.2.1 | Frequency bands identified for IMT Is the proposal able to utilize at least one frequency band identified for IMT in the ITU Radio Regulations?: YES / NO Specify in which band(s) the candidate RIT or candidate SRIT can be deployed. |  |  |  | Yes (DECT 2020 NR RIT component) |  |  |  |  |  | [Yes (DECT2020 NR component)] |  |  |  |
| 5.2.4.2.2 | Higher Frequency range/band(s) Is the proposal able to utilize the higher frequency range/band(s) above 24.25 GHz?: YES / NO Specify in which band(s) the candidate RIT or candidate SRIT can be deployed. NOTE 1 – In the case of the candidate SRIT, at least one of the component RITs need to fulfil this requirement. |  |  |  | Yes (DECT 2020 NR RIT component) |  |  |  |  |  | [Inconclusive (DECT2020 NR component)] |  |  |  |

No Compliance template for the SRIT submission provided

#### 5.2.4.3 	Compliance template for technical performance (SRIT)

TABLE  E.3a

#### 5.2.4.3 	Compliance template for technical performance (3GPP NR RIT component)

TABLE E.3c

Refer to Table A.3  for candidate technology – 3GPP RIT (IMT 2020/14)

#### 5.2.4.3 	Compliance template for technical performance (DECT 2020 NR RIT component)

TABLE  E.3c

| Minimum technical performance requirements item (5.2.4.3.x), units, and Report ITU-R M.2410-0 section reference(1) |  |  | Category |  |  |  |  |  |  | Required value |  |  | 5GIA |  |  | ATIS |  | ChEG |  | CEG |  | WWRF |  | TCOE (INDIA) |  | 5GMF |  | TTA SPG33 |  | TPCEG |  | 5GIF |  | AEG |  | Bnrist |  | CIRAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Usage scenario |  | Test environment |  |  | Downlink or uplink |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.1 Peak data rate (Gbit/s) (4.1) |  |  | eMBB |  | Not applicable |  |  | Downlink |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.2 Peak spectral efficiency (bit/s/Hz) (4.2) |  |  | eMBB |  | Not applicable |  |  | Downlink |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.3 User experienced data rate (Mbit/s) (4.3) |  |  | eMBB |  | Dense Urban – eMBB |  |  | Downlink |  | 100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.4 5th percentile user spectral efficiency (bit/s/Hz) (4.4) |  |  | eMBB |  | Indoor Hotspot – eMBB |  |  | Downlink |  | 0.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 0.21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Dense Urban – eMBB |  |  | Downlink |  | 0.225 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 0.15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Rural – eMBB |  |  | Downlink |  | 0.12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 0.045 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.5 Average spectral efficiency (bit/s/Hz/ TRxP) (4.5) |  |  | eMBB |  | Indoor Hotspot – eMBB |  |  | Downlink |  | 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 6.75 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Dense Urban – eMBB |  |  | Downlink |  | 7.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 5.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Rural – eMBB |  |  | Downlink |  | 3.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 1.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.6 Area traffic capacity (Mbit/s/m2) (4.6) |  |  | eMBB |  | Indoor-Hotspot – eMBB |  |  | Downlink |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.7 User plane latency (ms) (4.7.1) |  |  | eMBB |  | Not applicable |  |  | Uplink and Downlink |  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | URLLC |  | Not applicable |  |  | Uplink and Downlink |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | [No] |  |  |  |  |  |  |
| 5.2.4.3.8 Control plane latency (ms) (4.7.2) |  |  | eMBB |  | Not applicable |  |  | Not applicable |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | URLLC |  | Not applicable |  |  | Not applicable |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | [Yes] |  |  |  |  |  |  |
| 5.2.4.3.9 Connection density (devices/km2) (4.8) |  |  | mMTC |  | Urban Macro – mMTC |  |  | Uplink |  | 1 000 000 |  |  |  |  |  |  |  |  |  | Inconclusive |  |  |  |  |  |  |  |  |  |  |  | [Inconclusive] |  |  |  |  |  |  |
| 5.2.4.3.10 Energy efficiency (4.9) |  |  | eMBB |  | Not applicable |  |  | Not applicable |  | Capability to support a high sleep ratio and long sleep duration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.11 Reliability (4.10) |  |  | URLLC |  | Urban Macro –URLLC |  |  | Uplink or Downlink |  | 1-10−5 success probability of transmitting a layer 2 PDU (protocol data unit) of size 32 bytes within 1 ms in channel quality of coverage edge |  |  | NO |  |  |  |  | NO |  | Yes, Yes |  |  |  |  |  |  |  |  |  |  |  | [No] |  |  |  |  |  |  |
| 5.2.4.3.12 Mobility classes (4.11) |  |  | eMBB |  | Indoor Hotspot – eMBB |  |  | Uplink |  | Stationary, Pedestrian |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Dense Urban – eMBB |  |  | Uplink |  | Stationary, Pedestrian, Vehicular (up to 30 km/h) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Rural – eMBB |  |  | Uplink |  | Pedestrian, Vehicular, High speed vehicular |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.13 Mobility Traffic channel link data rates (bit/s/Hz) (4.11) |  |  | eMBB |  | Indoor Hotspot – eMBB |  |  | Uplink |  | 1.5 (10 km/h) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Dense Urban – eMBB |  |  | Uplink |  | 1.12 (30 km/h) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Rural – eMBB |  |  | Uplink |  | 0.8 (120 km/h) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 0.45 (500 km/h) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.14 Mobility interruption time (ms) (4.12) |  |  | eMBB and URLLC |  | Not applicable |  |  | Not applicable |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | [inconlusive, DECT2020, URLLC ] |  |  |  |  |  |  |
| 5.2.4.3.15 Bandwidth and Scalability (4.13) |  |  | Not applicable |  | Not applicable |  |  | Not applicable |  | At least 100 MHz |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | [No] |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Up to 1 GHz |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | [No] |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Support of multiple different bandwidth values(4) |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | [Yes] |  |  |  |  |  |  |

Table F

Candidate Technology –  Proponent Nufront RIT (IMT2020/18(Rev.1))

# F. Candidate Technology – EUHT RIT (IMT2020/18)

Sources

| 5GIA | ATIS | ChEG | CEG | WWRF | TCOE (INDIA) | 5GMF | TTA SPG33 | TPCEG | 5GIF | AEG | Bnrist | CIRAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5D/50 | NA- | -NA- | 5D/90 | 5D/120 | NA- | 5D/97 | NA- | NA- | -NA- | 5D/125 | 5D/67 | 5D/129 |

Note:  For the 5GIF column in the tables, information in [ ] is based on invited verbal interventions from 5GIF on the technical discussions related to the evaluation reports during  the WP 5D meeting #34 and is therefore indicated in [ ] format.   WP 5D will review the information and the [ ] status in Meeting #35.

#### 5.2.4.1 Compliance template for services

TABLE F.1

|  | Service capability requirements | 5GIA | ATIS | ChEG | CEG | WWRF | TCOE (INDIA) | 5GMF | TTA SPG33 | TPCEG | 5GIF | AEG | Bnrist | CIRAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.2.4.1.1 | Support for wide range of services Is the proposal able to support a range of services across different usage scenarios (eMBB, URLLC, and mMTC)?: YES / NO Specify which usage scenarios (eMBB, URLLC, and mMTC) the candidate RIT or candidate SRIT can support.(1) | NO |  |  | Inconclusive |  |  | NO |  |  | [NO] |  | YES | No |

#### 5.2.4.2 	Compliance template for spectrum

TABLE F.2

|  | Spectrum capability requirements | 5GIA | ATIS | ChEG | CEG | WWRF | TCOE (INDIA) | 5GMF | TTA SPG33 | TPCEG | 5GIF | AEG | Bnrist | CIRAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.2.4.2.1 | Frequency bands identified for IMT Is the proposal able to utilize at least one frequency band identified for IMT in the ITU Radio Regulations?: YES / NO Specify in which band(s) the candidate RIT or candidate SRIT can be deployed. |  |  |  | Yes |  |  |  |  |  | [Inconclusive] |  | YES | Inconclusive |
| 5.2.4.2.2 | Higher Frequency range/band(s) Is the proposal able to utilize the higher frequency range/band(s) above 24.25 GHz?: YES / NO Specify in which band(s) the candidate RIT or candidate SRIT can be deployed. NOTE 1 – In the case of the candidate SRIT, at least one of the component RITs need to fulfil this requirement. |  |  |  | Yes |  |  |  |  |  | [Inconclusive] |  | YES | Inconclusive |

#### 5.2.4.3 	Compliance template for technical performance

TABLE F.3

| Minimum technical performance requirements item (5.2.4.3.x), units, and Report ITU-R M.2410-0 section reference(1) |  |  | Category |  |  |  |  |  |  | Required value |  |  | 5GIA |  |  | ATIS |  | ChEG |  | CEG |  | WWRF |  | TCOE (INDIA) |  | 5GMF |  | TTA SPG33 |  | TPCEG |  | 5GIF |  | AEG |  | Bnrist |  | CIRAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Usage scenario |  | Test environment |  |  | Downlink or uplink |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.1 Peak data rate (Gbit/s) (4.1) |  |  | eMBB |  | Not applicable |  |  | Downlink |  | 20 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | [No] |  | Yes |  | Yes |  | No |
|  |  |  |  |  |  |  |  | Uplink |  | 10 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | [No] |  | Yes |  | Yes |  | No |
| 5.2.4.3.2 Peak spectral efficiency (bit/s/Hz) (4.2) |  |  | eMBB |  | Not applicable |  |  | Downlink |  | 30 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | [No] |  | Yes |  | Yes |  | Yes |
|  |  |  |  |  |  |  |  | Uplink |  | 15 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |
| 5.2.4.3.3 User experienced data rate (Mbit/s) (4.3) |  |  | eMBB |  | Dense Urban – eMBB |  |  | Downlink |  | 100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | [No] |  | Yes |  | Yes |  | No |
|  |  |  |  |  |  |  |  | Uplink |  | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | [No] |  | Yes |  | Yes |  | No |
| 5.2.4.3.4 5th percentile user spectral efficiency (bit/s/Hz) (4.4) |  |  | eMBB |  | Indoor Hotspot – eMBB |  |  | Downlink |  | 0.3 |  |  | No |  |  |  |  |  |  | Inconclusive |  |  |  |  |  | No |  |  |  |  |  | [No] |  |  |  | Yes |  | No |
|  |  |  |  |  |  |  |  | Uplink |  | 0.21 |  |  | No |  |  |  |  |  |  | Inconclusive |  |  |  |  |  | No |  |  |  |  |  | [No] |  |  |  | Yes |  | No |
|  |  |  | eMBB |  | Dense Urban – eMBB |  |  | Downlink |  | 0.225 |  |  | Yes |  |  |  |  |  |  | Inconclusive |  |  |  |  |  | Yes |  |  |  |  |  | [Yes] |  |  |  | Yes |  | Yes |
|  |  |  |  |  |  |  |  | Uplink |  | 0.15 |  |  | No |  |  |  |  |  |  | Inconclusive |  |  |  |  |  | No |  |  |  |  |  | [No] |  |  |  | Yes |  | No |
|  |  |  | eMBB |  | Rural – eMBB |  |  | Downlink |  | 0.12 |  |  |  |  |  |  |  |  |  | Inconclusive |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 0.045 |  |  | No |  |  |  |  |  |  | Inconclusive |  |  |  |  |  | No |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.2.4.3.5 Average spectral efficiency (bit/s/Hz/ TRxP) (4.5) |  |  | eMBB |  | Indoor Hotspot – eMBB |  |  | Downlink |  | 9 |  |  | No |  |  |  |  |  |  | Inconclusive |  |  |  |  |  | No |  |  |  |  |  | [No] |  |  |  | Yes |  | No |
|  |  |  |  |  |  |  |  | Uplink |  | 6.75 |  |  | No |  |  |  |  |  |  | Inconclusive |  |  |  |  |  | No |  |  |  |  |  | [No] |  |  |  | Yes |  | No |
|  |  |  | eMBB |  | Dense Urban – eMBB |  |  | Downlink |  | 7.8 |  |  | No |  |  |  |  |  |  | Inconclusive |  |  |  |  |  | No |  |  |  |  |  | [No] |  |  |  | Yes |  | No |
|  |  |  |  |  |  |  |  | Uplink |  | 5.4 |  |  | No |  |  |  |  |  |  | Inconclusive |  |  |  |  |  | No |  |  |  |  |  | [No] |  |  |  | Yes |  | No |
|  |  |  | eMBB |  | Rural – eMBB |  |  | Downlink |  | 3.3 |  |  |  |  |  |  |  |  |  | Inconclusive |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Inconclusive |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 1.6 |  |  | Yes (Config B) |  |  |  |  |  |  | Inconclusive |  |  |  |  |  | Yes (Config B) |  |  |  |  |  |  |  |  |  | Yes |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Inconclusive |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.2.4.3.6 Area traffic capacity (Mbit/s/m2) (4.6) |  |  | eMBB |  | Indoor-Hotspot – eMBB |  |  | Downlink |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | [No] |  | No |  | Yes |  | No |
| 5.2.4.3.7 User plane latency (ms) (4.7.1) |  |  | eMBB |  | Not applicable |  |  | Uplink and Downlink |  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No |  | Yes |  |  |
|  |  |  | URLLC |  | Not applicable |  |  | Uplink and Downlink |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No |  | Yes |  |  |
| 5.2.4.3.8 Control plane latency (ms) (4.7.2) |  |  | eMBB |  | Not applicable |  |  | Not applicable |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |
|  |  |  | URLLC |  | Not applicable |  |  | Not applicable |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |
| 5.2.4.3.9 Connection density (devices/km2) (4.8) |  |  | mMTC |  | Urban Macro – mMTC |  |  | Uplink |  | 1 000 000 |  |  |  |  |  |  |  |  |  | Inconclusive |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.2.4.3.10 Energy efficiency (4.9) |  |  | eMBB |  | Not applicable |  |  | Not applicable |  | Capability to support a high sleep ratio and long sleep duration |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | No |
| 5.2.4.3.11 Reliability (4.10) |  |  | URLLC |  | Urban Macro –URLLC |  |  | Uplink or Downlink |  | 1-10−5 success probability of transmitting a layer 2 PDU (protocol data unit) of size 32 bytes within 1 ms in channel quality of coverage edge |  |  | No |  |  |  |  |  |  | Inconclusive |  | No |  |  |  | No, No |  |  |  |  |  | [No] |  |  |  | Yes |  | No,No |
| 5.2.4.3.12 Mobility classes (4.11) |  |  | eMBB |  | Indoor Hotspot – eMBB |  |  | Uplink |  | Stationary, Pedestrian |  |  |  |  |  |  |  |  |  | Inconclusive |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
|  |  |  | eMBB |  | Dense Urban – eMBB |  |  | Uplink |  | Stationary, Pedestrian, Vehicular (up to 30 km/h) |  |  |  |  |  |  |  |  |  | Inconclusive |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
|  |  |  | eMBB |  | Rural – eMBB |  |  | Uplink |  | Pedestrian, Vehicular, High speed vehicular |  |  |  |  |  |  |  |  |  | Inconclusive |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.2.4.3.13 Mobility Traffic channel link data rates (bit/s/Hz) (4.11) |  |  | eMBB |  | Indoor Hotspot – eMBB |  |  | Uplink |  | 1.5 (10 km/h) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
|  |  |  | eMBB |  | Dense Urban – eMBB |  |  | Uplink |  | 1.12 (30 km/h) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
|  |  |  | eMBB |  | Rural – eMBB |  |  | Uplink |  | 0.8 (120 km/h) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
|  |  |  |  |  |  |  |  |  |  | 0.45 (500 km/h) |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |
| 5.2.4.3.14 Mobility interruption time (ms) (4.12) |  |  | eMBB and URLLC |  | Not applicable |  |  | Not applicable |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | [Inconclusive] |  | Yes |  | Yes |  | No |
| 5.2.4.3.15 Bandwidth and Scalability (4.13) |  |  | Not applicable |  | Not applicable |  |  | Not applicable |  | At least 100 MHz |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | [Yes] |  |  |  | Yes |  |  |
|  |  |  |  |  |  |  |  |  |  | Up to 1 GHz |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | [No] |  |  |  | Yes |  |  |
|  |  |  |  |  |  |  |  |  |  | Support of multiple different bandwidth values(4) |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | [Yes] |  |  |  | Yes |  |  |

Table G

Candidate Technology –  Proponent TSDSI RIT (IMT2020/19(Rev.1))

# G. Candidate Technology – TSDSI RIT (IMT2020/19)

Sources

| 5GIA | ATIS | ChEG | CEG | WWRF | TCOE (INDIA) | 5GMF | TTA SPG33 | TPCEG | 5GIF | AEG | Bnrist | CIRAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5D/50 | 5D/61 | -NA- | 5D/90 | 5D/120 | 5D/122 | -NA- | -NA- | 5D/94 | -NA- | 5D/124 | -NA- | -NA- |

#### 5.2.4.1 	Compliance template for services

TABLE G.1

|  | Service capability requirements | 5GIA | ATIS | ChEG | CEG | WWRF | TCOE (INDIA) | 5GMF | TTA SPG33 | TPCEG | 5GIF | AEG | Bnrist | CIRAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.2.4.1.1 | Support for wide range of services Is the proposal able to support a range of services across different usage scenarios (eMBB, URLLC, and mMTC)?: YES / NO Specify which usage scenarios (eMBB, URLLC, and mMTC) the candidate RIT or candidate SRIT can support.(1) |  |  |  |  |  | Yes |  |  |  |  |  |  |  |

#### 5.2.4.2 	Compliance template for spectrum

TABLE G.2

|  | Spectrum capability requirements | 5GIA | ATIS | ChEG | CEG | WWRF | TCOE (INDIA) | 5GMF | TTA SPG33 | TPCEG | 5GIF | AEG | Bnrist | CIRAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.2.4.2.1 | Frequency bands identified for IMT Is the proposal able to utilize at least one frequency band identified for IMT in the ITU Radio Regulations?: YES / NO Specify in which band(s) the candidate RIT or candidate SRIT can be deployed. |  |  |  |  |  | Yes |  |  |  |  |  |  |  |
| 5.2.4.2.2 | Higher Frequency range/band(s) Is the proposal able to utilize the higher frequency range/band(s) above 24.25 GHz?: YES / NO Specify in which band(s) the candidate RIT or candidate SRIT can be deployed. NOTE 1 – In the case of the candidate SRIT, at least one of the component RITs need to fulfil this requirement. |  |  |  |  |  | Yes |  |  |  |  |  |  |  |

#### 5.2.4.3 	Compliance template for technical performance

TABLE G.3

| Minimum technical performance requirements item (5.2.4.3.x), units, and Report ITU-R M.2410-0 section reference(1) |  |  | Category |  |  |  |  |  |  | Required value |  |  | 5GIA |  |  | ATIS |  | ChEG |  | CEG |  | WWRF |  | TCOE (INDIA) |  | 5GMF |  | TTA SPG33 |  | TPCEG |  | 5GIF |  | AEG |  | Bnrist |  | CIRAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Usage scenario |  | Test environment |  |  | Downlink or uplink |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.1 Peak data rate (Gbit/s) (4.1) |  |  | eMBB |  | Not applicable |  |  | Downlink |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.2 Peak spectral efficiency (bit/s/Hz) (4.2) |  |  | eMBB |  | Not applicable |  |  | Downlink |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.3 User experienced data rate (Mbit/s) (4.3) |  |  | eMBB |  | Dense Urban – eMBB |  |  | Downlink |  | 100 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.4 5th percentile user spectral efficiency (bit/s/Hz) (4.4) |  |  | eMBB |  | Indoor Hotspot – eMBB |  |  | Downlink |  | 0.3 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 0.21 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Dense Urban – eMBB |  |  | Downlink |  | 0.225 |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 0.15 |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
|  |  |  | eMBB |  | Rural – eMBB |  |  | Downlink |  | 0.12 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 0.045 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.5 Average spectral efficiency (bit/s/Hz/ TRxP) (4.5) |  |  | eMBB |  | Indoor Hotspot – eMBB |  |  | Downlink |  | 9 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 6.75 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Dense Urban – eMBB |  |  | Downlink |  | 7.8 |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 5.4 |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
|  |  |  | eMBB |  | Rural – eMBB |  |  | Downlink |  | 3.3 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Uplink |  | 1.6 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| 5.2.4.3.6 Area traffic capacity (Mbit/s/m2) (4.6) |  |  | eMBB |  | Indoor-Hotspot – eMBB |  |  | Downlink |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.7 User plane latency (ms) (4.7.1) |  |  | eMBB |  | Not applicable |  |  | Uplink and Downlink |  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | URLLC |  | Not applicable |  |  | Uplink and Downlink |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.8 Control plane latency (ms) (4.7.2) |  |  | eMBB |  | Not applicable |  |  | Not applicable |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | URLLC |  | Not applicable |  |  | Not applicable |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.9 Connection density (devices/km2) (4.8) |  |  | mMTC |  | Urban Macro – mMTC |  |  | Uplink |  | 1 000 000 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.10 Energy efficiency (4.9) |  |  | eMBB |  | Not applicable |  |  | Not applicable |  | Capability to support a high sleep ratio and long sleep duration |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.11 Reliability (4.10) |  |  | URLLC |  | Urban Macro –URLLC |  |  | Uplink or Downlink |  | 1-10−5 success probability of transmitting a layer 2 PDU (protocol data unit) of size 32 bytes within 1 ms in channel quality of coverage edge |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.12 Mobility classes (4.11) |  |  | eMBB |  | Indoor Hotspot – eMBB |  |  | Uplink |  | Stationary, Pedestrian |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Dense Urban – eMBB |  |  | Uplink |  | Stationary, Pedestrian, Vehicular (up to 30 km/h) |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Rural – eMBB |  |  | Uplink |  | Pedestrian, Vehicular, High speed vehicular |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.13 Mobility Traffic channel link data rates (bit/s/Hz) (4.11) |  |  | eMBB |  | Indoor Hotspot – eMBB |  |  | Uplink |  | 1.5 (10 km/h) |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Dense Urban – eMBB |  |  | Uplink |  | 1.12 (30 km/h) |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | eMBB |  | Rural – eMBB |  |  | Uplink |  | 0.8 (120 km/h) |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 0.45 (500 km/h) |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.14 Mobility interruption time (ms) (4.12) |  |  | eMBB and URLLC |  | Not applicable |  |  | Not applicable |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.4.3.15 Bandwidth and Scalability (4.13) |  |  | Not applicable |  | Not applicable |  |  | Not applicable |  | At least 100 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Up to 1 GHz |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Support of multiple different bandwidth values(4) |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

If a proponent determines that a specific question does not apply, the proponent should indicate that this is the case and provide a rationale for why it does not apply.