chapter 5

Meeting Report of Working Group Technology Aspects

5	Meeting Report of Working Group Technology Aspects

5.1	Meeting Report of Sub-working Group IMT Specifications

5.2	Workplan for a draft revision of Recommendation ITU-R M.1457-14

5.3	Workplan for development of draft new Recommendation ITU-R M.[IMT-2020.SPECS]

5.4	Working document towards preliminary draft new Recommendation ITU-R M.[IMT-2020.SPECS] – Detailed specifications of the terrestrial radio interfaces of International Mobile Telecommunications-2020 (IMT-2020)

5.5	Meeting report of Sub-working Group Coordination

5.6	Meeting report of Sub-working Group Evaluation

5.7	[Draft] detailed workplan for development of draft new Report ITU-R M.[IMT-2020.OUTCOME]

5.8	[Draft] Working document towards preliminary draft new Report ITU-R M.[IMT-2020.OUTCOME] – The outcome of the evaluation, consensus building and decision of the IMT-2020 process (Steps 4 to 7), including characteristics of IMT-2020 radio interfaces

chapter 5

Source:	Document 5D/TEMP/35

Meeting Report of Working Group Technology Aspects

# 1	General

Working Group (WG) Technology Aspects met two times during the 33rd meeting of ITU-R Working Party (WP) 5D. The draft agendas are available in Docs. 5D/ADM/4 and 9.

Main activities of WG Technology Aspects during this meeting were to:

i)	Review additional materials provided by the candidate IMT-2020 RIT/SRIT proponents ETSI (TC DECT) and DECT Forum, Nufront and TSDSI, per the agreed way forward at the 32nd meeting of WP 5D regarding their respective submissions;

ii)	Review of external activities in Independent Evaluation Groups through interim evaluation reports;

iii)	Continue work on revision of Recommendation ITU-R M.1457-14.

iv)	Start working on Report ITU-R M.[IMT-2020.OUTCOME].

v)	Start working on Recommendation ITU-R M.[IMT-2020.SPECS].

During this meeting, WG Technology Aspects established three Sub-Working Groups (SWG):

–	SWG Coordination (Chair: Mr. Yoshio HONDA)

–	SWG Evaluation (Co-Chair: Ms. Ying PENG)

–	SWG IMT Specifications (Chair: Mr. Yoshinori ISHIKAWA)

# 2	Activities

Review of updated materials of IMT-2020 submissions

As per the agreed way forward at the 32nd WP 5D meeting regarding candidate IMT-2020 RIT/SRIT submissions of ETSI (TC DECT) and DECT Forum, Nufront and TSDSI, the respective proponents provided updated materials of their submissions on September 10th of 2019.

| ETSI (TC DECT) and DECT Forum | Doc. 5D/1299 |
| --- | --- |
| Nufront | Doc. 5D/1300 |
| TSDSI | Doc. 5D/1301 |

After review of these updated materials of submissions under the IMT-2020 Process Step 3 – Submission / reception of the RIT and SRIT proposals and acknowledgement of receipt, the meeting determined that the submissions of ETSI (TC DECT) and DECT Forum, Nufront and TSDSI are “complete” per Section 5 of Report ITU-R M.2411. In the course of this review, documents for observations on the submissions were updated (Docs. 5D/TEMP/14, TEMP/15, and TEMP/16). Respective acknowledgements of the submissions were developed (Docs. 5D/TEMP/17, TEMP/18 and TEMP/19).

There are concerns on the submission in Document 5D/1300 regarding potential contradictions between its technical specification and self-evaluation report, which were not solved in the meeting. The proponent and parties with these concerns are encouraged to solve them urgently through discussions in Evaluation Group discussion area before the next meeting of WP 5D.

Review of interim evaluation reports

A workshop on IMT-2020 terrestrial radio interfaces evaluation was conducted at beginning of this WP 5D meeting, where the registered independent evaluation groups presented their activities and findings. In addition, some independent evaluation groups also submitted interim evaluation reports. The meeting reviewed these contributions and recorded them in Documents IMT-2020 (Docs. 5D/TEMP/1(Rev.1), TEMP/2(Rev.1), TEMP/3(Rev.1), TEMP/4(Rev.1), TEMP/5(Rev.1), TEMP/6(Rev.1), TEMP/7(Rev.1), TEMP/8(Rev.1), TEMP/9(Rev.1) and TEMP/10(Rev.1)).

Report ITU-R M.[IMT-2020.OUTCOME]

The meeting created the draft detailed workplan and draft working document towards a prelimininary draft new Report ITU-R M.[IMT-2020.OUTCOME] which will collect outcomes from Step 4 to 7 in IMT-2020 development process. Those two documents were carried forward to the next meeting for further work (Docs. 5D/TEMP/29 and TEMP/30).

Detailed schedule for development of IMT-2020.SPECS

Detailed workplan for development of draft new Recommendation ITU-R M.[IMT-2020.SPECS] and its working document were developed in the meeting (Docs. 5D/TEMP/25 and TEMP/24). A related liaison statement to the external organizations was also developed (Doc. 5D/TEMP/26).

Revision of Recommendations ITU-R M.1457

The meeting reviewed the input document from ETSI (Doc. 5D/13) regarding next revision of Rec. ITU-R M.1457 for inclusion the technical update of IMT-2000 FDMA/FDMA. The meeting agreed to finalize Revision 15 of Rec. ITU-R M.1457 based on the materials according to the schedule.

Carried forward documents

| SWG | Workplan Doc. (5D/TEMP) | TEMP Doc. (5D/TEMP) | Contribution Doc. (5D/) |
| --- | --- | --- | --- |
| Evaluation | 29 | 30 | – |
| IMT Specifications | 25, 27 | 24 | – |
| Radio Aspects | – | – | 1226, 1278, 1284 |

# 3	Objective for the 34th WP 5D meeting

The key objectives of WG Technology Aspects for the 34th WP 5D meeting are as follows:

i)	Review of external activities and evaluation reports of Independent Evaluation Groups.

ii)	Complete evaluation reports summary (IMT-2020/ZZZ).

iii)	Continue working on a new Report ITU-R M.[IMT-2020.OUTCOME].

iv)	Continue the work on “Over-the-air (OTA) TRP field measurements for IMT radio equipment utilizing AAS” based on the requested response from 3GPP and expected input from other organisations and administrations.

v)	Continue working on revision of Recommendation ITU-R M.1457-14.

vi)	Continue working on new Recommendation ITU-R M.[IMT-2020.SPECS]

vii)	Continue working on synchronization of multiple IMT-2020 TDD networks.

# 5	Concluding remarks

The Chair would like to thank all the participants that contributed to this meeting of WG Technology Aspects, with special thanks to the SWG Chairs, Ms. Ying PENG, Mr. Yoshio HONDA and Mr. Yoshinori ISHIKAWA.

Hu WANG
	Chair, Working Group Technology Aspects

Attachment 5.1

Source:	Document 5D/TEMP/28

Meeting Report of Sub-working Group IMT Specifications

Sub-working Group IMT-Specifications met for one period during WP 5D #33 meeting. The agenda for the meeting is available in Document ADM/007, as approved under agenda item 2.

The objectives of this WP 5D #33 meeting are continuing the work on draft revision of Recommendation ITU-R M.1457-14 and starting discussion on the preliminary draft new Recommendation ITU-R M.[IMT-2020.SPECS].

1	Recommendation ITU-R M.1457-15

Under agenda item 3 (Rec. ITU-R M.1457-15), the meeting received one contribution from GCS Proponent of TDMA/FDMA (Doc. 5D/13) and the meeting determined that this was meeting “X+1” for Revision 15 of Rec. ITU-R M.1457.

The meeting reviewed the input contribution and recognized that the input provides required information for meeting “X+1”. It was confirmed that the required materials to be submitted to ITU-R WP 5D to finalize Revision 15 of Rec. ITU-R M.1457 according to the update schedule per Doc. IMT-2000/8.

The summary of Revision 15 update of Rec. ITU-R M.1457 is as following:

Anticipated revision 15 update of Rec. ITU-R M.1457

| Section | RTT Name | GCS Proponents | Meeting X input | Meeting X+1 input | Meeting X+2 input | Transposing Organizations for Rev. 14 |
| --- | --- | --- | --- | --- | --- | --- |
| 5.1 | CDMA DS | ARIB, ATIS, CCSA, ETSI, TTA, TTC, TSDSI | – | – | – | – |
| 5.2 | CDMA MC | ARIB, CCSA, TIA, TTA, TTC | – | – | – | – |
| 5.3 | CDMA TDD | ARIB, ATIS, CCSA, ETSI, TTA, TTC, TSDSI | – | – | – | – |
| 5.4 | TDMA SC | ATIS | – | – | – | – |
| 5.5 | TDMA FDMA | ETSI | 5D/1223 | 5D/13 | – | – |
| 5.6 | OFDMA TDD WMAN | IEEE | – | – | – | – |

No new candidate RIT/SRIT submission was received by 2nd July 2019 (i.e. Cut-off date for submission of candidate RIT and SRIT proposal).

The workplan for Revision 15 of Rec. ITU-R M.1457 was updated as 5D/TEMP/27 accordingly.

# 2	M.[IMT-2020.SPECS]

Under agenda item 4 (PDN Rec. ITU-R M.[IMT-2020.SPECS]), the meeting received two input contributions from Korea, Japan (Doc. 5D/016) and China (Doc. 5D/23).

Both contributions propose the detailed workplan for developing the new Recommendation ITU-R M.[IMT-2020.SPECS] and the working document towards preliminary draft new Recommendation.

The meeting reviewed those proposals and discussed the workplan and contents for the working document and agreed the detailed workplan as Doc. 5D/TEMP/25 and the working document towards PDNR as 5D/TEMP/024.

The meeting also created a liaison statement to relevant External Organizations (RIT/SRIT Proponents, potential GCS Proponent(s) of IMT-2020) to request the inputs to 34th and 35th meetings in accordance with Doc. IMT-2020/21 as Doc. 5D/TEMP/26.

SWG-IMT Specification seeks approval of this liaison in WG Technology Aspects Plenary and WP 5D Plenary.

Finally, Chairman would like to thank all delegates, GCS Proponent of TDMA/FDMA and BR for their valuable and constructive support during the meeting.

Yoshinori ISHIKAWA
Chairman, SWG IMT Specifications

attachment 5.2

Source:	Document 5D/TEMP/27

Workplan for a draft revision of Recommendation ITU-R M.1457-14

| Title | Work plan for a draft Revision of Recommendation ITU-R M.1457-14 |
| --- | --- |
| Identifier | Recommendation ITU-R M.1457 |
| WP 5D Lead Group | WG Technology Aspects SWG-IMT Specifications |
| SWG Chair | Mr. Yoshinori Ishikawa - yoshinori.ishikawa.dh@hitachi.com |
| Editor |  |
| Focus for scope and work | To include latest technology updates to current IMT-2000 terrestrial radio interfaces based on the proposals from relevant External Organizations. To add new IMT-2000 radio interface technologies if new candidates are proposed, evaluated and agreed to be included as per current process. This workplan reflects the content of Document IMT-2000/8. |
| Related documents | Recommendation ITU-R M.1036-5 Recommendation ITU-R M.1225 Recommendation ITU-R M.1579-2 Recommendation ITU-R M.1580-5 Recommendation ITU-R M.1581-5 IMT-2000/1~4, 8 |
| Milestones | Meeting No. 31bis (February 2019, Geneva) (Meeting X-1) 1 Sends a liaison to relevant External Organizations to communicate the schedule of the development of the Revision of Recommendation M.1457-14. Meeting No. 32 (July 2019, Brazil) (Meeting X) 1. Receives and reviews the request for updating a particular IMT-2000 radio interface technology by GCS Proponents. 2 Responds to GCS Proponents if required. 3 Checks there are any new radio interface technology candidates for IMT-2000 received. (Note 1). Meeting No. 33 (December 2019, Geneva) (Meeting X+1) 1 Receives and reviews further information, including a summary and rationale of the proposed update by GCS Proponents. 2 Responds to GCS Proponents if required. Meeting No. 34 (February 2020, Geneva) (Meeting X+2A) 1 Receives and reviews the detailed update and related supporting material for the proposed Revision by GCS Proponents. (When X+2A submission selected). (Note 2) 2 Reaches its conclusion on the acceptability of the proposed update for inclusion in the draft revision of Recommendation ITU-R M.1457-14. (When X+2A submission selected). 3 Develops a working document towards the draft revision of Recommendation ITU-R M.1457-14. Meeting No. 35 (June/July 2020, China) (Meeting X+2B) 1 Receives and reviews the detailed update and related supporting material for the proposed Revision by GCS Proponents. (Note 2) 2 Reaches its conclusion on the acceptability of the proposed update for inclusion in the draft revision of Recommendation ITU-R M.1457-14. 3 Further develops and provisionally agrees the preliminary draft revision of Recommendation ITU-R M.1457-14 (without the detailed transposition references). 4 Sends liaison of the WP 5D provisionally agreed draft Revision of Recommendation ITU-R M.1457-14 (without detailed transposition references) to the Transposing Organizations for their use in developing their inputs of the detailed references. Meeting No. 36 (October 2020, India) (Note 3) 1 Performs a final quality and completeness check (with detailed transposition references) of the draft revision of Recommendation ITU-R M.1457-14. 2 Sends to Study Group 5 the Revision of Recommendation ITU-R M.1457-14 for adoption. |

NOTE 1 – No new RIT proposal was received by WP 5D #32 meeting.

NOTE 2 – If the GCS Proponent and the Transposing Organization consist of same organization, the GCS Proponent and the Transposing Organization can submit all required materials for updating its Section including Transposition References and Certification C by meeting X+2A or X+2B.

NOTE 3 – If the concerned GCS Proponent and the Transposing Organization in this revision have already submitted all required materials for updating its Section including Transposition References and Certification C by meeting X+2, WP 5D may finalize the draft revision of Recommendation ITU-R M.1457-14 earlier than WP 5D #36 meeting.

attachment 5.3

Source:	Document 5D/TEMP/25

Workplan for development of draft new Recommendation
ITU-R M.[IMT-2020.SPECS]

| Title | Detailed specifications of the terrestrial radio interfaces of International Mobile Telecommunications-2020 (IMT-2020) |
| --- | --- |
| Identifier | M.[IMT-2020.SPECS] |
| Document Type | Recommendation |
| WP 5D Lead Group | WG Technology Aspects SWG-IMT Specifications |
| SWG Chair | Mr. Yoshinori Ishikawa; E-mail: [yoshinori.ishikawa.dh@hitachi.com] |
| Editor | - |
| Focus for scope and work | Development of this Recommendation identifying the terrestrial radio interface technologies of International Mobile Telecommunications-2020 (IMT-2020) and providing the detailed radio interface specifications. M.[2020.SPECS] would contain the detailed specifications of the IMT-2020 technologies. |
| Related documents | Recommendation ITU-R M.1036 Recommendation ITU-R M.1224 Recommendation ITU-R M.1457 Recommendation ITU-R M.1579 Recommendation ITU-R M.1822 Recommendation IUT-R M.2012 Recommendation ITU-R M.2083 Report ITU-R M.2320 Report ITU-R M.2334 Report ITU-R M.2376 Report ITU-R M.2410 Report ITU-R M.2411 Report ITU-R M.2412 Report ITU-R M.[IMT-2020.OUTCOME] Resolution ITU-R 50 Resolution ITU-R 56 Resolution ITU-R 57 Resolution ITU-R 65 Doc. IMT-2020/1 Doc. IMT-2020/20 Doc. IMT-2020/21 |
| Milestones | Meeting No. 33 (Dec. 2019, Geneva) 1 Develop a detailed workplan. 2 Develop a working document towards PDN Rec. ITU-R M.[IMT-2020.SPECS]. Meeting No. 34 (Feb. 2020, Geneva) 1 Review the workplan, if necessary. 2 Continue developing the working document towards PDN Rec. ITU-R M.[IMT-2020.SPECS]. 3 Receive and take note of “Form A”, in order to determine the structure of the Recommendation. 4 Provide and send liaisons to RIT/SRIT Proponents and GCS Proponents. Meeting No. 35 (Jun. 2020, [China]) 1 Receive and review information, including the texts for its RIT/SRIT overview sections, List of Global Core Specifications and Certification B by GCS Proponents. 2 Reach its conclusion on the acceptability of the proposed materials for inclusion in the working document towards PDN Rec. ITU-R M.[IMT-2020.SPECS]. 3 Finalizes the working document including specific technologies (not necessarily including the detailed transposition references) and provisionally agree for promoting the document to preliminary draft new Recommendation. 4 Provide and send liaison of the provisionally agreed preliminary draft new Recommendation ITU-R M.[IMT-2020.SPECS] to the relevant GCS Proponents and Transposing Organizations for their use in developing their inputs of the detailed references. Meeting No. 36 (Oct. 2020, [India]) 1 Update PDNR if there are modifications proposed by GCS Proponent. 2 Perform a quality and completion check of the provisionally agreed final draft new Recommendation ITU-R M.[IMT-2020.SPECS] without the hyperlinks. 3 Have follow-up communications initiated with GCS Proponents and/or Transposing Organizations, if necessary. Meeting No. 36bis (Nov. 2020, Geneva) 1 Receive Transposition references and Certification C from each Transposing Organization. 2 Perform the final quality and completeness check (with detailed transposition references) of the preliminary draft new Recommendation and promotes it to draft new Recommendation. 3 Send the draft new Recommendation ITU-R M.[IMT-2020.SPECS] to Study Group 5 for consideration. |

attachment 5.4

Source:	Documents 5D/TEMP/24

Working document towards preliminary draft new Recommendation ITU-R M.[IMT-2020.SPECS]

Detailed specifications of the terrestrial radio interfaces of International Mobile Telecommunications-2020 (IMT-2020)

Summary

This element is placed in front of the Recommendation. It provides a brief overview of the purpose and contents giving the reasons for the study and motivations for development of this Recommendation, thus permitting ITU members and users to judge its usefulness for their work. In the case of revision/modification of an existing Recommendation this element should include brief description of changes in particular for Recommendation, incorporated by reference in the Radio Regulations. This element defines, without ambiguity, the intent or subject of the Recommendation and should clarify the objective of the Recommendation should indicate the limits of applicability The Summary should not be retained in the text of the Recommendation after its approval.

[Editor’s note: Summary section needs to be stable at WP 5D #36 meeting.]

RECOMMENDATION ITU-R M.[IMT-2020.SPECS]

Detailed specifications of the terrestrial radio interfaces of International
Mobile Telecommunications-2020 (IMT-2020)

([years of approval]l)

Scope

This Recommendation identifies the terrestrial radio interface technologies of International Mobile Telecommunications-2020 (IMT-2020) and provides the detailed radio interface specifications.

Korea, Japan

[These radio interface specifications detail the features and parameters of IMT-2020, which enable worldwide compatibility, international roaming, access to the services under the usage scenarios including enhanced mobile broadband (eMBB), massive machine type communications (mMTC) and ultra-reliable and low latency communications (URLLC).]

China

[These radio interface specifications detail the features and parameters of IMT-2020. This Recommendation includes the capability to ensure worldwi.de compatibility, international roaming, access to enhanced mobile broadband (eMBB), massive machine type communications (mMTC) and ultra reliability and low latency communications (URLLC).]

[Editor’s note: Scope section needs to be stable at early stage e.g. WP 5D #33 or #34 meeting.]

Keywords

IMT, IMT-2020, Radio interface specifications, Enhanced Mobile Broadband (eMBB), Machine Type Communications (MTC), Ultra-Reliable and Low Latency Communications (URLLC)

[Editor’s note: Keywords needs to be stable by WP 5D #35 meeting.]

Abbreviations/Glossary

eMBB	Enhanced Mobile Broadband

IMT	International Mobile Telecommunications

mMTC	Massive Machine Type Communications

MTC	Machine Type Communications

URLLC	Ultra-Reliable and Low Latency Communications

RIT	Radio interface technologies

SRIT	Set of Radio interface technologies

GCS	Global Core Specifications

[Editor’s note: Glossary needs to be filled by WP 5D #36 meeting.]

Related ITU Recommendations, Reports, Document and Handbook

Recommendation ITU-R M.1036	Frequency arrangements for implementation of the terrestrial component of International Mobile Telecommunications (IMT) in the bands identified for IMT in the Radio Regulations (RR)

Recommendation ITU-R M.1224	Vocabulary of Terms for International Mobile Telecommunications (IMT)

Recommendation ITU-R M.1579	Global circulation of IMT terrestrial terminals

Recommendation ITU-R M.1822	Framework for services supported by IMT

Recommendation ITU-R M.2083	IMT Vision – Framework and overall objectives of the future development of IMT for 2020 and beyond

Report ITU-R M.2320	Future technology trends of terrestrial IMT systems

Report ITU-R M.2334	Passive and active antenna systems for base stations of IMT systems

Report IMT-R M.2370	IMT traffic estimations for the years 2020 to 2030

Report IMT-R M.2375	Architecture and topology of IMT networks

Report ITU-R M.2376	Technical feasibility of IMT in bands above 6 GHz

Report ITU-R M.2410	Minimum requirements related to technical performance for IMT-2020 radio interface(s)

Report ITU-R M.2411	Requirements, evaluation criteria and submission templates for the development of IMT-2020

Report ITU-R M.2412	Guidelines for evaluation of radio interface technologies for IMT-2020

Report ITU-R M.2441	Emerging usage of the terrestrial component of International Mobile Telecommunication (IMT)

Report ITU-R M.[IMT-2020.OUTCOME]	The outcome of the evaluation, consensus building and decision of the IMT-2020 process (steps 4-7), including characteristics of IMT-2020 radio interfaces

Resolution ITU-R 50		Role of the Radiocommunication Sector in the ongoing development of IMT

Resolution ITU-R 56	Naming for International Mobile Telecommunications

Resolution ITU-R 57	[Principles for the process of future development of IMT-ADVANCED]

Resolution ITU-R 65	Principles for the process of future development of IMT for 2020 and beyond

Document IMT-2020/2(Rev.2)	Submission, evaluation process and consensus building for IMT-2020

Handbook on Global Trends in International Mobile Telecommunication

[Editor’s note: This section needs to be reviewed, added and modified for finalizing by WP 5D #36bis meeting including other New Recommendation(s)/Report(s) to be sent to SG5 at November 2020.]

The ITU Radiocommunication Assembly,

considering

a)	that IMT systems are mobile broadband systems including IMT-2000, IMT-Advanced and IMT-2020;

b)	that IMT-2020 systems include the new capabilities of IMT that go beyond those of IMT-2000 and IMT-Advanced, and will interwork with and complement existing IMT and its enhancements;

c) 	that IMT-2020 is envisaged to expand and support diverse usage scenarios and applications that will continue beyond the current IMT;

d)	that such systems provide access to a wide range of telecommunication services including advanced mobile services, supported by mobile and fixed networks;

e)	that ITU has contributed to standardization and harmonized use of IMT, which has provided telecommunication services on a global scale and global operation and economies of scale are key requirements for the success of mobile telecommunication systems;

f)	that IMT-2020 systems support low to high mobility applications and a wide range of data rates in accordance with user and service demands in multiple user environments;

g)	that the usage scenarios of IMT-2020 include enhanced Mobile Broadband, ultra-reliable and low latency communication and massive machine-type communication;

h)	that IMT-2020 systems support not only human communications but also the machine type communications;

i)	that IMT-2020 systems support ultra-reliable and low latency communications which are necessary in particular use cases;

j)	that IMT-2020 also has capabilities for high-quality multimedia applications within a wide range of services and platforms providing a significant improvement in performance, quality of service and user experience;

k)	that the key features of IMT-2020 are:

–	a high degree of commonality of functionality worldwide while retaining the flexibility to support a wide range of services and applications in a cost-efficient manner;

–	compatibility of services within IMT and with fixed networks;

–	capability of interworking with other radio access systems;

–	high-quality mobile services for maximum user experiences;

–	user equipment suitable for worldwide use;

–	worldwide roaming capability;

–	further enhanced peak data rates to support advanced services and applications (10 Gbit/s for uplink and 20 Gbit/s for downlink) and enables 100 Mbit/s user experienced data rate;

–	enhanced spectrum efficiency by three times higher compare to IMT-Advanced;

–	high downlink area traffic capacity of 10 Mbit/s/m2;

–	ultra low latency [of 1 ms] over-the air;

–	[high connection density of 1000000 devices/km2 /extreme connection density];

–	[high reliability communication of 1-10-5 success probability in certain condition / ultra reliability communication  success probability within 1 ms in certain condition];

–	high speed mobility station;

–	enhanced low energy consumption mechanism;

l)	that these features enable IMT-2020 to address evolving user needs in various usage scenarios;

m)	that the capabilities of IMT-2020 systems are being continuously enhanced in line with technology developments;

n)	the necessity of priority services (e.g. emergency calls shall be supported as higher priority than other commercial services);

o)	that IMT systems are considered as the major candidate for Public Protection and Disaster Relief (PPDR);

p)	that due to the large effective bandwidths required to support the very high data rates needed for the various services offered, allowances must be made for either much larger single carrier bandwidths (even as spectral efficiencies increase) or aggregation of RF carriers;

q)	that for some scenarios of IMT-2020 requiring bandwidths of [several hundred MHz/ at least 100 MHz,] up to at least 1 GHz, there would be a need to consider wideband contiguous spectrum above 6 GHz;

r)	that IMT-2020 will realize the Internet of Things by connecting a vast range of smart appliances, machines and other objects without human intervention;

s)	that applicable areas of [IMT/ IMT-2020 and beyond] are expected to be expanded further to various specific applications to facilitate the digital economy, e.g. e-manufacturing, e-agriculture, e-health, intelligent transport systems, smart city and traffic control, etc., which could bring requirements beyond current capabilities of IMT;

t)	that IMT-2020 should be able to provide these capabilities without undue burden on energy consumption, network equipment cost and deployment cost to make future IMT sustainable and affordable;

u)	that other capabilities may be also required for IMT-2020, which would make future IMT more flexible, reliable, and secure when providing diverse services in the intended usage scenarios,

[Editor’s note: Considering part needs to be reviewed, added and modified for finalizing at WP 5D #35 meeting.]

recognizing

that Resolution ITU-R 65 on the “Principles for the process of future development of IMT for 2020 and beyond” outlines the essential criteria and principles used in the process of developing the Recommendations and Reports for IMT-2020, including Recommendation(s) for the radio interface specification,

[Editor’s note: Recognizing part needs to be reviewed, added and modified for finalizing at WP 5D #35 meeting.]

noting

that Report ITU-R M.[IMT-2020.OUTCOME] contains the outcome and conclusions of Steps 4 through 7 of the IMT-2020 process, including the evaluation and consensus building, and provides the characteristics of the IMT-2020 terrestrial radio interfaces for the first release of Recommendation ITU-R M.[IMT-2020.SPECS],

[Editor’s note: Noting part needs to be reviewed, added and modified for finalizing at 35th meeting.]

recommends

1	that the terrestrial radio interfaces for IMT-2020 should be:

–	“[name of RIT/SRIT in this Recommendation]”;

–	“[name of RIT/SRIT in this Recommendation]”; and

･

･

–	“[name of RIT/SRIT in this Recommendation]”;

2	that the information provided or referenced in Annexes 1, 2, ･･･ and n should be used according to the terrestrial radio interfaces referred to in recommends 1 above as the complete set of standards for the detailed specifications of the terrestrial radio interfaces of IMT-2020.

[Editor’s note: recommends part needs to be finalized at WP 5D #35 meeting.]

[Editor’s note: Annexes including Transposition References (if GCS utilized) need to be finalized at WP 5D #36bis meeting.]

Annex x

Specification of the [name of RIT/SRIT in this Recommendation] radio interface technology

[Editor’s note: Template for RIT/SRIT in GCS utilized case.]

TABLE OF CONTENTS

Page

Background		xx

x.1	Overview of the radio interface technology		xx

x.2	Detailed specification of the radio interface technology		xx

Background

IMT-2020 is a system with global development activity and the IMT-2020 terrestrial radio interface specifications identified in this Recommendation have been developed by the ITU in collaboration with the GCS Proponents and the Transposing Organizations. It is noted from Document IMT-2020/20, that:

–	The GCS Proponent must be one of the RIT/SRIT Proponents for the relevant technology, and must have legal authority to grant to ITU-R the relevant legal usage rights to the relevant specifications provided within a GCS corresponding to a technology in Recommendation ITU-R M.[IMT-2020.SPECS].

–	A Transposing Organization must have been authorized by the relevant GCS Proponent to produce transposed standards for a particular technology, and must have the relevant legal usage rights.

It is further noted that GCS Proponents and Transposing Organizations must also qualify appropriately under the auspices of Resolution ITU-R 9 and the ITU-R “Guidelines for the contribution of material of other organizations to the work of the Study Groups and for inviting other organizations to take part in the study of specific matters (Resolution ITU-R 9)”.

The ITU has provided the global and overall framework and requirements, and has developed the Global Core Specification jointly with the GCS Proponent. The detailed standardization has been undertaken within the recognized Transposing Organizations which operate in concert with the GCS Proponent. This Recommendation therefore makes extensive use of references to externally developed specifications.

This approach was considered to be the most appropriate solution to enable completion of this Recommendation within the aggressive schedules set by the ITU and by the needs of administrations, operators and manufacturers.

This Recommendation has therefore been constructed to take full advantage of this method of work and to allow the global standardization time-scales to be maintained. The main body of this Recommendation has been developed by the ITU, with each Annex containing references pointing to the location of the more detailed information.

This Annex 1 contains the detailed information developed by the ITU and “[name of GCS Proponent]” (the GCS Proponent) and [name(s) of Transposing Organization(s)] (the Transposing Organizations).

Such use of referencing enables timely completion and update of the high-level elements of this Recommendation, with change control procedures, transposition, and public enquiry procedures being undertaken within the external organization. This information has generally been adopted unchanged, recognizing the need to minimize duplication of work, and the need to facilitate and support an ongoing maintenance and update process.

This general agreement, noting that the detailed information of the radio interface should to a large extent be achieved by reference to the work of external organizations, highlights not only the ITU’s significant role as a catalyst in stimulating, coordinating and facilitating the development of advanced telecommunications technologies, but also its forward-looking and flexible approach to the development of this and other telecommunications standards for the 21st century.

A more detailed understanding of the process for the development of the first release of this Recommendation may be found in Document IMT-2020/20.

## x.1	Overview of the radio interface technology

### [x.1.1 Overview of the RIT]

### [x.1.2 Overview of the SRIT]

#### [x.1.2.1 overview of the component RIT1]

#### [x.1.2.2 overview of the component RIT2]

## x.2	Detailed specification of the radio interface technology

Detailed specifications described in this Annex are developed around a “Global Core Specification” (GCS), which is related to externally developed materials incorporated by specific references for a specific technology. The process and use of the GCS, references, and related notifications and certifications are found as Document IMT-2020/20.

The IMT-2020 standards contained in this section are derived from the global core specification for [name of RIT/SRIT in this Recommendation] contained at http://ties.itu.int/u/itu-r/ede/rsg5/IMT-2020/GCS/M.[IMT-2020.SPECS]-0/XXXXXX/. The following notes apply to the sections below:

1)	The identified Transposing Organizations should make their reference material available from their website.

2)	This information was supplied by the Transposing Organizations and relates to their own deliverables of the transposed global core specification.

### x.2.1	Titles and synopses of the global core specification and the transposed standards

#### x.2.1.1	Introduction

The standards documents referenced below, as transposed from the GCS, are provided by the identified Transposing Organizations as the transposed sets of standards for the terrestrial radio interface of IMT-2020 identified as [name of RIT/SRIT in this Recommendation] and includes not only the key characteristics of IMT-2020 but also the additional capabilities of [name of RIT/SRIT in this Recommendation] both of which are continuing to be enhanced.

This part should: - contain technical details or description of methods/procedures - support or clarify the relevant recommends; - be numbered as Annex 1, Annex 2 etc. It is necessary for overall completeness and comprehensibility If the Annex text is more than 5 pages long a table of contents is needed.

[Editor’s note: Chinese proposal (Doc. 5D/23) does not contain Annex Y (DIS style).]

Annex y

Specification of the [name of RIT/SRIT in this Recommendation] radio interface technology

[Editor’s note: This annex is for RIT/SRIT in DIS case. The structure and titles of the sub-sections are examples.]

TABLE OF CONTENTS

Page

Background		xx

y.1	Overview of the radio interface technology		xx

y.2	System Aspects (including network architecture, system architecture, Layer 2 protocols, Physical layer, signaling details, etc.)		xx

y.3	Band class information and Unwanted emission characteristics (including Spectrum Mask, spurious, ACLR, etc.)		xx

y.4	Others		xx

Background

IMT-2020 is a system with global development activity and the IMT-2020 terrestrial radio interface specifications identified in this Recommendation have been developed by the ITU in collaboration with the GCS Proponents. It is noted from Document IMT-2020/20, that:

–	The GCS Proponent must be one of the RIT/SRIT Proponents for the relevant technology, and must have legal authority to grant to ITU-R the relevant legal usage rights to the DIS in the Recommendation ITU-R M.[IMT-2020.SPECS].

It is further noted that GCS Proponents must also qualify appropriately under the auspices of Resolution ITU-R 9 and the ITU-R “Guidelines for the contribution of material of other organizations to the work of the Study Groups and for inviting other organizations to take part in the study of specific matters (Resolution ITU-R 9)”.

The ITU has provided the global and overall framework and requirements, and has developed the Global Core Specification jointly with the GCS Proponent.

This Recommendation has therefore been constructed to take full advantage of this method of work and to allow the global standardization time-scales to be maintained. The main body of this Recommendation has been developed by the ITU, with each Annex containing references pointing to the location of the more detailed information.

This Annex 2 contains the detailed information developed by the ITU and “[name of GCS Proponent]” (the GCS Proponent).

A more detailed understanding of the process for the development of the first release of this Recommendation may be found in Document IMT-2020/20.

#### y.1	Overview of the radio interface technology

#### y.2	System Aspects (including network architecture, system architecture, Layer 2 protocols, Physical layer, signaling details, etc.)

#### y.3	Band class information and Unwanted emission characteristics (including Spectrum Mask, spurious, ACLR, etc.)

#### y.4	Others

Attachment(s) to Annex (if needed):

This part should: - contain material which is supplementary to and associated with an Annex of a Recommendation; - clarify the relevant recommends. It is not essential to Rec completeness and comprehensibility If its text is more than 5 pages long a table of contents is needed.

Appendix(es) should not be used as a part of Recommendation in order to avoid confusion with Appendix(es) used in the RR

attachment 5.5

Source:	Document 5D/TEMP/33

Meeting report of Sub-working Group Coordination

Sub-working Group (SWG) Coordination met four times including joint sessions with SWG Evaluation in WP 5D #33 meeting.

The objectives of SWG Coordination are as follows:

1	Review of updated submissions for IMT-2020 radio interfaces technology, and provide them with acknowledgement/reception in Step 3 of IMT-2020 development process;

2	Draft liaisons to the proponents;

3	Draft Addendum 5 of Circular Letter 5/LCCE/59;

4	Draft a liaison to the external organizations to inform the availability of Addendum 5.

During the meeting, the following documents were considered:

|  |  | Document 5D/ |
| --- | --- | --- |
| 1 | IMT-2020 revised submissions | 5D/1299, 5D/1300, 5D/1301, 5D/12 |
| 2 | Draft Addendum 5 of Circular Letter 5/LCCE/59 | 5D/18 |
| 3 | Others | 5D/10, 5D/20 |

IMT-2020 final submissions and their acknowledgement/reception

There were updated submissions from three proponents, ETSI (TC DECT)/DECT Forum (SRIT), NuFront (RIT) and TSDSI (RIT) based on the way forward (IMT-2020/22) agreed in 32nd meeting. Considering the conclusions and observations in SWG Evaluation on the updated submissions, SWG Coordination drafted acknowledgements for the submissions based on IMT-2020/YYY template (Attachment 5.9 to Doc. 5D/1110).

It was determined that the following submissions are “complete” per Section 5 of Report ITU-R M.2411 and were acknowledged below:

−	Doc. IMT-2020/17(Rev.1) − Acknowledgement of candidate SRIT submission from ETSI (TC DECT) and DECT Forum under Step 3 of the IMT-2020 process (Doc. 5D/TEMP/19)

–	Doc. IMT-2020/18(Rev.1) − Acknowledgement of candidate RIT submission from Nufront under Step 3 of the IMT-2020 process (Doc. 5D/TEMP/18)

−	Doc. IMT-2020/19(Rev.1) − Acknowledgement of candidate RIT submission from TSDSI under Step 3 of the IMT-2020 process (Doc. 5D/TEMP/17)

Liaison statements to the proponents to inform the acknowledgements respectively were drafted (Docs. 5D/TEMP/20, 5D/TEMP/21 and 5D/TEMP/22).

Considering the revised submissions, IMT-2020 Documents to record them were revised (Docs. 5D/TEMP/13, 5D/TEMP/11, 5D/TEMP/12).

Draft Addendum 5 of Circular Letter 5/LCCE/59 and a liaison

The SWG Coordination prepared a draft Addendum 5 for further acknowledgement of the updated technology submissions (Doc. 5D/TEMP/31), and a related liaison statement to the external organizations informing the availability of Addendum 5 (Doc. 5D/TEMP/32).

Documents for approval

5D/TEMP/13		Draft Revision 4 to Document IMT-2020/6

5D/TEMP/11		Draft Revision 1 to Document IMT-2020/12

5D/TEMP/12		Draft Revision 4 to Document IMT-2020/7

5D/TEMP/19	Acknowledgement, SRIT submission from ETSI (TC DECT) and DECT Forum (Doc. IMT-2020/17(Rev.1))

5D/TEMP/18	Acknowledgement, RIT submission from Nufront (Doc. IMT-2020/18(Rev.1))

5D/TEMP/17	Acknowledgement, RIT submission from TSDSI (Doc. IMT-2020/19(Rev.1))

5D/TEMP/22	Draft liaison statement to “ETSI (TC DECT) and DECT Forum”

5D/TEMP/21	Draft liaison statement to “Nufront”

5D/TEMP/20	Draft liaison statement to “TSDSI”

5D/TEMP/31	Draft Addendum 5 of Circular Letter 5/LCCE/59

5D/TEMP/32	A liaison to External Organizations (Addendum 5 availability)

Document to carry forward to next WP 5D meeting

None.

Finally, the chairman would like to appreciate all the participants for their constructive discussions, collaborations and excellence advices to the meeting.

# Mr. Yoshio HONDA

# Chairman, SWG Coordination

Attachment 5.6

Source:	Document 5D/TEMP/34

Meeting Report of Sub-Working Group Evaluation

Sub-working Group (SWG) Evaluation met in 8 sessions (including 1 joint meeting with workshop session 3 and SWG Coordination) during the 33rd meeting of WP 5D, with an average of 70-80 participants per meeting. There are 38 input contributions received and 6 carried forward documents in this meeting.

The objectives of SWG Evaluation for 33rd WP 5D meeting are as following:

−	Review and Step 3 decision of received additional materials from ETSI (TC DECT) and DECT Forum, Doc. 5D/1299; Doc. Nufront, 5D/1300; Doc. TSDSI, 5D/1301 for section 4.3 of IMT-2020/YYY documents;

−	Review and develop IMT-2020/k to the received interim evaluation report(s);

−	Collect the intention of evaluations for certain candidate IMT-2020 technologies and decide next steps;

−	Discussion of detailed workplan and working document towards PDNR M.[IMT.OUTCOME].

During the meeting, the following carried forward and new input contributions were considered and presented:

–	Working document towards a template of IMT2020-ZZZ, Attachment 5.3 of Doc. 5D/1184

–	Detailed workplan for development of the Document IMT-2020/ZZZ, Attachment 5.4 of Doc. 5D/1184

–	Template for IMT-2020/k (Completed), 5D/TEMP/607

–	Self-evaluation results, Doc. 5D/1299, 12 (ETSI (TC DECT) and DECT Forum), Doc. 5D/1300 (Nufront), Doc. 5D/1301 (TSDSI), Docs. 5D/10 (ATIS), 4, 5 (ATIS IEG), Doc. 5D/22 (China)

–	Document for information, Doc. 5D/34 (Nokia)

–	Inputs from independent evaluation groups, Docs. 5D/1302, 3, 4, 5, 6, 7, 8, 9 (ATIS), 5D/1303, 28 (CEG), 5D/1304, 27 (5GIF), 5D/1305, 24 (ChEG), 5D/1306, 15 (TTA SPG33), 5D/1307, 26 (5GMF), 5D/1308, 11 (5G PPP, 5GIA), 5D/25 (TPCEG), 5D/29, 30 (TCOE), 5D/31, 32, 33 (AEG)

–	Proposals on detailed workplan and working documents towards PDNR M.[IMT.OUTCOME], 5D/17 (Japan, Korea), 5D/19, 20 (China)

Review of Self-Evaluation from the received submissions

Review of self-evaluation were proceeded during the meeting based on the received updated submissions in Docs. 5D/1299, 12 from ETSI (TC DECT), 5D/1300 from Nufront and 5D/1301 from TSDSI. The meeting had long time technique discussions based on the related input contributions, including the issues from the previous meeting and new issues from evaluation discussion area and online meeting. The meeting generated three excel files respectively for three submissions, including check list of all those issues, responses from the proponents and observations from SWG Evaluation. Also, conclusion had been reached to the submissions from ETSI (TC DECT) and TSDSI, that indicated both submissions meet the completeness of step 3 (section 4.3 in IMT-2020/YYY). Two observation Documents 5D/TEMP/16 and 5D/TEMP/14 for the submission from ETSI (TC DECT) and the submission from TSDSI were updated based on Docs. IMT-2020/26 and IMT-2020/28 created in the 32nd meeting, including the check list and considerations on supplementary materials as attachments.

However there was not conclusion reached for the submission from Nufront at the level of SWG Evaluation so that this submission, the excel file including its check list and the draft of observation document were delivered to WG Technology for further discussion.

Received interim evaluation reports and intention of evaluation from IEGs

Based on the agreed document structure of IMT-2020/k (Attachment 5.6 of Doc. 5D/1011), the meeting developed Docs. 5D/TEMP/1~10 according to the received interim evaluation reports. The meeting also clarified the questions raised during the dialogs between IEGs and the proponents. There is an issue about link budget results which has been discussed during the meeting and offline discussion between IEG-5GIF and Propoennt-3GPP. The clarification will be posted to Evaluation Group discussion area in the website of IMT-2020 submission and evaluation process. Also there is a contribution from Nokia, Doc. 5D/34, which provided technique information to the meeting. To facilitate the evaluation activities in IEGs, this document will also be posted to Evaluation Group discussion area.

The meeting also reviewed the intention of evaluation from IEGs. A table of summary is attached in this report for information.

Detailed workplan and working document towards PDNR M. [IMT-2020.OUTCOME]

The meeting created the draft detailed workplan and draft working document towards PDNR M.[IMT-2020.OUTCOME] based on the carried forward document and input contributions to this meeting. Two TEMP Documents, 5D/TEMP/29 and 5D/TEMP/30, were created accordingly and will be carried forward to the next meeting.

Documents for approval

5D/TEMP/1(Rev.1)	Evaluation report received from TPCEG on the candidate IMT-2020 Radio Interface Technology proposals (To be Doc. IMT-2020/8(Rev.1))

5D/TEMP/2(Rev.1)	Evaluation report received from TCOE on the candidate IMT-2020 Radio Interface Technology proposals (To be Doc. IMT-2020/9(Rev.1))

5D/TEMP/3(Rev.1)	Evaluation report received from ChEG on the candidate IMT-2020 Radio Interface Technology proposals (To be Doc. IMT-2020/10(Rev.1))

5D/TEMP/4(Rev.1)	Evaluation report received from 5GIF on the candidate IMT-2020 Radio Interface Technology proposals (To be Doc. IMT-2020/11(Rev.1))

5D/TEMP/5(Rev.1)	Evaluation report received from ATIS on the candidate IMT-2020 Radio Interface Technology proposals (To be Doc. IMT-2020/29)

5D/TEMP/6(Rev.1)	Evaluation report received from CEG on the candidate IMT-2020 Radio Interface Technology proposals (To be Doc. IMT-2020/30)

5D/TEMP/7(Rev.1)	Evaluation report received from TTA SPG33 on the candidate IMT-2020 Radio Interface Technology proposals (To be Doc. IMT-2020/31)

5D/TEMP/8(Rev.1)	Evaluation report received from 5GMF on the candidate IMT-2020 Radio Interface Technology proposals (To be Doc. IMT-2020/32)

5D/TEMP/9(Rev.1)	Evaluation report received from 5GIA on the candidate IMT-2020 Radio Interface Technology proposals (To be Doc. IMT-2020/33)

5D/TEMP/10(Rev.1)	Evaluation report received from AEG on the candidate IMT-2020 Radio Interface Technology proposals (To be Doc. IMT-2020/34)

5D/TEMP/14(Rev.1)	Observations of SWG Evaluation (Proponent TSDSI) (To be Doc. IMT-2020/28(Rev.1))

5D/TEMP/16(Rev.1)	Observations of SWG Evaluation (Proponent ETSI(TC DECT)) (To be Doc. IMT-2020/26(Rev.1))

Documents carried forward to next WP 5D meeting

5D/TEMP/30	[Draft] Working document towards PDNR ITU-R M.[IMT-2020.OUTCOME]

5D/TEMP/29	Detailed workplan for development of draft new Report ITU-R M.[IMT-2020.OUTCOME]

The SWG Evaluation chair would like to thank all the participants for their patience, cooperation and excellent supports during the meeting sessions.

# Ying Peng 
	Chair, SWG Evaluation

Attachment 5.7

Source:	Document 5D/TEMP/29

[Draft] detailed workplan for development of draft new
Report ITU-R M.[IMT-2020.OUTCOME]

| Title | The outcome of the evaluation, consensus building and decision of the IMT-2020 process (Steps 4 to 7), including characteristics of IMT-2020 radio interfaces |
| --- | --- |
| Identifier | Report ITU-R M.[IMT-2020.OUTCOME] |
| Document Type | ITU-R Report |
| ITU-R WP 5D Lead Group | WG Technology Aspects |
| Sub-working group | SWG Evaluation |
| Sub-working group Co-Chairs | Ms. Ying PENG; E-mail: [pengying@catt.cn] Mr. Jungsoo JUNG; E-mail: [jungsoo@samsung.com] |
| Focus for scope and work | This Report is the record of the work performed after receipt of complete proposals for IMT-2020 candidate RITs and SRITs, including evaluation activity and consensus building. This text contains outcome and conclusions of Steps 4-7 of the IMT-2020 process. These steps correspond to: – Step 4: Evaluation of candidate RITs or SRITs by Independent Evaluation Groups. – Step 5: Review and coordination of outside evaluation activities. – Step 6: Review to assess compliance with minimum requirements. – Step 7: Consideration of evaluation results, consensus building and decision. The details of these steps are provided in Document IMT-2020/2(Rev.2). |
| Related documents | [Editor note: IMT-2020/YYY will be updated when step 3 is done for IMT-2020/17,18,19] [Editor note: IMT-2020/ZZZ will be updated next Feb. when step 4 is done] Report ITU-R M.2410 Report ITU-R M.2411 Report ITU-R M.2412 IMT-2020/2(Rev.2) IMT-2020/13,14,15,16, [17, 18, 19] IMT-2020/ZZZ |
| Milestones | Meeting No. 33 (December 2019, Geneva, Switzerland) 1 Review the interim evaluation report(s) from IEG(s). 2 Co-work with SWG Coordination for exceptional cases on completeness of RIT/SRIT submissions. 3 Develop a detailed work plan. 4 Create a working document towards PDN Report ITU-R M.[IMT-2020.OUTCOME]. Meeting No. 34 (February 2020, Geneva, Switzerland) 1 Review the work plan if necessary. 2 Review the final evaluation report(s) from IEG(s). 3 Continue developing the working document towards PDN Report ITU-R M.[IMT-2020.OUTCOME] and Promote the working document to a preliminary draft new Report. 4 Provide necessary information to SWG Coordination and SWG IMT Specifications to facilitate the relative work, e.g. preliminary assessments from Evaluation Report Summaries. Meeting No. 35 (June 2020, [TBD]) 1 Finalize the draft new Report ITU-R M.[IMT-2020.OUTCOME] and send it to SG 5 for approval. 2 Provide final decision and necessary information to SWG Coordination and SWG IMT Specifications to facilitate the related work. |

Attachment 5.8

Source:	Document 5D/TEMP/30

[Draft] Working document towards preliminary draft new Report ITU-R M.[IMT-2020.OUTCOME]

The outcome of the evaluation, consensus building and decision of the IMT-2020 process (Steps 4 to 7), including characteristics of IMT-2020 radio interfaces

[Editor’s note: the yellow highlighted texts are related to the submissions which WP 5D at 32nd meeting did not yet determine whether they are “complete” per Section 5 of Report ITU-R M.2411. Further considerations are required according to conclusions on the submissions at 33rd WP 5D meeting.]

[Editor’s note: The differences from Doc. 5D/17 are in green, and from Doc. 5D/20 in red.]

# 1	Introduction

[Editor’s note: this section includes general description and key features of IMT-2020 systems.]

xxx

Resolution ITU-R 56 defines a new term “IMT-2020” applicable to those systems, system components, and related aspects that provide far more enhanced capabilities than those described in Recommendation ITU-R M.1645.

In this regard, International Mobile Telecommunications-2020 (IMT-2020) systems are mobile systems that include the new capabilities of IMT that go beyond those of IMT-Advanced. Recommendation ITU-R M.2083 “IMT Vision – Framework and overall objectives of the future development of IMT for 2020 and beyond” identifies capabilities for IMT-2020 which would make IMT-2020 more efficient, fast, flexible, and reliable when providing diverse services in the intended usage scenarios.

The usage scenario of IMT-2020 will extend to enhanced mobile broadband (eMBB), massive machine type communications (mMTC) and ultra-reliable and low latency communications (URLLC).

IMT-2020 systems support low to high mobility applications and much enhanced data rates in accordance with user and service demands in multiple user environments. IMT-2020 also has capabilities for enabling massive connections for a wide range of services, and guarantee ultra-reliable and low latency communications for future deployed services even in critical environments.

The capabilities of IMT-2020 include:

–	very high peak data rate;

–	very high and guaranteed user experienced data rate;

–	quite low air interface latency;

–	quite high mobility while providing satisfactory quality of service;

–	enabling massive connection in very high density scenario;

–	very high energy efficiency for network and device side;

–	greatly enhanced spectral efficiency;

–	significantly larger area traffic capacity;

–	high spectrum and bandwidth flexibility;

–	ultra high reliability and good resilience capability;

–	enhanced security and privacy.

These features enable IMT-2020 to address evolving user and industry needs.

The capabilities of IMT-2020 systems are being continuously enhanced in line with user and industry trends, and consistent with technology developments.

[Editor’s note: Also, this section includes the information from IMT-2020 process.]

The first invitation for the submission of proposals for candidate radio interface technologies (RITs) or a set of RITs (SRITs) for the terrestrial components of IMT-2020 was announced by/issued with Circular Letter 5/LCCE/59 on 22 March 2016. The Circular Letter and its addenda, initiated an ongoing process to evaluate the candidate RITs or SRITs for IMT-2020, inviting the formation of independent evaluation groups and the subsequent submission of evaluation reports on these candidate RITs or SRITs.

The submission and evaluation process for IMT-2020 development is included in the Revision 2 to Document IMT-2020/2 /the Document IMT-2020/2 (Rev.2) and is illustrated in Fig. 2/ Fig. 1, reproduced here for reference in understanding the steps of the IMT-2020 process. The requirements, evaluation criteria and submission templates for the development of IMT-2020 are included in Report ITU-R M.2411. The minimum requirements related to technical performance for IMT-2020 radio interface(s) are included in Report ITU-R M.2410. The guidelines for evaluation of radio interface technologies for IMT-Advanced are included in Report ITU-R M.2412.(Hyperlink embedded)

# 2	Scope

This Report is the record of the work performed after receipt of the complete proposals for IMT-2020 candidate RITs and SRITs, including the evaluation activity and the consensus building. This text contains the outcome and conclusions of Steps 4-7 of the IMT-2020 process. These steps correspond to:

–	Step 4: Evaluation of candidate RITs or SRITs by Independent Evaluation Groups.

–	Step 5: Review and coordination of outside evaluation activities.

–	Step 6: Review to assess compliance with minimum requirements.

–	Step 7: Consideration of evaluation results, consensus building and decision.

The details of these steps are provided in Document IMT-2020/2(Rev.2). (Hyperlink embedded)

This Report also states the decisions reached by ITU-R on each of the candidate proposals and provides the technical characteristics of the RITs and SRITs for IMT-2020.

Note that the actual specifications of the agreed IMT-2020 radio interfaces will be contained in a future Recommendation which is being developed.

Figure 2

IMT-2020 terrestrial component radio interface development process

IMT-2020 2-02

IMT-2020 2-02

Step 1

Circular Letter to invite proposals for radio interface technologies and evaluations

Step 2

Development of candidate radio interface technologies

Step 5

Review and coordination of outside evaluation activities

Step 6

Review to assess compliance with minimum requirements

Step 7

Consideration of evaluation results, consensus building, and decision

Descriptions of proposed radio interface technologies and evaluation reports

Step 8

Development of radio interface Recommendation(s)

Radio interface specifications (SPECS), sufficiently detailed to enable worldwide compatibility

Step 9

Implementation of Recommendation(s)

Step 4

Evaluation of candidate radio interface technologies by independent evaluation groups, grouping of the technologies through consensus building

Coordination between independent evaluation groups

ITU

-

R

Outside ITU

-

R

Step 3

Submission/Reception of the RIT and SRIT proposals and acknowledgement of receipt

Step 1

Circular Letter to invite proposals for radio interface technologies and evaluations

Step 2

Development of candidate radio interface technologies

Step 5

Review and coordination of outside evaluation activities

Step 6

Review to assess compliance with minimum requirements

Step 7

Consideration of evaluation results, consensus building, and decision

Descriptions of proposed radio interface technologies and evaluation reports

Step 8

Development of radio interface Recommendation(s)

Radio interface specifications (SPECS), sufficiently detailed to enable worldwide compatibility

Step 9

Implementation of Recommendation(s)

Step 4

Evaluation of candidate radio interface technologies by independent evaluation groups, grouping of the technologies through consensus building

Coordination between independent evaluation groups

ITU

-

R

Outside ITU

-

R

Step 3

Submission/Reception of the RIT and SRIT proposals and acknowledgement of receipt

# 3	Related text references

[Editor’s note: to complement the necessary and related text references.]

–	Recommendation ITU-R M.1036	–	Frequency arrangements for implementation of the terrestrial component of International Mobile Telecommunications (IMT) in the bands identified for IMT in the Radio Regulations

–	Recommendation ITU-R M.2083	–	IMT vision -Framework and overall objectives of the future development of IMT-2020 and beyond

–	Recommendation ITU-R M.1822–	Framework for services supported by IMT

–	[Recommendation ITU-R M. …]–	Detailed specifications of the radio interfaces of IMT-2020

–	Report ITU-R M.2320	–			Future technology trends of terrestrial IMT systems

–	Report ITU-R M.2370	–			IMT traffic estimates for the years 2020-2030

–	Report ITU-R M.2376	–		Technical feasibility of IMT in bands above 6 GHz

–	Report ITU-R M.2411	–			Requirements, evaluation criteria and submission templates for the development of IMT-Advanced

–	Report ITU-R M.2410	–			Requirements related to technical performance for IMT-Advanced radio interface(s)

–	Report ITU-R M.2412	–			Guidelines for evaluation of radio interface technologies for IMT-Advanced

–	Resolution ITU-R 56	–			Naming for International Mobile Telecommunications

–	Resolution ITU-R 65	–			Principles for the process of development of IMT for 2020 and beyond

–	Doc. IMT-2020/1	–			Background on IMT-Advanced

–	Doc. IMT-2020/2(Rev.2)	–		Submission and evaluation process and consensus building

–	Doc. IMT-2020/20	–			Process and the use of Global Core Specification (GCS), references, and related certifications in conjunction with Recommendation ITU-R M.[IMT.RSPEC]

## 3.1	List of acronyms and abbreviations

[Editor’s note: to complement the necessary abbreviations.]

[Editor’s note:  Additional acronyms to be added.]

IMT	International Mobile Telecommunications

RIT	Radio interface technology

SRIT	Set of radio interface technologies

TDD	Time division duplex

FDD	Frequency division duplex

# 4	Summary of submissions

[Editor’s note: the submissions in square brackets will be reviewed in the meeting #33]

Following the guidelines of the IMT-2020 process, the candidate technology submissions provided to the ITU-R in July [and December 2019] [with additional information provided by September 10 2019]  were reviewed and the following were acknowledged as “complete” candidate technology submissions as per § 5 of Report ITU-R M.2411:

–	IMT-2020/13 – Acknowledgement of candidate SRIT submission from 3GPP proponent under step 3 of the IMT-2020 process.

–	IMT-2020/14 – Acknowledgement of candidate RIT submission from 3GPP proponent under step 3 of the IMT-2020 process.

–	IMT-2020/15 – Acknowledgement of candidate RIT submission from China (People’s Republic of) under step 3 of the IMT-2020 process.

–	IMT-2020/16 – Acknowledgement of candidate RIT submission from Korea (Republic of) under Step 3 of the IMT-2020 process

–	[IMT-2020/17 – Acknowledgement of candidate SRIT submission from ETSI (TC DECT) and DECT Forum under step 3 of the IMT-2020 process.] (highlighted in yellow)

–	[IMT-2020/18 – Acknowledgement of candidate RIT submission from Nufront under Step 3 of the IMT-2020 process] (highlighted in yellow)

–	[IMT-2020/19 – Acknowledgement of candidate RIT submission from TSDSI under step 3 of the IMT-2020 process.] (highlighted in yellow)

For convenience, these submissions are attached to Annex 1 of this Report.

[Editor’s note: The important issues discussed during the submission and its process in ITU will be recorded in this section.]

xxx

# 5	Conclusion for Steps 4 to 7

## 5.1	Results of Steps 4, “Evaluation of candidate RITs or SRITs by Evaluation Groups” and Step 5, “Review and coordination of outside evaluation activities”

[Editor’s note: this section is to describe the work and results under step 4 and 5, which should be done at Feb. 2020.]

[Editor’s note: Describe the summary of evaluation activities using IMT-2020/ZZZ documents and other coordination activities.]

These evaluation reports from the respective Independent Evaluation Groups are included in Annex 2 of this Report.

### 5.1.1	Summary of the evaluations received for the 3GPP technology candidate

[Editor’s note: “technically identical” conclusions was done in WP 5D #32 meeting, as below.]

It was concluded by ITU-R that the IMT-2020/15 candidate technology proposal was technically identical to the IMT-2020/14 candidate technology proposal and NB-IoT part of IMT-2020/13 candidate technology proposal, so that a single evaluation of IMT-2020/14 and a single evaluation of NB-IoT part of IMT-2020/13 would also be applicable to the candidate technology proposal IMT-2020/15.

It was concluded that IMT-2020/16 candidate technology proposal was technically identical to the IMT-2020/14 candidate technology proposal, so that a single evaluation would be applicable to both technologies.

[Editor’s note: to record the details of Steps 4 and 5 and the results, e.g. the received evaluation reports indicates the evaluation groups are of the opinion that the candidate RIT/SRIT proposals meet the minimum requirements of all required test environments.]

xxx

[Editor’s note: to list all final evaluation reports.]

xxx

[Editor’s note: the following sections need further checking after completion of section 4 and the received evaluation reports.]

xxx

### 5.1.1	Summary of the evaluations received for the candidate SRIT submission from 3GPP Proponent

### 5.1.2	Summary of the evaluations received for the candidate RIT submission from 3GPP Proponent

### 5.1.3	Summary of the evaluations received for the candidate RIT submission from China (People’s Republic of)

### 5.1.4	Summary of the evaluations received for the candidate RIT submission from Korea (Republic of)

### [5.1.2	Summary of the evaluations received for the [ETSI (TC DECT) and DECT Forum] technology candidate

It was concluded by ITU-R that “3GPP 5G NR” RIT as one component RIT of the IMT-2020/17 candidate technology proposal is technically identical to the IMT-2020/14 candidate technology proposal, so that a single evaluation would be applicable to both technologies.

(highlighted in yellow in Doc. 5D/xxxx)

### 5.1.3	Summary of the evaluations received for the [TSDSI] technology candidate

(highlighted in yellow in Doc. 5D/xxxx)

### 5.1.4	Summary of the evaluations received for the [Nufront] technology candidate]

(highlighted in yellow in Doc. 5D/xxxx)

## 5.2	Results of Step 6, “Review to assess compliance with minimum requirements”

[Editor’s note: Describe the summary of assessment after evaluation activities by WP 5D.]

Under Step 6 of the IMT-2020 process and guidelines, an assessment of each proposal was made as to whether it met a version of the minimum technical requirements and evaluation criteria of the IMT-2020 process in force as described in Report ITU-R M.2411. The evaluation methodology is described in Report ITU-R M.2412. The version of the minimum technical requirements used is described in Report ITU-R M.2410.

In this step, the evaluated proposal for an RIT/SRIT is assessed as a qualifying RIT/SRIT, if an RIT/SRIT fulfills the minimum requirements for the five test environments comprising the three usage scenarios.

Such a qualified RIT/SRIT will go forward for further consideration in Step 7.

Based on a review of the evaluations carried out by the Independent Evaluation Groups as well as the self-evaluations from the proponents, the conclusions of the ITU-R for Step 6 are presented in the following sub-sections. Each candidate technology submission is separately addressed for compliance with regard to services, spectrum and technical performance and for confirmation as a qualifying RIT/SRIT:

[Editor’s note: The followings are examples, which will be filled after the completion of Step 6.]

–	[Section 5.2.1 Candidate submission from 3GPP (IMT-2020/13).]

–	[Section 5.2.2 Candidate submission from 3GPP (IMT-2020/14).]

–	[Section 5.2.3 Candidate submission from China (People’s Republic of) (IMT-2020/15).]

–	[Section 5.2.4 Candidate submission from Korea (IMT-2020/16).]

–	[Section 5.2.5 Candidate submission from ETSI(TC DECT) and DECT Forum (IMT-2020/17).]

–	[Section 5.2.6 Candidate submission from TSDSI  (IMT-2020/19).]

–	[Section 5.2.6 Candidate submission from Nufront (IMT-2020/xx).]

### 5.2.1	Candidate submission from 3GPP (IMT-2020/13)

[Editor’s note: The followings are examples, which will be filled after the completion of Step 6.]

[The ITU-R summary view of the candidate technology submission from the 3GPP (IMT-2020/13) is presented below. The individual detailed analysis of compliance for each of the defined items in Report ITU-R M.1411 is included in the tables in Annex 3.

Compliance related to services

The 3GPP technology proposed by 3GPP (IMT-2020/13)  meets the minimum requirements for services.

Compliance related to spectrum

The 3GPP technology proposed by 3GPP (IMT-2020/13) meets the minimum requirements for spectrum.

Compliance related to technical performance

The 3GPP technology proposed by 3GPP (IMT-2020/13) meets the minimum requirements for technical performance.

Assessment of the candidate technology proposal as a qualifying RIT/SRIT

ITU-R confirms that the SRIT of the candidate technology submission in IMT-2020/13 meets the minimum requirements of the Indoor Hotspot-eMBB test environment.

ITU-R confirms that the SRIT of the candidate technology submission in IMT-2020/13 meets the minimum requirements of the Dense Urban-eMBB test environment.

ITU-R confirms that the SRIT of the candidate technology submission in IMT-2020/13 meets the minimum requirements of the Rural-eMBB test environment.

ITU-R confirms that the SRIT of the candidate technology submission in IMT-2020/13 meets the minimum requirements of the Urban Macro-mMTC test environment.

ITU-R confirms that the SRIT of the candidate technology submission in IMT-2020/13 meets the minimum requirements of the Urban Macro-URLLC test environment.

Therefore, the 3GPP technology proposed by 3GPP (IMT-2020/13) meets the requirements of a qualifying SRIT and moves forward for consideration in Step 7.]

### [5.2.2	Candidate submission from 3GPP (IMT-2020/14)]

### [5.2.3	Candidate submission from China (People’s Republic of) (IMT-2020/15)]

### [5.2.4	Candidate submission from Korea (IMT-2020/16)]

### [5.2.5	Candidate submission from ETSI(TC DECT) and DECT (IMT-2020/17)]

### [5.2.6	Candidate submission from Nufront (IMT-2020/18)]

### [5.2.7	Candidate submission from TSDSI (IMT-2020/19)]

（There are highlights in yellow from 5.2.5-5.2.7 in Doc. 5D/xxxx）

## 5.3	Result of Step 7, “Consideration of evaluation results, consensus building and decision”

[Editor’s note: Describe the evaluation results, summary of consensus building and decision towards Step 8]

### 5.3.1	Consideration of evaluation results

xxx

### 5.3.2	Consensus building and decision

As concluded in §§ 5.1.1 and 5.1.2 above:

–	IMT-2020/15 candidate technology proposal was technically identical to the IMT-2020/14 candidate technology proposal and NB-IoT part of IMT-2020/13 candidate technology proposal;

–	IMT-2020/16 candidate technology proposal was technically identical to the IMT-2020/14 candidate technology proposal;

–	[“3GPP 5G NR” RIT as one component RIT of the IMT-2020/17 candidate technology was technically identical to the IMT-2020/14 candidate technology proposal;]

[Editor’s note: the work and results of the consensus building in ITU-R will be described in this sections with e.g. necessary grouping. The conclusion of ITU-R will also be made in this section.]

xxx

# 6	Characteristics of the A technologies and basis of the specifications for Step 8

[Editor’s note: Describe the characteristics of selected IMT-2020 technologies]

In Step 8, a (set of) IMT-2020 terrestrial component radio interface Recommendation(s) is developed within the ITU-R on the basis of the results of Step 7, sufficiently detailed to enable worldwide compatibility of operation and equipment, including roaming.

## 6.1	Characteristics of radio interface technologies for IMT-2020

[Based on the consensus views in § 5.3, “xxxx” is accepted for Step 8. The basis for specifying the “xxxx” technology in Step 8 is Document IMT-2020/YYY.

As provided for in Document IMT-2020/20 “Process and the use of Global Core Specification (GCS), References, and Related Certifications in Conjunction with Recommendation ITU-R M.[IMT-2020.SPECS])”, the GCS Proponent for the “xxxx” in Step 8 is “XXXX”.]

## 6.2	Detailed specifications for the radio interface technologies for IMT-2020 in Step 8

Under Step 8 of the IMT-2020 process the detailed technical specifications for the Terrestrial Radio Interface Technologies in IMT-2020 will be provided in a Recommendation to be developed (“Detailed specifications of the radio interfaces of IMT-2020”).

### 6.2.1	For A

xxx

Annex 1

Submission of candidate technologies

[Editor’s note: Attaches IMT-2020/13~19 documents]

Doc. IMT-2020/13 – Acknowledgement of candidate SRIT submission from 3GPP proponent under step 3 of the IMT-2020 process.

Doc. IMT-2020/14 – Acknowledgement of candidate RIT submission from 3GPP proponent under step 3 of the IMT-2020 process.

Doc. IMT-2020/15 – Acknowledgement of candidate RIT submission from China (People’s Republic of) under step 3 of the IMT-2020 process.

Doc. IMT-2020/16 – Acknowledgement of candidate RIT submission from Korea (Republic of) under Step 3 of the IMT-2020 process

[Editor’s note: the submissions in square brackets will be reviewed according to WP 5D  #33 meeting.]

[Doc. IMT-2020/17 – Acknowledgement of candidate SRIT submission from ETSI (TC DECT) and DECT Forum under Step 3 of the IMT-2020 process.]

[Doc. IMT-2020/18 – Acknowledgement of candidate RIT submission from Nufront under Step 3 of the IMT-2020 process]

[Doc. IMT-2020/19 – Acknowledgement of candidate RIT submission from TSDSI under Step 3 of the IMT-2020 process.]

Observations of self-evaluation

Doc. IMT-2020/23 – Observations of SWG Evaluation (3GPP Proponent)

Doc. IMT-2020/24 – Observations of SWG Evaluation (Proponent China)

Doc. IMT-2020/25 – Observations of SWG Evaluation (Proponent Korea)

[Editor’s note: the insert of the observation documents will be according to WP 5D #33 meeting.]

Doc. [IMT-2020/26 - Observations of SWG Evaluation (Proponents ETSI (TC DECT) and DECT Forum)

Doc. IMT-2020/27 - Observations of SWG Evaluation (Proponent Nufront)

Doc. IMT-2020/28 - Observations of SWG Evaluation (Proponent TSDSI)

]

[Editor’s note: the additional observations can be added according to WP 5D #33 meeting.]

xxx

Annex 2

Summary and details of Evaluation Reports
from Independent Evaluation Groups

[Editor’s note: Attaches IMT-2020/ZZZ documents.]

Annex 3

Detailed compliance template summaries

[Editor’s note: Describe the summary of compliance templates after assessment]

A	Candidate submission from 3GPP Proponent (IMT-2020/13)

B	Candidate submission from 3GPP Proponent (IMT-2020/14)

C	Candidate submission from China (People’s Republic of) (IMT-2020/15)

D	Candidate submission from Korea (Republic of) (IMT-2020/16)

E	Candidate submission from ETSI (TC DECT) and DECT Forum (IMT-2020/17)

F	Candidate submission from Nufront (IMT-2020/18)

G	Candidate submission from TSDSI (IMT-2020/19)

_______________

____________________

If the GCS Proponent and the Transposing Organization consist of same organization, the GCS Proponent and the Transposing Organization can submit all required materials for updating its Section including Transposition References and Certification C by meeting X+2A or X+B.

If the GCS Proponentrnal (potential GCS Proponent) decides to use DIS style, it doesn’t need to submit List of Global Core Specifications but needs to submit full materials for describing its RIT/SRIT in the Recommendation and Form B.

The latest edition of the Recommendation/Report in force should be used.

[Recommendations ITU-R M.2012 and ITU-R M.[IMT-2020.SPECS] are two separate, independent, and self-contained Recommendations, each one with a specific scope. Both Recommendations will evolve independently, and there could be some overlap reflected by commonality in content between the two documents.]

Developed by [name of proponent] as [name of proposed RIT/SRIT].

Developed by [name of proponent] as [name of proposed RIT/SIRT].

Developed by [name of proponent] as [name of proposed RIT/SRIT].

Developed by [name of proponent] as [name of proposed RIT/SRIT].

Global Core Specifications.

Radio Interface Technology.

Set of Radio Interface Technologies.

A “GCS” (Global Core Specification) is the set of specifications that defines a single RIT, an SRIT, or a component RIT within an SRIT.

The following identified Transposing Organizations have provided their transposed sets of standards information contained in this section:

–	[Name of Transposing Organization 1].

–	[Name of Transposing Organization 2].

–	.

–	.

–	[Name of Transposing Organization m].

Developed by [name of proponent] as [name of proposed RIT/SRIT].

Global Core Specifications.

Radio Interface Technology.

Set of Radio Interface Technologies.

IMT-2020 documents referred to in this Report are found on the ITU-R web page: “IMT-2020 submission and evaluation process (https://www.itu.int/en/ITU-R/study-groups/rsg5/rwp5d/imt-2020/Pages/submission-eval.aspx)”.

Once the Recommendation has been developed the exact reference to the Recommendation will be added as an editorial action.

Once the Recommendation has been developed the exact reference to the Recommendation will be added as an editorial action.

“3GPP 5G NR” RIT is “3GPP 5G candidate for inclusion in IMT-2020: Submission 2 for IMT-2020 (RIT)” in Document IMT-2020/14.

“3GPP 5G NR” RIT is “3GPP 5G candidate for inclusion in IMT-2020: Submission 2 for IMT-2020 (RIT)” in Document IMT-2020/14.

Once the Recommendation has been developed the exact reference to the Recommendation will be added as an editorial action.

Reference Report ITU-R M.2411, § 5.2.4.