IMT-2020: Evaluation of area traffic capacity (NR) Document for discussion and acceptance CEG meeting Montreal, Canada August 8th, 2019 Source : Interdigital Inc.

# Abbreviations

3GPP: 3rd Generation Partnership Project

AP: Access point

BS: Base-station

BW: Bandwidth

CCE: Control channel element

CP: Control-plane or Cyclic Prefix

CSI: Channel state information

CSI-RS: Channel state information reference signal

DU: Dense Urban

DL: Downlink

DMRS: Demodulation reference signal

eMBB: enhanced mobile broadband

FDM: Frequency division multiplexing

gNB: g-NodeB

GoS: Grade-of-service

HARQ: Hybrid automatic repeat request

InH: Indoor Hotspot

ITU: International Telecommunication Union

LDPC: Low-density parity code

LMLC: low-mobility large-cell

LoS: Line-of-sight

MAC: Medium access control

MIMO: multiple-input multiple-output

mMTC: massive Machine-Type Communications

NB-IoT: Narrowband-Internet of Things

NLoS: non-Line-of-sight

NR: New Radio

OFDM: Orthogonal frequency division multiplexing

PDCCH: Physical downlink control channel, PDSCH: Physical downlink shared channel

PDCP: Packet data convergence protocol

PRB: Physical resource block

PT-RS: Phase tracking reference signal

PUCCH: Physical uplink control channel; PUSCH: Physical uplink shared channel

QoS: Quality-of-service

RAN: Radio access network

RB: Resource block

RE: Resource element

RIT: Radio-interface technology

RLC: Radio link control

RU: Rural

SCS: Sub-carrier spacing

SDU: Service data unit

SINR/SNR: Signal-to-interference noise ratio/Signal-to-noise ratio

SR: Scheduling request

SRS: Sounding reference symbol

SSB: Synchronization signal block

SCM: Stochastic channel model

TBS: Transport block size

TRxP: Transmission and reception point

TRS: Tracking reference signal

TTI: Transmission time interval

UE: User equipment

UL: Uplink

UP: User-plane

urLLC: ultra-reliable low-latency communications

# Introduction

In this contribution, we present evaluation results on area traffic capacity for the enhanced Mobile Broadband (eMBB) usage scenario in an indoor hotspot (InH) geographic environment, as described in [3] and shows that the corresponding IMT-2020 requirements can be met.

# Requirements on area traffic capacity

Area traffic capacity is defined as the total traffic throughput served per geographic area (in Mbits/s/m2). The throughput is the number of correctly received bits, i.e. the number of bits contained in the service data units delivered to layer 3, over a certain period of time.

This can be derived for one frequency band and one transmission reception point (TRxP) layer, based on the achievable average spectral efficiency, network deployment (e.g., TRxP (site) density) and bandwidth.

Let W denote the channel bandwidth and ρ the TRxP density (TRxP/m2). The area traffic capacity Carea is related to average spectral efficiency SEavg through the following equation

Carea=ρ W SEavg.

In case multiple bands are aggregated, the area traffic capacity will be summed over the bands.

The requirement is defined for the purpose of evaluation in the Indoor Hotspot (InH) eMBB test environment, where the target value for the area traffic capacity on the downlink is 10 Mbits/s/m2.

The Indoor Hotspot-eMBB test environment consists of one floor of a building. The height of the ceiling is 3 m. The floor has a surface of 120 m × 50 m and 12 BSs/sites which are placed in 20 meters spacing as shown in Fig. 1, with a LOS probability as defined by channel model in Annex 1, Table A1-9 of [3]. In Fig. 1, internal walls are not explicitly shown but are modeled via the stochastic LOS probability model.

The type of site deployed (e.g. one TRxP per site or 3 TRxPs per site) is not defined and should be reported by the proponent.

Fig. 1

Indoor Hotspot sites layout

If we take 12 TRxP in the above scenario, then we can compute ρ as follows:

ρ = 12 / (120m X 50m) = 0.002 TRxP/m2

For FDD with DL with 32x4 MU-MIMO Type II Codebook, and SCS = 15KHz the average spectrum efficiency has been derived as:

Channel Model A: SEavg = 13.24 for 40MHz carrier bandwidth.

Channel Model B: SEavg = 13.54 for 40MHz carrier bandwidth.

For this FDD configuration, using a 400MHz aggregation bandwidth we will have:

Channel Model A:

Carea = 0.002 X 400MHz X 13.24 = 10.59 Mbits/s/Hz

Channel Model B:

Carea = 0.002 X 400MHz X 13.54 = 10.83 Mbits/s/Hz

Observation 1: For FDD configuration the minimum requirement for area traffic capacity can be met with a minimum aggregated channel bandwidth of 400MHz.

For TDD with DL with 32x4 MU-MIMO Type II Codebook reciprocity based, 4T SRS, SCS = 15KHz and DDDSU frame structure, the average spectrum efficiency has been derived as:

Channel Model A: SEavg = 14.65 for 40MHz carrier bandwidth.

Channel Model B: SEavg = 14.64 for 40MHz carrier bandwidth.

So, for the above TDD configuration with 360MHz aggregated bandwidth we will find:

Channel Model A:

Carea = 0.002 X 360 MHz X 14.65 = 10.54 Mbits/s/Hz

Channel Model B:

Carea = 0.002 X 360 MHz X 14.64 = 10.54 Mbits/s/Hz

Observation 2: For TDD configuration the minimum requirement for area traffic capacity can be met with a minimum aggregated channel bandwidth of 360MHz.

# Summary and conclusion

In this contribution, it has been shown that the IMT-2020 requirements on area traffic capacity can be fulfilled in the specified test environment. In the Annex we present a text proposal for CEG report.

# References

- R15-WP5D-C-1050 WP-5D meeting, Fukuoka, Japan

- ITU-R M.2410 IMT-2020.TECH PERF REQ

- ITU-R M.2412 IMT-2020.EVAL

Annex

Text proposal for CEG report on Energy Efficiency requirement

### 1 Conclusion: CEG concluded that traffic area requirement is met by the IMT-2020 3GPP submission.

2. Verification:

The requirement is defined for the purpose of evaluation in the Indoor Hotspot (InH) eMBB test environment, where the target value for the area traffic capacity on the downlink is 10 Mbits/s/m2.

The Indoor Hotspot-eMBB test environment consists of one floor of a building. The height of the ceiling is 3 m. The floor has a surface of 120 m × 50 m and 12 BSs/sites which are placed in 20 meters spacing as shown in Fig. 1, with a LOS probability as defined by channel model in Annex 1, Table A1-9 of [3]. In FIG. 1, internal walls are not explicitly shown but are modeled via the stochastic LOS probability model.

The type of site deployed (e.g. one TRxP per site or 3 TRxPs per site) is not defined and should be reported by the proponent.

Fig. 1

Indoor Hotspot sites layout

If we take 12 TRxP in the above scenario, then we can compute ρ as follows:

ρ = 12 / (120m X 50m) = 0.002 TRxP/m2

For FDD with DL with 32x4 MU-MIMO Type II Codebook, and SCS = 15KHz the average spectrum efficiency has been derived as:

Channel Model A: SEavg = 13.24 for 40MHz carrier bandwidth.

Channel Model B: SEavg = 13.54 for 40MHz carrier bandwidth.

For this FDD configuration, using a 400MHz aggregation bandwidth we will have:

Channel Model A:

Carea = 0.002 X 400MHz X 13.24 = 10.59 Mbits/s/Hz

Channel Model B:

Carea = 0.002 X 400MHz X 13.54 = 10.83 Mbits/s/Hz

Observation 1: For FDD configuration the minimum requirement for area traffic capacity can be met with a minimum aggregated channel bandwidth of 400MHz.

For TDD with DL with 32x4 MU-MIMO Type II Codebook reciprocity based, 4T SRS, SCS = 15KHz and DDDSU frame structure, the average spectrum efficiency has been derived as:

Channel Model A: SEavg = 14.65 for 40MHz carrier bandwidth.

Channel Model B: SEavg = 14.64 for 40MHz carrier bandwidth.

So, for the above TDD configuration with 360MHz aggregated bandwidth we will find:

Channel Model A:

Carea = 0.002 X 360 MHz X 14.65 = 10.54 Mbits/s/Hz

Channel Model B:

Carea = 0.002 X 360 MHz X 14.64 = 10.54 Mbits/s/Hz

Observation 2: For TDD configuration the minimum requirement for area traffic capacity can be met with a minimum aggregated channel bandwidth of 360MHz.