IMT-2020 Evaluation:

Link Budget analysis text proposal

for IMT-2020 3GPP submission

Document for discussion and acceptance

CEG meeting

Montreal

July 3rd , 2019

Source: Interdigital Inc., TELUS

# Abbreviations

3GPP: 3rd Generation Partnership Project

BLER: Block error rate

BS: Base-station

BW: Bandwidth

CCE: Control channel element

CP: Control-plane

CSI: Channel state information

CSI-RS: Channel state information reference signal

DU: Dense urban

DL: Downlink

DM-RS: Demodulation reference signal

eMBB: enhanced mobile broadband

gNB: g-NodeB

GoS: Grade-of-service

HARQ: Hybrid automatic repeat request

InH: Indoor Hotspot

ITU: International Telecommunication Union

LMLC: low-mobility large-cell

MIMO: multiple-input multiple-output

mMTC: massive Machine-Type Communications

NR: New Radio

NB-IoT: Narrowband-Internet of Things

OLLA: outer loop link adaptation

PDCCH: Physical downlink control channel, PDSCH: Physical downlink shared channel

PRB: Physical resource block

PT-RS: Phase tracking reference signal

PUCCH: Physical uplink control channel; PUSCH: Physical uplink shared channel

QoS: Quality-of-service

RAN: Radio access network

RE: Resource element

RIT: Radio-interface technology

RU: Rural

SCM: Stochastic channel model

SCS: Sub-carrier spacing

SDU: Service data unit

SR: Scheduling request

SRS: Sounding reference symbol

SSB: Synchronization signal block

TRxP: Transmission and reception point

TRS: Tracking reference signal

TTI: Transmission time interval

UE: User equipment

UL: Uplink

ULPC: Uplink power control

UP: User-plane

urLLC: ultra-reliable low-latency communications

1 Introduction

### This contribution provides a proposed text for the link budget analysis component of the CEG IMT-2020 Evaluation Report.

Link Budget Analysis text proposal

Link budget calculation is an important network planning tool that efficiently provides a first order approximation of cell coverage for a given level of service (and vice versa) and enables comparing the performance of different frequency bands during the network planning phase. One of the main development targets of the 3GPP based NR technology was to match or exceed the link budget of IMT-Advanced technologies. As part of the Canadian Evaluation Group (CEG) study, the calculations provided by 3GPP have been verified to determine whether the IMT-2020 targets would be met by the NR technology.

Inspection of the link budget template tables provided by 3GPP clearly shows that they are well prepared, cover the considered deployment scenarios and are appropriate for link budget evaluation. Further, it has been verified that all setup parameters for the deployment scenarios under consideration are within the ranges suggested by the ITU-R WP5D in the M.2411 and M.2412 documents.

Focus of the verification efforts was centred on deriving the shadow fading margins, penetration margins and data rate to signal to interference (SINR) mapping as these values have been used in the tables without providing sufficient details. For both considered channel models (Channel Model A and B), the theoretical derivation and numerical calculations, confirm that the shadowing margins, coverage areas and receiver sensitivity points all either match or are sufficiently close in value to what has been provided by 3GPP. Furthermore, in the instances where a small difference was observed the 3GPP was found to have utilized more conservative values.

Shadow fading margin (SFM) derivation methodology

For each of the deployment scenarios under consideration the cell area coverage for a single omnidirectional site has been considered to substantially reduce the complexity of the problem.

Starting with the following cell area coverage probability integral

Fu = 0 2πRPcovrpr,φdrdφ	 	(1)

where the probability of coverage at a distance r from the site with the pathloss Lr can be expressed as:

Pcov r=PLr+X>Lr-SFM r 	(2)

after substituting and resolving the integral, the cell coverage probability becomes:

### Fu=Qa+e21-abb21-Qa-2b      (3)

where:

### a=-SFMσ

### b=1σ10nlog10e

Q-function is the tail distribution function of the standard normal distribution.

In all eMBB and URLLC deployment scenarios, the cell coverage probability of 90% and 95% have been considered for data and control channels, respectively.

For the mMTC deployment scenarios, 99% cell area coverage was considered for both data and control channels.

Using the above cell coverage area coverage probability functional points along with the pathloss equations for Channel Model A and B, the SFM was derived as a function of the pathloss exponent and shadow fading margin.

Shadow Fading Standard Deviation considerations:

The eMBB and URLLC deployment scenarios were considered to be the most challenging cases, particularly the NLOS, NLOS-Outdoor-Indoor and NLOS In-Car scenarios, with  σcar = 5, and the outdoor σ having a different value.

Since there is only a single σ value that can be inserted into the calculation equation, scenarios with two independent standard deviations combined them using the following rule:

σ =a2+b2      (4)

For NLOS cases of eMBB and URLLC:

a = σoutdoor

b =  σcar

and for NLOS-O-I cases:

a = σoutdoor

b = σindoor

For Channel Model A, where an explicit σindoor value is not defined, the σindoor is derived and approximated using a generic uniform distribution of a variable into an interval (a, b), U (a, b), with the following characteristics:

The median u is defined as follows

u = (a + b)/2      (5)

while the standard deviation σ is derived as follows:

σindoor=  b-a2/12      (6)

The pathloss exponent it is determined by the applicable pathloss equations found in the M.2412 document along with the rest of the used shadow fading margin σ for each specific scenario.

The summary of the results for SFM values are presented in the following tables for each Channel Model. They all fall well within the values from 3GPP self-evaluation template.

|  | SFM eMBB - Channel Model A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | InH (4GHz) |  | DU (4GHz) |  |  |  | Rural(700MHz) |  |  |  |
| Results origin | 3GPP | CEG | 3GPP |  | CEG |  | 3GPP |  | CEG |  |
| Control Channel SFM (95%) | 2.80 | 2.84 | NLOS | NLOS O-I | NLOS | NLOS O-I | NLOS | NLOS O-I | NLOS | NLOS O-I |
|  |  |  | 8.07 | 6.95 | 8.12 | 6.97 | 10.45 | 8.45 | 10.01 | 8.24 |
| Data Channel SFM (90%) | O.91 | 0.94 | NLOS | NLOS O-I | NLOS | NLOS O-I | NLOS | NLOS O-I | NLOS | NLOS O-I |
|  |  |  | 4.85 | 4.03 | 4.89 | 4.04 | 6.61 | 5.13 | 6.24 | 4.86 |

|  | SFM eMBB - Channel Model B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | InH (4GHz) |  | DU (4GHz) |  |  |  | Rural(700MHz) |  |  |  |
| Results origin | 3GPP | CEG | 3GPP |  | CEG |  | 3GPP |  | CEG |  |
| Control Channel SFM (95%) | 8.50 | 8.49 | NLOS | NLOS O-I | NLOS | NLOS O-I | NLOS | NLOS O-I | NLOS | NLOS O-I |
|  |  |  | 8.07 | 9.04 | 8.12 | 9.59 | 10.45 | 10 | 10.01 | 9.66 |
| Data Channel SFM (90%) | 5.20 | 5.20 | NLOS | NLOS O-I | NLOS | NLOS O-I | NLOS | NLOS O-I | NLOS | NLOS O-I |
|  |  |  | 4.85 | 5.60 | 4.89 | 5.99 | 6.61 | 6.30 | 6.24 | 5.92 |

|  | SFM URLLC - Channel Model B |  |  |  |
| --- | --- | --- | --- | --- |
| Scenario | UMa (700MHz) |  |  |  |
| Results origin | 3GPP |  | CEG |  |
| Control Channel SFM (95%) | NLOS | NLOS O-I | NLOS | NLOS O-I |
|  | 8.11 | 8.30 | 8.12 | 7.59 |
| Data Channel SFM (90%) | NLOS | NLOS O-I | NLOS | NLOS O-I |
|  | 4.89 | 5.10 | 4.89 | 4.50 |

|  | SFM URLLC - Channel Model A |  |  |  |
| --- | --- | --- | --- | --- |
| Scenario | UMa (700MHz) |  |  |  |
| Results origin | 3GPP |  | CEG |  |
| Control Channel SFM (95%) | NLOS | NLOS O-I | NLOS | NLOS O-I |
|  | 8.11 | 7 | 8.12 | 7.28 |
| Data Channel SFM (90%) | NLOS | NLOS O-I | NLOS | NLOS O-I |
|  | 4.89 | 4.08 | 4.89 | 4.15 |

|  | SFM mMTC - Channel Model A |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | UMa NB-IoT (700MHz) |  |  |  |  |  | UMa eMTC (700MHz) |  |  |  |  |  |
| Results origin | 3GPP |  |  | CEG |  |  | 3GPP |  |  | CEG |  |  |
| Control Channel SFM (99%) | LOS | NLOS | NLOS O-I | LOS | NLOS | NLOS O-I | LOS | NLOS | NLOS O-I | LOS | NLOS | NLOS O-I |
|  | 6.30 | 10.26 | 12.22 | 6.24 | 10.26 | 12.32 | 6.30 | 10.26 | 12.22 | 6.24 | 10.26 | 12.32 |
| Data Channel SFM (99%) | LOS | NLOS | NLOS O-I | LOS | NLOS | NLOS O-I | LOS | NLOS | NLOS O-I | LOS | NLOS | NLOS O-I |
|  | 6.30 | 10.26 | 12.22 | 6.24 | 10.26 | 12.32 | 6.30 | 10.26 | 12.22 | 6.24 | 10.26 | 12.32 |

|  | SFM mMTC - Channel Model B |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | UMa NB-IoT (700MHz) |  |  |  |  |  | UMa eMTC (700MHz) |  |  |  |  |  |
| Results origin | 3GPP |  |  | CEG |  |  | 3GPP |  |  | CEG |  |  |
| Control Channel SFM (99%) | LOS | NLOS | NLOS O-I | LOS | NLOS | NLOS O-I | LOS | NLOS | NLOS O-I | LOS | NLOS | NLOS O-I |
|  | 6.3 | 10.26 | 17 | 6.24 | 10.26 | 16.18 | 6.3 | 10.26 | 17 | 6.24 | 10.26 | 16.18 |
| Data Channel SFM (99%) | LOS | NLOS | NLOS O-I | LOS | NLOS | NLOS O-I | LOS | NLOS | NLOS O-I | LOS | NLOS | NLOS O-I |
|  | 6.3 | 10.26 | 17 | 6.24 | 10.26 | 16.18 | 6.3 | 10.26 | 17 | 6.24 | 10.26 | 16.18 |

Penetration Margin derivation

The penetration margin calculations were performed using the instructions and information from M.2412 for both Channel Model A and B. On the other hand, the car penetration part utilized a conducted study on LTE mobiles mounted on various car models that verified the values agreed on for NLOS eMBB scenarios.

Also, for mMTC scenarios the high loss equations for building penetration loss were used due to the 99% cell area coverage requirement which is considered to be the most conservative case.

The tables below detail and compare the derived penetration loss values for all scenarios with 3GPP derived values. All derived values are within a 1dB range or less.

|  | Penetration Margin eMBB - Channel Model A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | InH (4GHz) |  | DU (4GHz) |  |  |  | Rural(700MHz) |  |  |  |
| Results origin | 3GPP | CEG | 3GPP |  | CEG |  | 3GPP |  | CEG |  |
| Penetration Margin | 0 | 0 | NLOS | NLOS O-I | NLOS | NLOS O-I | NLOS | NLOS O-I | NLOS | NLOS O-I |
|  |  |  | 9 | 26.25 | 9 | 26.25 | 9 | 12.5 | 9 | 12.5 |

|  | Penetration Margin eMBB - Channel Model B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | InH (4GHz) |  | DU (4GHz) |  |  |  | Rural(700MHz) |  |  |  |
| Results origin | 3GPP | CEG | 3GPP |  | CEG |  | 3GPP |  | CEG |  |
| Penetration Margin | 0 | 0 | NLOS | NLOS O-I | NLOS | NLOS O-I | NLOS | NLOS O-I | NLOS | NLOS O-I |
|  |  |  | 9 | 17.98 | 9 | 17.98 | 9 | 11.90 | 9 | 11.96 |

|  | Penetration Margin URLLC - Channel Model A |  |  |  |
| --- | --- | --- | --- | --- |
| Scenario | UMa (700MHz) |  |  |  |
| Results origin | 3GPP |  | CEG |  |
| Penetration Margin | NLOS | NLOS O-I | NLOS | NLOS O-I |
|  | 9 | 26.25 | 9 | 26.25 |

|  | Penetration Margin URLLC - Channel Model B |  |  |  |
| --- | --- | --- | --- | --- |
| Scenario | UMa (700MHz) |  |  |  |
| Results origin | 3GPP |  | CEG |  |
| Penetration Margin | NLOS | NLOS O-I | NLOS | NLOS O-I |
|  | 9 | 14.41 | 9 | 14.46 |

|  | Penetration Margin mMTC - Channel Model A |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | UMa NB-IoT (700MHz) |  |  |  |  |  | UMa eMTC (700MHz) |  |  |  |  |  |
| Results origin | 3GPP |  |  | CEG |  |  | 3GPP |  |  | CEG |  |  |
| Penetration Margin | LOS | NLOS | NLOS O-I | LOS | NLOS | NLOS O-I | LOS | NLOS | NLOS O-I | LOS | NLOS | NLOS O-I |
|  | 0 | 0 | 26.25 | 0 | 0 | 26.25 | 0 | 0 | 26.25 | 0 | 0 | 26.25 |

|  | Penetration Margin mMTC - Channel Model B |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | UMa NB-IoT (700MHz) |  |  |  |  |  | UMa eMTC (700MHz) |  |  |  |  |  |
| Results origin | 3GPP |  |  | CEG |  |  | 3GPP |  |  | CEG |  |  |
| Penetration Margin | LOS | NLOS | NLOS O-I | LOS | NLOS | NLOS O-I | LOS | NLOS | NLOS O-I | LOS | NLOS | NLOS O-I |
|  | 0 | 0 | 21.92 | 0 | 0 | 22.01 | 0 | 0 | 21.92 | 0 | 0 | 22.01 |

SNR verification

SNR verification was done using link level simulations. The methodology used was based on maintaining the same spectrum efficiency from 3GPP self-evaluation templates and computing the equivalent channel overhead for each specified bandwidth. The number of antennas and all other RF characteristics was maintained to provide a correct verification of the proposed results.

The simulations verified that all suggested SNR values in the 3GPP link budget templates were within 1-2 dB margin from the simulated values, which is below the receiver implementation loss of 2 dB. For this reason, and acknowledging the simulators implementation margins, it is concluded that the proposed SNR values are correct.

### =================================================================================