IMT-2020 Evaluation:

Link Budget analysis for UMa-eMTC and UMa-NB-IOT scenarios

Document for discussion and acceptance

CEG meeting

Montreal

June 12th, 2019

Source: Interdigital Inc., TELUS

# Abbreviations

3GPP: 3rd Generation Partnership Project

BLER: Block error rate

BS: Base-station

BW: Bandwidth

CCE: Control channel element

CP: Control-plane

CSI: Channel state information

CSI-RS: Channel state information reference signal

DU: Dense urban

DL: Downlink

DM-RS: Demodulation reference signal

eMBB: enhanced mobile broadband

gNB: g-NodeB

GoS: Grade-of-service

HARQ: Hybrid automatic repeat request

InH: Indoor Hotspot

ITU: International Telecommunication Union

LMLC: low-mobility large-cell

MIMO: multiple-input multiple-output

mMTC: massive Machine-Type Communications

NR: New Radio

NB-IoT: Narrowband-Internet of Things

OLLA: outer loop link adaptation

PDCCH: Physical downlink control channel, PDSCH: Physical downlink shared channel

PRB: Physical resource block

PT-RS: Phase tracking reference signal

PUCCH: Physical uplink control channel; PUSCH: Physical uplink shared channel

QoS: Quality-of-service

RAN: Radio access network

RE: Resource element

RIT: Radio-interface technology

RU: Rural

SCM: Stochastic channel model

SCS: Sub-carrier spacing

SDU: Service data unit

SR: Scheduling request

SRS: Sounding reference symbol

SSB: Synchronization signal block

TRxP: Transmission and reception point

TRS: Tracking reference signal

TTI: Transmission time interval

UE: User equipment

UL: Uplink

ULPC: Uplink power control

UP: User-plane

urLLC: ultra-reliable low-latency communications

1 Introduction

In this contribution we are presenting a link budget analysis of the IMT-2020 technology submitted by 3GPP for the Uma-eMTC, and Uma-NB-IOT deployments. This analysis is intended to evaluate and verify the link budget templates submitted by 3GPP to ITU-R WP5D in October meeting in [1].

All the setup parameters for the deployment scenarios under consideration are in the ranges suggested by ITU-R WP5D in M.2411 and M.2412 documents, and thus we will preserve them in our analysis to compare and validate data submitted by 3GPP. In terms of signal propagation, LOS, NLOS and NLOS O-I cases all apply for the eMTC link budget analysis.

In our spreadsheet analysis, the following parameters are kept unchanged:

- Number of antennas, transmission ports, antenna element gain, array gain, feeder loss, BS and UE output power.

- The cell area reliability as 99% for both data and control channels.

- The target packet error for eMBB scenarios data and control channels as 10% and 1% respectively and the HARQ gain is 0 as the gain here is coming from packet repetitions (repetition coding).

- The Noise Figure for UE and BS are kept the same. However, we believe that the values used by 3GPP are conservative. We believe that the UE and BS will perform better that this minimum requirement.

- Receiver interference density for both data and control channels remain the same as presented in the 3GPP link budget template (they are the same as LTE Advanced cases from the past). Since we are facing similar deployments for eMBB cases under consideration, we will simply assume the same values.

- The receiver implementation margin.

- Suggested spectrum efficiency for these cases will be the same.

Our analysis focuses on verification of shadow fading margins (SFM) and penetration margins calculations.

The SNR validation for the control and data channels is provided in [2].

### 2 Discussion

### 2.1 Shadow Fading Margin (SFM) derivation

### Our analysis is based on a single cell assumption which simplifies the approach considerably. The goal is to find a mathematical expression of the Cell Area Coverage probability Fu as a function of SFM, the pathloss exponent “n”, and the shadow fading standard deviation “σ” that define each of the deployments considered in this paper:

### - UMa-NB-IOT-LOS

### - UMa-NB-IOT- NLOS

### - UMa-NB-IOT-NLOS O-I

### - UMa-eMTC-LOS

### - UMa- eMTC - NLOS

### - UMa- eMTC -NLOS O-I

First, we will show how the Cell Area Coverage probability function is derived.

### We can write the cell area coverage as the well know integral:

### Pcov(r) is the coverage probability at distance “r” and can be expressed as follows:

### , where L is the pathloss as a function of “r” and can be expressed as follows:

### We obtain the following form of the equation where Fu is a function of SFM, n and σ.

### Where the Q-function is the tail distribution function of the standard normal distribution.

### To resolve the integral equation for Fu, the following substitutions were used:

### That leads to the following form of the equation:

### The above equation can be solved by using integration by parts. In the end, we obtain the cell area coverage probability equation as a function of SFM, standard deviation σ, and pathloss exponent n:

### Fu=Qa+e2(1-ab)b21-Qa-2b

### where:

### a=-SFMσ

### b=1σ10nlog10e

### With these equations, we are ready to plot and derive the SFM for the deployment scenarios considered in this paper.

### 2.1.1 UMa MTC LOS Channel Model A and B

### In this case for Channel Model A and B we have σ = 4 and the pathloss exponent n = 4 based on the UMa pathloss equations.

### These inputs will yield the following curve:

### Thus, we have SFM =   6.24 for 99% cell reliability for both control and data channels respectively.

### This value is very closed to 6.3, the value proposed by 3GPP.

### 2.1.2 UMa MTC NLOS Channel Model A and B

### In this case for Channel Model A we have σ = 6 and the pathloss exponent n = 3.908 based on the UMa pathloss equations.

### For Channel Model A (for f < 6GHz), the pathloss exponent is computed as follows:

### n= (43.42 – 3.1log10(hBS))/10,

### After substituting hBS=25 m., we obtain:

### n= (43.42 -3.1log10(hBS))/10 = (43.42- 4.33)/10

### Therefore,  n =3.908.

### Similarly, for Channel Model B, we obtain n=3.908 (directly from the pathloss equation).

### These inputs will yield the following curve:

### Thus, we have SFM =   10.26 for 99% cell reliability for both control and data channels respectively.

### This value matches perfectly the 3GPP proposed one.

### 2.1.4 UMa MTC NLOS O-I Channel Model A

### In this case for both Channel Model A and B we the pathloss exponent n = 3.908 based on the UMa pathloss equations and the explanation given for NLOS case.

### Then we compute the global standard deviation as:

### With median u = (a + b)/2 and σ = SQRT (((b - a)^2) / 12)

### So, in our case a=0, b=12.5dB. Thus u=6.25, σ = SQRT (12.5^2/12) = SQRT (13.02) = 3.61

### Thus, we can consider having an equivalent indoor standard deviation σ_indoor = 3.61

### σ = SQRT (σ_outdoor^2 + σ_indoor^2) = SQRT (36+13.03) = 7.00 (σ_indoor = 3.61)

### These inputs will yield the following curve:

### Thus, we have SFM = 12.32 for 99% cell reliability for both control and data channels respectively.

### This value is very closed to 12.22, the value proposed by 3GPP.

### 2.1.5 UMa MTC NLOS O-I Channel Model B

### In this case for Channel Model B we the pathloss exponent n = 3.908 based on the UMa pathloss equations.

### Then we compute the global standard deviation as:

### σ = SQRT (σ_outdoor^2 + σ_indoorr^2) = SQRT (36+42.25) = 8,84 (σ_indoor = 6.5 High Loss case)

### These inputs will yield the following curve:

### Thus, we have SFM = 16.18 for 99% cell reliability for both control and data channels respectively.

### This value is very closed to 17, the value proposed by 3GPP.

### 2.2 Penetration Margin

### Penetration margin is considered in link budget analysis for the scenarios where there is a loss to be considered from an outdoor to indoor propagation loss. Thus, only NLOS O-I and scenarios for Channel Model A and B are considered here.

### 2.2.2 MTC NLOS O-I penetration margin

### The Penetration Margin is given by the following equation:

### Penetration Margin = PLtw + PLin + N (0, σP2)

### For Channel Model A:

### For Channel Model A, indoor UEs, in ITU-R M.2412 document the following a simple approach is suggested:

### Frequency range: FR1

### Path loss through external wall: PLtw [dB] = 20

### Indoor loss: PLin [dB] = 0.5d2D-in

### Standard deviation: σP [dB] = 0

### The UEs are uniformly distributed over 25m indoor. With this said, the penetration margin for the very worst case is:

### Penetration Margin (pessimistic case) = PLtw + PLin = 20 + 0.5*25 = 20 + 12.5 = 32.5dB

### The uniform distribution of a variable into an interval (a, b) can be modeled as X ~U (a, b) which has a median value of u = (a+b)/2.

### Thus, the uniform distribution that deals with PLin max = 12.5dB maximum penetration loss for indoor UEs, says that the mean value is

### u = PLin max /2 = 12.5dB/2 = 6.25dB

### In the end, to have a more reasonable approach for the penetration loss without taking the most pessimistic case, as the UE are uniformly distributed.

### We can use the median value and have a similar value as 3GPP link budget suggested for this case:

### Penetration Margin for Channel Model A MTC NLOS O-I as PLtw + u = 26.25dB

### This value matches the value proposed by 3GPP.

### For Channel Model B:

### For outdoor to indoor penetration loss, in ITU-R M.2412 document the following equation is suggested:

### Penetration Margin = PLtw + PLin + N (0, σP2)

### Frequency range: FR1

### Path loss through external wall low loss:

### PLtw [dB] =  5-10log100.3⋅10-Lglass10+0.7⋅10-Lconcrete10

### Indoor loss: PLin [dB] = 0.5d2D-in

### Standard deviation low loss: σP [dB] = 4.4

### Standard deviation high loss: σP [dB] = 6.5

### The UEs are uniformly distributed over 25m indoor. With this said, the penetration margin for the very worst case is:

### For f = 0.7GHz

### Lconcrete=5+4⋅f = 7.8dB

### Lglass=2+0.2⋅f = 2.14dB

### LIRRglass=23+0.3⋅f = 23+0.21= 23.21dB

### Low Loss:

### PLtw [dB] = 5 – 10*log (0.3*10^(-0.214) + 0.7*10^(-0.78)) = 5 – 10* log (0.3*0.61 + 0.7*0.16) =

### = 5- 10*log (0.183 +0.116) = 5 – (10*(-0.524) = 5 + 5.24 = 10.24dB

### Thus:

### Low Loss PLtw [dB] = 10.24dB

### High Loss:

### PLtw [dB] =  5-10log100.7⋅10-LIRRglass10+0.3⋅10-Lconcrete10 = 5 – 10*log (0.7*10^(-2.321) + 0.3*10^(-0.78)) = 5 – 10* log (0.7*0.004 + 0.7*0.165)=

### = 5- 10*log (0.0028 +0.049) = 5 – (10*(-1.28) = 17.85dB

### Thus

### High Loss PLtw [dB] = 17.85dB

### Observation: Since the cell reliability for mMTC is supposed to be 99%, the appropriate penetration loss to consider is the High loss case for the link budget evaluation.

### Thus, in the final computation we will consider PLtw [dB] = 17.85dB.

### PLin [dB] = 0.5d2D-in

### Worst case is 25m:  PLin [dB] = 12.5dB

### However, for the Channel Model B we have a uniform distribution driven by 2 independent random variables as specified by M.2412.

### This will lead to an expression of the probability to be in the (0, M) depth as:

### pd2D-in=2M2(M-x)2  , if 0≤x≤M.

### So, for the probability density we need to integrate the above function and we will have the following antiderivative:

### 23M2(M-x)3

### If we take M/2 as the median depth, and using the antiderivative between [0 and M/2] depth d2D-in, we find:

### M/3

### Thus, the indoor pathloss would be

### PLin [dB] = 0.5d2D-in/3 = 12.5/3 = 4.16dB

### Thus, the penetration loss will be:

### Penetration Margin = 17.85 + 4.16 = 22.01dB

### Penetration Margin for Channel Model B Uma MTC NLOS O-I is PLtw + u = 22.01dB.

### This value is very close to the 21.92dB value proposed by 3GPP.

### 2.3 SNR analysis

### The SNR values provided in [2] were derived for each of the scenarios under study at 1% (Control Channel) and 10% (Data Channel) for 3GPP link budget templates. They are quite clsoe if not identical to the values in the 3GPP link budget template and spreadsheet. Thus, we are conclude that the 3GPP suggested SNR values for eMTC and NB-IOT are correct.

### Conclusion

### We are attaching the spreadsheets for Channel Model A and B with our link budget calculation results for reference.

### Observation 1: The Shadow Fading Margin (SFM) values computed in this paper are very close, if not identical to those suggested by 3GPP submission and thus, we consider that the SFM self-evaluation correct.

### Observation 2: The penetration margins computed in this paper using our assumptions as explained in the second section of this document prove to be identical to those suggested by 3GPP and thus, we conclude that they are correct.

### Observation 3: The SNR presented in [2] prove that the self-evaluation proposed values are correct and achievable. Thus, we conclude that they are correct.

### We can conclude that 3GPP submission for IMT-2020 link budget for UMa eMTC and Uma NB-IOT scenarios meet the pathloss availability, cell radius, and the cell area coverage requirements.

### References

- R15-WP5D-C-1050 WP-5D meeting, Fukuoka, Japan

- RP-181889, IMT-2020 self-evaluation: mMTC link budget, Ericsson, Sierra Wireless, Gold Coast, Australia, TSG- RAN Meeting #81, September 10-13, 2018