IMT-2020 Evaluation:

Peak Data Rate and Peak Spectral Efficiency Evaluations for NR

Document for discussion and acceptance

CEG meeting

Sep xxth, 2019

Source: BELL MOBILITY, TELUS

# Peak Spectral Efficiency and Peak Data Rate

As specified in Report ITU-R M.2410-0 [1], the minimum requirements for peak spectral efficiencies for the purpose of evaluation in the eMBB usage scenario are defined as below:

|  | ### Antenna Configuration (No. of Spatial Layers) | ### Peak Spectral Efficiency (bit/s/Hz) | ### Peak Data Rate (Gbps) |
| --- | --- | --- | --- |
| ### Downlink | ### 8 | ### 30 | ### 20 |
| ### Uplink | ### 4 | ### 15 | ### 10 |

The following outlines the results of the detailed analyses of Peak Spectral Efficiency for both NR and LTE

## NR peak spectral efficiency

For the NR, the generic peak spectral efficiency per carrier component can be calculated using the formula given below:

Where Rmax is the maximum coding rate, and

For the j-th CC:

- νLayers(j) is the maximum number of MIMO layers

- Qm(j)is the highest modulation order

- f(j) is the scaling factor

- μ is the numerology (as defined in TS 38.211)

- Tsμ is the average OFDM symbol duration in a subframe for numerology μ, that is, Tsμ=10-314×2μ. (Normal cyclic prefix is assumed)

- NPRBBWj, μ is the maximum RB allocation in bandwidth

- BWj with numerology μ  is the UE supported maximum bandwidth in the given band or band combination.

- OH(j) is the overhead and takes the following values (3GPP 38.306) [2]:

- 0.14, for frequency range FR1 for DL

- 0.18, for frequency range FR2 for DL

- 0.08, for frequency range FR1 for UL

- 0.10, for frequency range FR2 for UL

For NR FDD frequency range of FR1 (410 MHz – 7125 MHz) DL peak spectral efficiencies for different channel bandwidths are calculated and presented in Table 2. The detailed assumptions used in the evaluation are provided in Table 1:

Table 1 Assumptions for peak spectral efficiency and peak data rate

| ### Parameters | ### Downlink | ### Uplink |
| --- | --- | --- |
| ### νLayers(j) | ### 8 | ### 4 |
| ### Qm(j) | ### 8 | ### 8 |
| ### f(j) | ### 1 | ### 1 |
| ### Rmax | ### 948/1024 | ### 948/1024 |
| ### μ | ### FR1: 0,1,2 ### FR2: 2,3 | ### FR1: 0,1,2 ### FR2: 2,3 |
| ### NPRBBWj, μ | ### We have used maximum number of RBs for the specific component carrier bandwidth and SCS | ### We have used maximum number of RBs for the specific component carrier bandwidth and SCS |

Table 2 NR FDD frequency range of FR1 (410 MHz – 7125 MHz) DL peak spectral efficiency (bit/s/Hz)

8-layer downlink transmission, with 256QAM modulation, and a maximum coding rate of Rmax=0.9258

| SCS (KHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 42.80 | 44.51 | 45.08 | 45.37 | 45.54 | 45.65 | 46.23 | 46.23 | n/a | n/a | n/a | n/a |
| 30 | 37.66 | 41.09 | 43.37 | 43.66 | 44.51 | 44.51 | 45.37 | 45.54 | 46.23 | 46.44 | 46.61 | 46.74 |
| 60 | n/a | 37.66 | 41.09 | 41.09 | 42.46 | 43.37 | 43.66 | 44.51 | 45.08 | 45.80 | 46.04 | 46.23 |

For NR FDD frequency range of FR1 (410 MHz – 7125 MHz) UL peak spectral efficiency is calculated and presented in Table 3. The detailed assumptions are presented in Table 1.

Table 3 NR FDD frequency range of FR1 (410 MHz – 7125 MHz) UL peak spectral efficiency (bit/s/Hz)

4-layer uplink transmission, with 256QAM modulation, and a maximum coding rate of Rmax=0.9258

| SCS (KHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 22.89 | 23.81 | 24.11 | 24.27 | 24.36 | 24.42 | 24.72 | 24.72 | n/a | n/a | n/a | n/a |
| 30 | 20.14 | 21.98 | 23.20 | 23.35 | 23.81 | 23.81 | 24.27 | 24.36 | 24.72 | 24.84 | 24.93 | 25.00 |
| 60 | n/a | 20.15 | 21.98 | 21.98 | 22.71 | 23.20 | 23.35 | 23.81 | 24.11 | 24.50 | 24.62 | 24.72 |

Similarly, for NR TDD frequency range of FR1 (410 MHz – 7125 MHz) DL peak spectral efficiency is calculated and presented in Table 4. In TDD, effective bandwidth, which is the operating bandwidth normalized considering the uplink/downlink ratio, needs to be considered. The DL/UL ratio of 4:1 is evaluated.

Table 4 NR TDD frequency range of FR1 (410 MHz – 7125 MHz) DL peak spectral efficiency (bit/s/Hz)

8-layer downlink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258, DL/UL 4:1

| SCS (KHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 42.80 | 44.51 | 45.08 | 45.37 | 45.54 | 45.65 | 46.23 | 46.23 | n/a | n/a | n/a | n/a |
| 30 | 37.66 | 41.09 | 43.37 | 43.66 | 44.51 | 44.51 | 45.37 | 45.54 | 46.23 | 46.44 | 46.61 | 46.74 |
| 60 | n/a | 37.66 | 41.09 | 41.09 | 42.46 | 43.37 | 43.66 | 44.51 | 45.08 | 45.80 | 46.04 | 46.23 |

Further, for NR TDD frequency range of FR1 (410 MHz – 7125 MHz) UL peak spectral efficiency is calculated and presented in Table 5. The DL/UL ratio of 4:1 is evaluated.

Table 5 NR TDD frequency range of FR1 (410 MHz – 7125 MHz) UL peak spectral efficiency (bit/s/Hz)

4-layer uplink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258, DL/UL 4:1

| SCS (KHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 22.89 | 23.81 | 24.11 | 24.27 | 24.36 | 24.42 | 24.72 | 24.72 | n/a | n/a | n/a | n/a |
| 30 | 20.14 | 21.98 | 23.20 | 23.35 | 23.81 | 23.81 | 24.27 | 24.36 | 24.72 | 24.84 | 24.93 | 25.00 |
| 60 | n/a | 20.15 | 21.98 | 21.98 | 22.71 | 23.20 | 23.35 | 23.81 | 24.11 | 24.50 | 24.62 | 24.72 |

For NR TDD frequency range of FR2 (24250 MHz – 52600 MHz), UL and DL peak spectral efficiencies are calculated and presented in Table 6 and 7, respectively. In TDD mode, effective bandwidth, which is the operating bandwidth normalized by the uplink/downlink duty cycle ratio, needs to be considered. Again, the detailed assumptions are presented in Table 1. The DL/UL ratio of 4:1 is evaluated.

Table 6 NR TDD frequency range of FR2 (24250 MHz – 52600 MHz) DL peak spectral efficiency (bit/s/Hz)

8-layer downlink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258, DL/UL 4:1

| SCS (KHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 43.20 | 45.20 | 45.20 | n/a |
| 120 | 43.83 | 45.20 | 45.20 | 45.20 |

Table 7 NR TDD frequency range of FR2 (24250 MHz – 52600 MHz) UL peak spectral efficiency (bit/s/Hz)

4-layer uplink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258, DL/UL 4:1

| SCS (KHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 23.65 | 23.65 | 23.65 | n/a |
| 120 | 22.93 | 23.65 | 23.65 | 23.65 |

Similar tables for the FR2 range can be calculated assuming a lower modulation order of 64 QAM.

Table 8 NR TDD frequency range of FR2 (24250 MHz – 52600 MHz) DL peak spectral efficiency (bit/s/Hz)

8-layer downlink transmission, with 64QAM modulation, and a maximum coding rate of 0.9258, DL/UL 4:1

| SCS (KHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 32.4 | 33.9 | 33.9 | n/a |
| 120 | 32.9 | 33.9 | 33.9 | 33.9 |

Table 9 NR TDD frequency range of FR2 (24250 MHz – 52600 MHz) UL peak spectral efficiency (bit/s/Hz)

4-layer uplink transmission, with 64QAM modulation, and a maximum coding rate of 0.9258, DL/UL 4:1

| SCS (KHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 17.74 | 17.74 | 17.74 | n/a |
| 120 | 17.20 | 17.74 | 17.74 | 17.74 |

Evidently, all the values in the tables 2 to 9 meet the peak spectral efficiency requirements outlined in ITU-R M.2410-0 evaluation criteria. For the 256QAM modulation order, spectral efficiency values exceed the target for instance 30 b/s/Hz in the DL by a wide margin, thus leaving a significant margin for larger overheads than the values used in the analysis herein (e.g., OH=0.14 in the DL). Even in 64QAM modulation order scenario, which applies to FR2 range only, there is still some room left for a higher overhead both in the DL and in the UL, but in general, spectral efficiency calculated in tables 8 and 9 meets the ITU-R requirements.

## Peak data rate calculation for the NR RIT

In what follows, the peak data rate is calculated following the methodology specified in § 4.1 of Report ITU-R M.2410-0 [1], using peak spectral efficiency and maximum assignable channel bandwidth.

Table 10 NR FDD frequency range of FR1 (410 MHz – 7125 MHz) DL peak data rate (Gbps)

8-layer downlink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258

| SCS (KHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 0.214 | 0.445 | 0.676 | 0.907 | 1.14 | 1.37 | 1.85 | 2.31 | n/a | n/a | n/a | n/a |
| 30 | 0.188 | 0.411 | 0.650 | 0.873 | 1.11 | 1.33 | 1.81 | 2.28 | 2.77 | 3.71 | 4.19 | 4.67 |
| 60 | n/a | 0.377 | 0.616 | 0.822 | 1.06 | 1.30 | 1.75 | 2.22 | 2.70 | 3.66 | 4.14 | 4.62 |

Table 11 NR FDD frequency range of FR1 (410 MHz – 7125 MHz) UL peak data rate (Gbps)

4-layer uplink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258

| SCS (KHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 0.114 | 0.238 | 0.362 | 0.485 | 0.609 | 0.733 | 0.989 | 1.23 | n/a | n/a | n/a | n/a |
| 30 | 0.100 | 0.220 | 0.348 | 0.467 | 0.595 | 0.714 | 0.970 | 1.22 | 1.48 | 1.99 | 2.24 | 2.5 |
| 60 | n/a | 0.201 | 0.330 | 0.439 | 0.568 | 0.696 | 0.934 | 1.19 | 1.45 | 1.96 | 1.22 | 2.47 |

Table 12 NR TDD frequency range of FR1 (410 MHz – 7125 MHz) DL peak data rate (Gbps)

8-layer downlink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258, DL/UL 4:1

| SCS (KHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 0.171 | 0.356 | 0.541 | 0.726 | 0.911 | 1.09 | 1.48 | 1.85 | n/a | n/a | n/a | n/a |
| 30 | 0.151 | 0.329 | 0.520 | 0.698 | 0.890 | 1.07 | 1.45 | 1.82 | 2.22 | 2.97 | 3.35 | 3.74 |
| 60 | n/a | 0.301 | 0.493 | 0.657 | 0.849 | 1.04 | 1.40 | 1.78 | 2.16 | 2.93 | 3.31 | 3.70 |

Table 13 NR TDD frequency range of FR1 (410 MHz – 7125 MHz) UL peak data rate (Gbps)

4-layer uplink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258, DL/UL 4:1

| SCS (KHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 0.023 | 0.048 | 0.072 | 0.097 | 0.122 | 0.146 | 0.198 | 0.247 | n/a | n/a | n/a | n/a |
| 30 | 0.020 | 0.044 | 0.069 | 0.093 | 0.119 | 0.143 | 0.194 | 0.243 | 0.297 | 0.397 | 0.449 | 0.500 |
| 60 | n/a | 0.040 | 0.066 | 0.088 | 0.113 | 0.139 | 0.187 | 0.238 | 0.289 | 0.392 | 0.443 | 0.494 |

Table 14 NR TDD frequency range of FR2 (24250 MHz – 52600 MHz) DL peak data rate (Gbps)

8-layer downlink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258, DL/UL 4:1

| SCS (KHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 1.81 | 3.61 | 7.23 | n/a |
| 120 | 1.75 | 3.61 | 7.23 | 14.46 |

If we assume %100 transmission in the downlink instead of %80 evaluation shown in Table 6, we get:

Table 15 NR TDD frequency range of FR2 (24250 MHz – 52600 MHz) DL peak data rate (Gbps)

8-layer downlink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258, DL only

| SCS (KHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 2.26 | 4.52 | 9.04 | n/a |
| 120 | 2.19 | 4.52 | 9.04 | 18.08 |

Considering the possibility of aggregating minimum two FR2 carriers with 400 BW each (800 MHz in total), we can get a peak data rate of almost 36 Gbps. This would fulfil the minimum requirement of 20 Gbps.

Table 16 NR TDD frequency range of FR2 (24250 MHz – 52600 MHz) UL peak data rate (Gbps)

4-layer uplink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258, DL/UL 4:1

| SCS (KHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 0.236 | 0.473 | 0.946 | n/a |
| 120 | 0.229 | 0.473 | 0.946 | 1.89 |

If we assume %100 transmission in the uplink instead of %20 evaluation shown in Table 8, we get:

Table 17 NR TDD frequency range of FR2 (24250 MHz – 52600 MHz) UL peak data rate (Gbps)

4-layer uplink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258, UL only

| SCS (KHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 1.18 | 2.36 | 4.73 | n/a |
| 120 | 1.15 | 2.36 | 4.73 | 9.46 |

Following the same methodology, but assuming 64QAM modulation order, the peak speeds in FR2 frequency range are calculated and the values are shown below.

Table 18 NR TDD frequency range of FR2 (24250 MHz – 52600 MHz) DL peak data rate (Gbps)

8-layer downlink transmission, with 6QAM modulation, and a maximum coding rate of 0.9258, DL/UL 4:1

| SCS (KHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 1.36 | 2.71 | 5.42 | n/a |
| 120 | 1.31 | 2.71 | 5.42 | 10.85 |

If we assume %100 transmission in the downlink instead of %80 evaluation shown in Table 6, we get:

Table 19 NR TDD frequency range of FR2 (24250 MHz – 52600 MHz) DL peak data rate (Gbps)

8-layer downlink transmission, with 64QAM modulation, and a maximum coding rate of 0.9258, DL only

| SCS (KHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 1.70 | 3.39 | 6.78 | n/a |
| 120 | 1.64 | 3.39 | 6.78 | 13.56 |

Considering the possibility of aggregating at least two FR2 carriers with 400 BW each (800 MHz in total), we can get a peak data rate of almost 27 Gbps. This would fulfil the minimum requirement of 20 Gbps even if 64QAM is used.

Table 20 NR TDD frequency range of FR2 (24250 MHz – 52600 MHz) UL peak data rate (Gbps)

4-layer uplink transmission, with 64QAM modulation, and a maximum coding rate of 0.9258, DL/UL 4:1

| SCS (KHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 0.18 | 0.35 | 0.71 | n/a |
| 120 | 0.17 | 0.35 | 0.71 | 1.42 |

If we assume %100 transmission in the uplink instead of %20 evaluation shown in Table 8, we get:

Table 21 NR TDD frequency range of FR2 (24250 MHz – 52600 MHz) UL peak data rate (Gbps)

4-layer uplink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258, UL only

| SCS (KHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 0.89 | 1.77 | 3.55 | n/a |
| 120 | 0.86 | 1.77 | 3.55 | 7.10 |

## LTE peak data rates and spectral efficiencies

In order to evaluate the LTE peak spectral efficiencies for different bandwidths, we first calculate the peak speeds for the channel bandwidths listed in Table 22:

Table 22 LTE channel bandwidths and associated number of resource blocks

| ### Channel bandwidths | 5 ### MHz | 10 ### MHz | 15 ### MHz | 20 ### MHz |
| --- | --- | --- | --- | --- |
| ### NRB | 25 | 50 | 75 | 100 |

To determine maximally supported peak LTE data rates assuming 8 spatial layers of transmission, we have relied on transport block sizes specified in TS 36.213, section 7.1.7.

Assuming 1024QAM in the downlink(Qm=10), and using the number of resource blocks listed in Table 22 (above), maximal transport block sizes for a single spatial (MIMO) layer from TS 36.213 are listed below:

Table 23 LTE transport block sizes for given channel bandwidths

| ### Channel bandwidths | 5 ### MHz | 10 ### MHz | 15 ### MHz | 20 ### MHz |
| --- | --- | --- | --- | --- |
| ### NRB | 25 | 50 | 75 | 100 |
| ### TBS | 30576 | 61664 | 93800 | 125808 |

After mapping from one to four layers and accounting for 2 codewords (for 8 layer transmission), the peak data rates and spectral efficiency for the DL components are:

Table 24 LTE peak data rates and spectral efficiencies in the DL for 1024QAM

| ### Channel bandwidths | 5 ### MHz | 10 ### MHz | 15 ### MHz | 20 ### MHz |
| --- | --- | --- | --- | --- |
| ### Rpeak (Gbps) | 0.25 | 0.49 | 0.75 | 1.00 |
| ### SE | 49.78 | 49.12 | 50.05 | 50.26 |

Given that LTE allows up to 32 component carriers in carrier aggregation, the peak DL data arte can be as high as 32 Gbps assuming 20 MHz channels. At the same time, the peak spectral efficiency values of around 50 b/s/Hz leave more than sufficient room for any overhead variation that would account for a mix of MBSFN and non-MBSFN subframes for example.

Similarly, for the UL, using the transport block size index of ITBS=34, a 20 MHz channel can have transport block size of 105528 bits per transmit time interval, which is mapped onto 422232 bits for a 4 layer UL transmission. This corresponds to about 420 Mb/s of peak data rates for a single 20 MHz channel, or to SEUL = 21 b/s/Hz.

# Conclusion

Observation 1: The Peak Spectral Efficiency values computed in this paper, for both NR and LTE, fulfill the ITU-R targets for these technical performance requirements.

Observation 2: The Peak Data Rate values computed in this paper, for both NR and LTE, fulfill the ITU-R targets for these technical performance requirements.

# References

- ITU-R M.2410-0, Minimum requirements related to technical performance for IMT-2020 radio interface(s)

- 3GPP 38.306, NR; User Equipment (UE) radio access capabilities