IMT-2020 Evaluation:

Peak Data Rate and Peak Spectral Efficiency Evaluations for NR

Document for discussion and acceptance

CEG meeting

August 08th, 2019

Source: BELL MOBILITY, TELUS

### Peak Spectral Efficiency and Peak Data Rate

### As per Report ITU-R M.2410-0 [1], the minimum requirements for peak spectral efficiencies for the purpose of evaluation in the eMBB usage scenario are defined as below:

|  | ### Antenna Configuration (No. of Spatial Layers) | ### Peak Spectral Efficiency (bit/s/Hz) | ### Peak Data Rate (Gbps) |
| --- | --- | --- | --- |
| ### Downlink | ### 8 | ### 30 | ### 20 |
| ### Uplink | ### 4 | ### 15 | ### 10 |

### The following outlines the results of the detailed analyses of Peak Spectral Efficiency. The generic peak spectral efficiency per carrier component can be calculated using below formula;

### Where Rmax is the maximum coding rate, and

### For the j-th CC:

- νLayers(j) is the maximum number of MIMO layers

- Qm(j)is the highest modulation order

- f(j) is the scaling factor

- μ is the numerology (as defined in TS 38.211)

- Tsμ is the average OFDM symbol duration in a subframe for numerology μ, that is, Tsμ=10-314×2μ. (Normal cyclic prefix is assumed)

- NPRBBWj, μ is the maximum RB allocation in bandwidth

- BWj with numerology μ  is the UE supported maximum bandwidth in the given band or band combination.

- OH(j) is the overhead and takes the following values (3GPP 38.306) [2]:

  - 0.14, for frequency range FR1 for DL

  - 0.18, for frequency range FR2 for DL

  - 0.08, for frequency range FR1 for UL

  - 0.10, for frequency range FR2 for UL

### For NR FDD frequency range of FR1 (410 MHz – 7125 MHz) DL peak spectral efficiency is calculated and presented in Table 2. The detailed assumptions used in the evaluation are as follows:

### Table 1 Assumptions for peak spectral efficiency and peak data rate

| ### Parameters | ### Downlink | ### Uplink |
| --- | --- | --- |
| ### νLayers(j) | ### 8 | ### 4 |
| ### Qm(j) | ### 8 | ### 8 |
| ### f(j) | ### 1 | ### 1 |
| ### Rmax | ### 948/1024 | ### 948/1024 |
| ### μ | ### FR1: 0,1,2 ### FR2: 2,3 | ### FR1: 0,1,2 ### FR2: 2,3 |
| ### NPRBBWj, μ | ### We have used maximum number of RBs for the specific component carrier bandwidth and SCS | ### We have used maximum number of RBs for the specific component carrier bandwidth and SCS |

### Table 2 NR FDD frequency range of FR1 (410 MHz – 7125 MHz) DL peak spectral efficiency (bit/s/Hz)

### 8-layer downlink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258

| ### SCS (KHz) | 5 ### MHz | 10 ### MHz | 15 ### MHz | 20 ### MHz | 25 ### MHz | 30 ### MHz | 40 ### MHz | 50 ### MHz | 60 ### MHz | 80 ### MHz | 90 ### MHz | 100 ### MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ### 15 | ### 42.80 | ### 44.51 | ### 45.08 | ### 45.37 | ### 45.54 | ### 45.65 | ### 46.23 | ### 46.23 | ### n/a | ### n/a | ### n/a | ### n/a |
| ### 30 | ### 37.66 | ### 41.09 | ### 43.37 | ### 43.66 | ### 44.51 | ### 44.51 | ### 45.37 | ### 45.54 | ### 46.23 | ### 46.44 | ### 46.61 | ### 46.74 |
| ### 60 | ### n/a | ### 37.66 | ### 41.09 | ### 41.09 | ### 42.46 | ### 43.37 | ### 43.66 | ### 44.51 | ### 45.08 | ### 45.80 | ### 46.04 | ### 46.23 |

### For NR FDD frequency range of FR1 (410 MHz – 7125 MHz) UL peak spectral efficiency is calculated and presented in Table 3. The detailed assumptions are presented in Table 1.

### Table 3 NR FDD frequency range of FR1 (410 MHz – 7125 MHz) UL peak spectral efficiency (bit/s/Hz)

### 4-layer uplink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258

| ### SCS (KHz) | 5 ### MHz | 10 ### MHz | 15 ### MHz | 20 ### MHz | 25 ### MHz | 30 ### MHz | 40 ### MHz | 50 ### MHz | 60 ### MHz | 80 ### MHz | 90 ### MHz | 100 ### MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ### 15 | ### 22.89 | ### 23.81 | ### 24.11 | ### 24.27 | ### 24.36 | ### 24.42 | ### 24.72 | ### 24.72 | ### n/a | ### n/a | ### n/a | ### n/a |
| ### 30 | ### 20.14 | ### 21.98 | ### 23.20 | ### 23.35 | ### 23.81 | ### 23.81 | ### 24.27 | ### 24.36 | ### 24.72 | ### 24.84 | ### 24.93 | ### 25.00 |
| ### 60 | ### n/a | ### 20.15 | ### 21.98 | ### 21.98 | ### 22.71 | ### 23.20 | ### 23.35 | ### 23.81 | ### 24.11 | ### 24.50 | ### 24.62 | ### 24.72 |

### For NR TDD frequency range of FR1 (410 MHz – 7125 MHz) DL peak spectral efficiency is calculated and presented in Table 4. In TDD, effective bandwidth, which is the operating bandwidth normalized considering the uplink/downlink ratio, needs to be considered. The DL/UL ratio of 4:1 is evaluated.

### Table 4 NR TDD frequency range of FR1 (410 MHz – 7125 MHz) DL peak spectral efficiency (bit/s/Hz)

### 8-layer downlink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258, DL/UL 4:1

| ### SCS (KHz) | 5 ### MHz | 10 ### MHz | 15 ### MHz | 20 ### MHz | 25 ### MHz | 30 ### MHz | 40 ### MHz | 50 ### MHz | 60 ### MHz | 80 ### MHz | 90 ### MHz | 100 ### MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ### 15 | ### 42.80 | ### 44.51 | ### 45.08 | ### 45.37 | ### 45.54 | ### 45.65 | ### 46.23 | ### 46.23 | ### n/a | ### n/a | ### n/a | ### n/a |
| ### 30 | ### 37.66 | ### 41.09 | ### 43.37 | ### 43.66 | ### 44.51 | ### 44.51 | ### 45.37 | ### 45.54 | ### 46.23 | ### 46.44 | ### 46.61 | ### 46.74 |
| ### 60 | ### n/a | ### 37.66 | ### 41.09 | ### 41.09 | ### 42.46 | ### 43.37 | ### 43.66 | ### 44.51 | ### 45.08 | ### 45.80 | ### 46.04 | ### 46.23 |

### For NR TDD frequency range of FR1 (410 MHz – 7125 MHz) UL peak spectral efficiency is calculated and presented in Table 5. The DL/UL ratio of 4:1 is evaluated.

### Table 5 NR TDD frequency range of FR1 (410 MHz – 7125 MHz) UL peak spectral efficiency (bit/s/Hz)

### 4-layer uplink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258, DL/UL 4:1

| ### SCS (KHz) | 5 ### MHz | 10 ### MHz | 15 ### MHz | 20 ### MHz | 25 ### MHz | 30 ### MHz | 40 ### MHz | 50 ### MHz | 60 ### MHz | 80 ### MHz | 90 ### MHz | 100 ### MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ### 15 | ### 22.89 | ### 23.81 | ### 24.11 | ### 24.27 | ### 24.36 | ### 24.42 | ### 24.72 | ### 24.72 | ### n/a | ### n/a | ### n/a | ### n/a |
| ### 30 | ### 20.14 | ### 21.98 | ### 23.20 | ### 23.35 | ### 23.81 | ### 23.81 | ### 24.27 | ### 24.36 | ### 24.72 | ### 24.84 | ### 24.93 | ### 25.00 |
| ### 60 | ### n/a | ### 20.15 | ### 21.98 | ### 21.98 | ### 22.71 | ### 23.20 | ### 23.35 | ### 23.81 | ### 24.11 | ### 24.50 | ### 24.62 | ### 24.72 |

### For NR TDD frequency range of FR2 (24250 MHz – 52600 MHz) UL and DL peak spectral efficiencies are calculated and presented in Table 6 and 7, respectively. In TDD, effective bandwidth, which is the operating bandwidth normalized considering the uplink/downlink ratio, needs to be considered. The detailed assumptions are presented in Table 1. The DL/UL ratio of 4:1 is evaluated.

### Table 6 NR TDD frequency range of FR2 (24250 MHz – 52600 MHz) DL peak spectral efficiency (bit/s/Hz)

### 8-layer downlink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258, DL/UL 4:1

| ### SCS (KHz) | 50 ### MHz | 100 ### MHz | 200 ### MHz | 400 ### MHz |
| --- | --- | --- | --- | --- |
| ### 60 | ### 43.20 | ### 45.20 | ### 45.20 | ### n/a |
| ### 120 | ### 43.83 | ### 45.20 | ### 45.20 | ### 45.20 |

### Table 7 NR TDD frequency range of FR2 (24250 MHz – 52600 MHz) UL peak spectral efficiency (bit/s/Hz)

### 4-layer uplink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258, DL/UL 4:1

| ### SCS (KHz) | 50 ### MHz | 100 ### MHz | 200 ### MHz | 400 ### MHz |
| --- | --- | --- | --- | --- |
| ### 60 | ### 23.65 | ### 23.65 | ### 23.65 | ### n/a |
| ### 120 | ### 22.93 | ### 23.65 | ### 23.65 | ### 23.65 |

### In what follows, the peak data rate is calculated as specified in § 4.1 of Report ITU-R M.2410-0 [1], using peak spectral efficiency and maximum assignable channel bandwidth.

### Table 8 NR FDD frequency range of FR1 (410 MHz – 7125 MHz) DL peak data rate (Gbps)

### 8-layer downlink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258

| ### SCS (KHz) | 5 ### MHz | 10 ### MHz | 15 ### MHz | 20 ### MHz | 25 ### MHz | 30 ### MHz | 40 ### MHz | 50 ### MHz | 60 ### MHz | 80 ### MHz | 90 ### MHz | 100 ### MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ### 15 | ### 0.214 | ### 0.445 | ### 0.676 | ### 0.907 | ### 1.14 | ### 1.37 | ### 1.85 | ### 2.31 | ### n/a | ### n/a | ### n/a | ### n/a |
| ### 30 | ### 0.188 | ### 0.411 | ### 0.650 | ### 0.873 | ### 1.11 | ### 1.33 | ### 1.81 | ### 2.28 | ### 2.77 | ### 3.71 | ### 4.19 | ### 4.67 |
| ### 60 | ### n/a | ### 0.377 | ### 0.616 | ### 0.822 | ### 1.06 | ### 1.30 | ### 1.75 | ### 2.22 | ### 2.70 | ### 3.66 | ### 4.14 | ### 4.62 |

### Table 9 NR FDD frequency range of FR1 (410 MHz – 7125 MHz) UL peak data rate (Gbps)

### 4-layer uplink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258

| ### SCS (KHz) | 5 ### MHz | 10 ### MHz | 15 ### MHz | 20 ### MHz | 25 ### MHz | 30 ### MHz | 40 ### MHz | 50 ### MHz | 60 ### MHz | 80 ### MHz | 90 ### MHz | 100 ### MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ### 15 | ### 0.114 | ### 0.238 | ### 0.362 | ### 0.485 | ### 0.609 | ### 0.733 | ### 0.989 | ### 1.23 | ### n/a | ### n/a | ### n/a | ### n/a |
| ### 30 | ### 0.100 | ### 0.220 | ### 0.348 | ### 0.467 | ### 0.595 | ### 0.714 | ### 0.970 | ### 1.22 | ### 1.48 | ### 1.99 | ### 2.24 | ### 2.5 |
| ### 60 | ### n/a | ### 0.201 | ### 0.330 | ### 0.439 | ### 0.568 | ### 0.696 | ### 0.934 | ### 1.19 | ### 1.45 | ### 1.96 | ### 1.22 | ### 2.47 |

### Table 10 NR TDD frequency range of FR1 (410 MHz – 7125 MHz) DL peak data rate (Gbps)

### 8-layer downlink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258, DL/UL 4:1

| ### SCS (KHz) | 5 ### MHz | 10 ### MHz | 15 ### MHz | 20 ### MHz | 25 ### MHz | 30 ### MHz | 40 ### MHz | 50 ### MHz | 60 ### MHz | 80 ### MHz | 90 ### MHz | 100 ### MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ### 15 | ### 0.171 | ### 0.356 | ### 0.541 | ### 0.726 | ### 0.911 | ### 1.09 | ### 1.48 | ### 1.85 | ### n/a | ### n/a | ### n/a | ### n/a |
| ### 30 | ### 0.151 | ### 0.329 | ### 0.520 | ### 0.698 | ### 0.890 | ### 1.07 | ### 1.45 | ### 1.82 | ### 2.22 | ### 2.97 | ### 3.35 | ### 3.74 |
| ### 60 | ### n/a | ### 0.301 | ### 0.493 | ### 0.657 | ### 0.849 | ### 1.04 | ### 1.40 | ### 1.78 | ### 2.16 | ### 2.93 | ### 3.31 | ### 3.70 |

### Table 11 NR TDD frequency range of FR1 (410 MHz – 7125 MHz) UL peak data rate (Gbps)

### 4-layer uplink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258, DL/UL 4:1

| ### SCS (KHz) | 5 ### MHz | 10 ### MHz | 15 ### MHz | 20 ### MHz | 25 ### MHz | 30 ### MHz | 40 ### MHz | 50 ### MHz | 60 ### MHz | 80 ### MHz | 90 ### MHz | 100 ### MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ### 15 | ### 0.023 | ### 0.048 | ### 0.072 | ### 0.097 | ### 0.122 | ### 0.146 | ### 0.198 | ### 0.247 | ### n/a | ### n/a | ### n/a | ### n/a |
| ### 30 | ### 0.020 | ### 0.044 | ### 0.069 | ### 0.093 | ### 0.119 | ### 0.143 | ### 0.194 | ### 0.243 | ### 0.297 | ### 0.397 | ### 0.449 | ### 0.500 |
| ### 60 | ### n/a | ### 0.040 | ### 0.066 | ### 0.088 | ### 0.113 | ### 0.139 | ### 0.187 | ### 0.238 | ### 0.289 | ### 0.392 | ### 0.443 | ### 0.494 |

### Table 6 NR TDD frequency range of FR2 (24250 MHz – 52600 MHz) DL peak data rate (Gbps)

### 8-layer downlink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258, DL/UL 4:1

| ### SCS (KHz) | 50 ### MHz | 100 ### MHz | 200 ### MHz | 400 ### MHz |
| --- | --- | --- | --- | --- |
| ### 60 | ### 1.81 | ### 3.61 | ### 7.23 | ### n/a |
| ### 120 | ### 1.75 | ### 3.61 | ### 7.23 | ### 14.46 |

### If we assume %100 transmission in the downlink instead of %80 evaluation shown in Table 6, we get:

### Table 7 NR TDD frequency range of FR2 (24250 MHz – 52600 MHz) DL peak data rate (Gbps)

### 8-layer downlink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258, DL only

| ### SCS (KHz) | 50 ### MHz | 100 ### MHz | 200 ### MHz | 400 ### MHz |
| --- | --- | --- | --- | --- |
| ### 60 | ### 2.26 | ### 4.52 | ### 9.04 | ### n/a |
| ### 120 | ### 2.19 | ### 4.52 | ### 9.04 | ### 18.08 |

Considering the possibility of aggregating minimum two FR2 carriers with 400 BW each (800 MHz in total), we can get a peak data rate of almost 36 Gbps. This would fulfil the minimum requirement of 20 Gbps.

### Table 8 NR TDD frequency range of FR2 (24250 MHz – 52600 MHz) UL peak data rate (Gbps)

### 4-layer uplink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258, DL/UL 4:1

| ### SCS (KHz) | 50 ### MHz | 100 ### MHz | 200 ### MHz | 400 ### MHz |
| --- | --- | --- | --- | --- |
| ### 60 | ### 0.236 | ### 0.473 | ### 0.946 | ### n/a |
| ### 120 | ### 0.229 | ### 0.473 | ### 0.946 | ### 1.89 |

### If we assume %100 transmission in the uplink instead of %20 evaluation shown in Table 8, we get:

### Table 8 NR TDD frequency range of FR2 (24250 MHz – 52600 MHz) UL peak data rate (Gbps)

### 4-layer uplink transmission, with 256QAM modulation, and a maximum coding rate of 0.9258, UL only

| ### SCS (KHz) | 50 ### MHz | 100 ### MHz | 200 ### MHz | 400 ### MHz |
| --- | --- | --- | --- | --- |
| ### 60 | ### 1.18 | ### 2.36 | ### 4.73 | ### n/a |
| ### 120 | ### 1.15 | ### 2.36 | ### 4.73 | ### 9.46 |

### Conclusion

### Observation 1: The Peak Spectral Efficiency values computed in this paper fulfill the ITU-R targets for these technical performance requirements.

### Observation 2: The Peak Data Rate values computed in this paper fulfill the ITU-R targets for these technical performance requirements.

### References

- ITU-R M.2410-0, Minimum requirements related to technical performance for IMT-2020 radio interface(s)

- 3GPP 38.306, NR; User Equipment (UE) radio access capabilities