IMT-2020: Evaluation of control-plane latency Document for discussion and acceptance Canadian Evaluation Group 2nd April 2019

# Abbreviations

3GPP: 3rd Generation Partnership Project

BS: Base-station

BW: Bandwidth

CCE: Control channel element

CP: Control-plane

CSI-RS: Channel state information reference signal

DL: Downlink

DM-RS: Demodulation reference signal

eMBB: enhanced mobile broadband

gNB: g-NodeB

GoS: Grade-of-service

HARQ: Hybrid automatic repeat request

ITU: International Telecommunication Union

mMTC: massive Machine-Type Communications

NR: New Radio

NB-IoT: Narrowband-Internet of Things

PDCCH: Physical downlink control channel, PDSCH: Physical downlink shared channel

PRB: Physical resource block

PT-RS: Phase tracking reference signal

PUCCH: Physical uplink control channel; PUSCH: Physical uplink shared channel

QoS: Quality-of-service

RAN: Radio access network

RE: Resource element

RIT: Radio-interface technology

SCS: Sub-carrier spacing

SDU: Service data unit

SR: Scheduling request

SRS: Sounding reference symbol

SSB: Synchronization signal block

SCM: Stochastic channel model

TRxP: Transmission and reception point

TRS: Tracking reference signal

TTI: Transmission time interval

UE: User equipment

UL: Uplink

UP: User-plane

urLLC: ultra-reliable low-latency communications

# Introduction

The ITU target for control-plane (CP) latency in IMT 2020 has been set to 20 ms [1], while in 3GPP [2], to 10 ms. In this paper, Ericsson makes an evaluation of the CP latency in NR with different configurations and shows that the target can be reached in both FDD and TDD schemes.

# CP Latency

## Definition

ITU has defined the CP latency and the requirement on it as follows [1]:

Control-plane latency refers to the transition time from a most “battery efficient” state (e.g. Idle state) to the start of continuous data transfer (e.g. Active state).

This requirement is defined for the purpose of evaluation in the eMBB and urLLC usage scenarios.

The minimum requirement for control-plane latency is 20 ms. Proponents are encouraged to consider lower control-plane latency, e.g. 10 ms.

A most battery efficient state should be interpreted here as Idle or Inactive state, both of which have the same energy consumption.

The start of continuous data transfer should be interpreted as the point in time where the UE is ready to be scheduled, i.e. when it enters the Active state.

## Control-plane signalling

According to the definition of CP latency, it is necessary to study the transition from the RRC Inactive state to the RRC Active state, as shown in Figure 1.

Figure 1: Illustration of CP signalling during transition from Inactive to Active states.

## Processing delay

In this paper, the assumption is that the minimum timing capabilities have been agreed for NR. With the UE capability, the minimum UL timing is set to be 3 symbols for both 15 kHz and 30 kHz SCS. For 120kHz, the assumption is made of 9 symbols timing.

With mini-slots, the TTIs can have different lengths and Ericsson has therefore counted the processing in terms of the shortest considered TTI, which is 4 symbols in this paper. For simplicity, the processing delay is therefore set to 1 TTI for both 15 and 30 kHz SCS and 3 TTI at 120 kHz SCS, in both gNB and UE.

The RRC processing delays are assumed to be of a fixed value of 3 ms, as discussed in [3].

## Achievable latency in FDD

For the evaluation of latency, it is assumed that the UE works with n+2 timing and the gNB with n+3 timing as the fastest options, i.e. that the processing budget is 1 and 2 TTIs, respectively. This is for 15 and 30 kHz SCS. For 120kHz, the processing delay is doubled in TTIs, giving n+3 timing for the UE and n+5 timing for gNB.

With the assumptions described above, the resulting CP latency will be as outlined in Table 1. As can be seen, the total worst-case delay sums up in the range 9-14 TTIs + 6ms for FDD.

| Component | Description | Latency |  |
| --- | --- | --- | --- |
|  |  | 15/30kHz | 120kHz |
| 1 | Worst-case delay due to RACH scheduling period (1TTI period) | 1TTI | 1TTI |
| 2 | Transmission of RACH Preamble | 1TTI | 1TTI |
| 3 | Preamble detection and processing in gNB | 1TTI | 3TTI |
| 4 | Transmission of RA response | 1TTI | 1TTI |
| 5 | UE Processing Delay (decoding of scheduling grant, timing alignment and C-RNTI assignment + L1 encoding of RRC Connection Request) | 1TTI | 2TTI |
| 6 | Transmission of RRC Connection Resume Request | 1 TTI | 1 TTI |
| 7 | Processing delay in gNB (L2 and RRC) | 3 ms | 3 ms |
| 8 | Transmission of RRC Connection Resume (and UL grant) | 1 TTI | 1 TTI |
| 9 | Processing delay in the UE (L2 and RRC) | 3 ms | 3 ms |
| 10 | Transmission of RRC Connection Resume Complete (including NAS Service Request) | 1 TTI | 1 TTI |
| 11 | Processing delay in gNB (Uu –> S1-C) | 1 TTI | 3 TTI |
|  | Total delay | 9 TTI + 6 ms | 14 TTI + 6ms |

Table 1: CP latency in TTIs in NR Rel-15 FDD.

- The worst-case CP latency in NR Rel-15 FDD is estimated to 9TTI+6ms at 15/30kHz SCS and 14TTI+6ms at 120kHz SCS.

### Achievable FDD latency in ms

With different TTI lengths and SCSs, the absolute delay will differ, as shown in Table 2. Here, the values calculated above in Table 1 have been assumed.

| CP latency (ms) | 15kHz SCS | 30kHz SCS | 120kHz SCS |
| --- | --- | --- | --- |
| 14-symbol TTI | 15 | 10.5 | 7.8 |
| 7-symbol TTI | 10.5 | 8.3 | 6.9 |
| 4-symbol TTI | 8.6 | 7.3 | 6.5 |

Table 2: Achievable CP latency for NR Rel-15 in ms.

As can be seen in the table, all considered configurations fulfil the 20ms 5G target on CP latency, and almost all configurations also reach the 10ms target.

- NR Rel-15 FDD can reach the 3GPP and ITU 5G targets on CP latency.

## Achievable latency in TDD

For the TDD slot sequence, two cases are studied: an alternating UL-DL sequence, and a DL-heavy UL-DL-DL-DL sequence. Due to the slot sequence, additional alignment delays are added.

With the assumptions described above, the resulting CP latency will be as outlined in Table 3. As can be seen, the total worst-case delay sums up in the range 12-26 TTI + 6ms for TDD.

| Component | Description | UL-DL Latency |  | UL-DL-DL-DL Latency |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 15/30kHz | 120kHz | 15/30kHz | 120kHz |
| 1 | Worst-case delay due to RACH scheduling period (1TTI period) | 2 TTI | 2 TTI | 4 TTI | 4 TTI |
| 2 | Transmission of RACH Preamble | 1 TTI | 1 TTI | 1 TTI | 1 TTI |
| 3 | Preamble detection and processing in gNB | 1 TTI | 3 TTI | 1 TTI | 3 TTI |
| 4 | DL slot alignment | 1 TTI | 1 TTI | 0 TTI | 1 TTI |
| 5 | Transmission of RA response | 1 TTI | 1 TTI | 1 TTI | 1 TTI |
| 6 | UE Processing Delay (decoding of scheduling grant, timing alignment and C-RNTI assignment + L1 encoding of RRC Connection Request) | 1 TTI | 3 TTI | 1 TTI | 3 TTI |
| 7 | UL slot alignment | 1 TTI | 1 TTI | 0 TTI | 3 TTI |
| 8 | Transmission of RRC Connection Resume Request | 1 TTI | 1 TTI | 1 TTI | 1 TTI |
| 9 | Processing delay in gNB (L2 and RRC) | 3 ms | 3 ms | 3 ms | 3 ms |
| 10 | DL slot alignment | 1 TTI | 1 TTI | 0 TTI | 1 TTI |
| 11 | Transmission of RRC Connection Resume (and UL grant) | 1 TTI | 1 TTI | 1 TTI | 1 TTI |
| 12 | Processing delay in the UE (L2 and RRC) | 3 ms | 3 ms | 3 ms | 3 ms |
| 13 | UL slot alignment | 1 TTI | 1 TTI | 0 TTI | 3 TTI |
| 14 | Transmission of RRC Connection Resume Complete (including NAS Service Request) | 1 TTI | 1 TTI | 1 TTI | 1 TTI |
| 15 | Processing delay in gNB (Uu –> S1-C) | 1 TTI | 3 TTI | 1 TTI | 3 TTI |
|  | Total delay | 14 TTI + 6 ms | 20 TTI + 6 ms | 12 TTI + 6 ms | 26 TTI + 6 ms |

Table 3: CP latency in TTIs in NR Rel-15 TDD.

- The worst-case CP latency in NR Rel-15 TDD with alternating UL-DL pattern is estimated to 14TTI+6ms for 15/30kHz SCS and 20TTI+6ms for 120kHz SCS.

### Achievable TDD latency in ms

With different TTI lengths and SCSs, the absolute delay will differ, as shown in Table 2. For the alternating UL-DL TDD pattern, the latency obtained is indicated in Table 4, and for the UL-DL-DL-DL TDD pattern, the resulting latency is shown in Table 5.

| CP latency (ms) | 15kHz SCS | 30kHz SCS | 120kHz SCS |
| --- | --- | --- | --- |
| 14-symbol TTI | 20 | 13 | 8.5 |
| 7-symbol TTI | 13 | 9.5 | 7.3 |
| 4-symbol TTI | 10 | 8.0 | 6.7 |

Table 4: Achievable CP latency for NR Rel-15 in ms for TDD with alternating UL-DL pattern.

| CP latency (ms) | 15kHz SCS | 30kHz SCS | 120kHz SCS |
| --- | --- | --- | --- |
| 14-symbol TTI | 18 | 12 | 9.3 |
| 7-symbol TTI | 12 | 9.0 | 7.6 |
| 4-symbol TTI | 9.4 | 7.7 | 6.9 |

Table 5: Achievable CP latency for NR Rel-15 in ms for TDD with UL-DL-DL-DL pattern.

As can be seen in the tables, all considered configurations fulfil the 20ms 5G target on CP latency for the alternating UL-DL TDD pattern, and several configurations can also fulfill the 10ms requirement.

- The worst-case CP latency in NR Rel-15 TDD with alternating UL-DL pattern is estimated to 14TTI+6ms for 15/30kHz SCS and 20TTI+6ms for 120kHz SCS.

The observation above may be used for the discussion on CP latency.

# Conclusion

In the previous section, the following observations were made:

- The worst-case CP latency in NR Rel-15 FDD is estimated to 9TTI+6ms at 15/30kHz SCS and 14TTI+6ms at 120kHz SCS.

- NR Rel-15 FDD can reach the 3GPP and ITU 5G targets on CP latency.

- The worst-case CP latency in NR Rel-15 TDD with alternating UL-DL pattern is estimated to 14TTI+6ms for 15/30kHz SCS and 20TTI+6ms for 120kHz SCS.

- NR Rel-15 TDD can reach the ITU and 3GPP 5G targets on CP latency.

These observations can be used to conclude on CP latency.

# References

ITU-R Report M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”

3GPP TR 38.913, “Study on scenarios and requirements for next generation access technologies”

R2-1802686, “RRC UE processing time for Standalone NR”, Ericsson

ITU-R Report M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”