IMT-2020: INITIAL EVALUATION REPORT (NR) Document for discussion CEG meeting Montreal, Canada September 12th, 2019 Source: Wireless Lab, EMT Centre, INRS.

# Abbreviations

3GPP: 3rd Generation Partnership Project

ASE: Average Spectral Efficiency

BS: Base-station

BW: Bandwidth

CP: Control-plane or Cyclic Prefix

CSI: Channel state information

CSI-RS: Channel state information reference signal

DU: Dense Urban

DL: Downlink

DMRS: Demodulation reference signal

eMBB: enhanced mobile broadband

FDM: Frequency division multiplexing

HARQ: Hybrid automatic repeat request

InH: Indoor Hotspot

ITU: International Telecommunication Union

MIMO: multiple-input multiple-output

NR: New Radio

OFDM: Orthogonal frequency division multiplexing

PDCCH: Physical downlink control channel, PDSCH: Physical downlink shared channel

PUCCH: Physical uplink control channel; PUSCH: Physical uplink shared channel

RIT: Radio-interface technology

SCS: Sub-carrier spacing

SINR/SNR: Signal-to-interference noise ratio/Signal-to-noise ratio

SCM: Stochastic channel model

TRxP: Transmission and reception point

UE: User equipment

USE: User spectral Efficiency

UL: Uplink

- Introduction

Research and development of signal processing algorithms for 5G NR require a realistic and flexible simulation environment. In this work, we use a MATLAB-based downlink/uplink physical-layer simulator for the evaluation of submitted 5G standard candidates.

In this contribution, we provide the simulation results of the 3GPP candidate in terms of ASE and 5% USE. The results for eMBB with its 3 test environments (i.e., Indoor hotspot, dense urban and rural) are provided in the downlink case. A summary of the simulation performed is shown in the table 1 below:

Table 1. Tested environments

| Test environment | Does the Evaluation Report indicate that the minimum technical performance requirements are met in the test environment? |
| --- | --- |
| Indoor Hotspot-eMBB | Yes No Partial evaluation |
| Dense Urban-eMBB | Yes No Partial evaluation |
| Rural-eMBB | Yes No Partial evaluation |
| Urban Macro–mMTC | Yes No Partial evaluation |
| Urban Macro–URLLC | Yes No Partial evaluation |

In table. 2, we reproduce the list of parameters to be evaluated using link- and system-level simulation from M.2412 [1]

Table 2. Parameters to be evaluated

| Minimum technical performance requirements item (5.2.4.3.x), units, and Report ITU-R M.2410-0 section reference(1) | Category |  |  | Required value | Requirement met? |
| --- | --- | --- | --- | --- | --- |
|  | Usage scenario | Test environment | DL or UL |  |  |
| 5.2.4.3.4 5th percentile user spectral efficiency (bit/s/Hz) (4.4) | eMBB | Indoor Hotspot – eMBB | DL | 0.3 | Yes No |
|  |  |  | UL | 0.21 | Yes No |
|  | eMBB | Dense Urban – eMBB | DL | 0.225 | Yes No |
|  |  |  | UL | 0.15 | Yes No |
|  | eMBB | Rural – eMBB | DL | 0.12 | Yes No |
|  |  |  | UL | 0.045 | Yes No |
| 5.2.4.3.5 Average spectral efficiency (bit/s/Hz/ TRxP) (4.5) | eMBB | Indoor Hotspot – eMBB | DL | 9 | Yes No |
|  |  |  | UL | 6.75 | Yes No |
|  | eMBB | Dense Urban – eMBB | DL | 7.8 | Yes No |
|  |  |  | UL | 5.4 | Yes No |
|  | eMBB | Rural – eMBB | DL | 3.3 | Yes No |
|  |  |  | UL | 1.6 | Yes No |
| 5.2.4.3.9 Connection density (devices/km2) (4.8) | mMTC | Urban Macro – mMTC | UL | 1 000 000 | Yes No |
| 5.2.4.3.11 Reliability (4.10) | URLLC | Urban Macro –URLLC | UL or DL | 1-10−5 success probability of transmitting a layer 2 PDU (protocol data unit) of size 32 bytes within 1 ms in channel quality of coverage edge | Yes No |
| 5.2.4.3.13 Mobility Traffic channel link data rates (bit/s/Hz) (4.11) | eMBB | Indoor Hotspot – eMBB | UL | 1.5 (10 km/h) | Yes No |
|  | eMBB | Dense Urban – eMBB | UL | 1.12 (30 km/h) | Yes No |
|  | eMBB | Rural – eMBB | UL | 0.8 (120 km/h) | Yes No |
|  |  |  |  | 0.45 (500 km/h) | Yes No |

- Evaluation summary

Based on the configuration and assumption in section 3, the evaluation results are shown in the following tables:

- Results of eMBB Indoor Hotspot Test Environment

Table 3. Evaluation Result of Indoor Hotspot – eMBB (Configuration A)

| Candidate: 3GPP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| eMBB – Indoor hotspot |  | Configuration A (4GHz) |  |  |  |  |
| Metric | Link | M.2410 | INRS | UofT | MEDIATEK | TPCEG |
| ASE [bps/Hz/TRxP] | DL | 9.000 | 10.75 | … | 11.120 | 10.544 |
|  | UL | 6.750 | … | … | 8.820 | 8.588 |
| 5% USE [bps/Hz] | DL | 0.300 | 0.331 | … | 0.330 | 0.360 |
|  | UL | 0.210 | … | … | 0.590 | 0.411 |

Table 4. Evaluation Result of Indoor Hotspot – eMBB (Configuration B)

| Candidate: 3GPP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| eMBB – Indoor hotspot |  | Configuration B (30GHz) |  |  |  |  |
| Metric | Link | M.2410 | INRS | UofT | MEDIATEK | TPCEG |
| ASE [bps/Hz/TRxP] | DL | 9.000 | … | … | X | X |
|  | UL | 6.750 | … | … | X | X |
| 5% USE [bps/Hz] | DL | 0.300 | … | … | X | X |
|  | UL | 0.210 | … | … | X | X |

- Results of eMBB Dense Test Environment

Table 5. Evaluation Result of Dense Urban – eMBB (Configuration A)

| Candidate: 3GPP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| eMBB – Dense Urban |  | Configuration A (4GHz) |  |  |  |  |
| Metric | Link | M.2410 | INRS | UofT | MEDIATEK | TPCEG |
| ASE [bps/Hz/TRxP] | DL | 7.800 | 11.200 | 11.270 | 11.390 | 11. 867 |
|  | UL | 5.400 | … | 6.512 | 8.790 | 8.702 |
| 5% USE [bps/Hz] | DL | 0.225 | 0.248 | 0.380 | 0.400 | 0.421 |
|  | UL | 0.150 | … | 0.228 | 0.505 | 0.347 |

Table 6. Evaluation Result of Dense Urban – eMBB (Configuration B)

| Candidate: 3GPP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| eMBB – Dense Urban |  | Configuration B (30GHz) |  |  |  |  |
| Metric | Link | M.2410 | INRS | UofT | MEDIATEK | TPCEG |
| ASE [bps/Hz/TRxP] | DL | 7.800 | 13.752 | 11.360 | X | X |
|  | UL | 5.400 | … | 6.397 | X | X |
| 5% USE [bps/Hz] | DL | 0.225 | 0.4902 | 0.350 | X | X |
|  | UL | 0.1500 | … | 0.264 | X | X |

- Results of eMBB Rural Test Environment

Table 7. Evaluation Result of Rural Urban – eMBB (Configuration A)

| Candidate: 3GPP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| eMBB – Rural |  | Configuration A (700MHz) |  |  |  |  |
| Metric | Link | M.2410 | INRS | UofT | MEDIATEK | TPCEG |
| ASE [bps/Hz/TRxP] | DL | 3.3 | 11.6 | 6.152 | 5.64 | 5.774 |
|  | UL | 1.6 | … | 6.951 | 4.637 | 6.243 |
| 5% USE [bps/Hz] | DL | 0.12 | 0.1744 | 0.162 | 0.128 | 0.155 |
|  | UL | 0.045 | … | 0.248 | 0.231 | 0.113 |

Table 8. Evaluation Result of Rural Urban – eMBB (Configuration B)

| Candidate: 3GPP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| eMBB – Rural |  | Configuration B (4GHz) |  |  |  |  |
| Metric | Link | M.2410 | INRS | UofT | MEDIATEK | TPCEG |
| ASE [bps/Hz/TRxP] | DL | 3.3 | 13.8915 | 6.48 | 11.64 | 11.063 |
|  | UL | 1.6 | … | 7.125 | 3.988 | 6.231 |
| 5% USE [bps/Hz] | DL | 0.12 | 0.278 | 0.187 | 0.452 | 0.321 |
|  | UL | 0.045 | … | 0.189 | 0.19 | 0.126 |

- Evaluation Assumptions and Configuration

- Indoor Hotspot – eMBB

Table 9. Assumptions and Configuration of Indoor Hotspot-eMBB (Downlink Case)

| Configuration A - Downlink |  |  |  |
| --- | --- | --- | --- |
|  | ITU-R M.2412 | MEDIATEK | INRS-EMT |
| Baseline configuration |  |  |  |
| Carrier frequency for evaluation | 4 GHz | 4 GHz | 4 GHz |
| BS antenna height | 3 m | 3 m | 3 m |
| Total transmit power per TRxP | 24 dBm for 20 MHz bandwidth 21 dBm for 10 MHz bandwidth | 21 dBm for 10 MHz bandwidth | 21 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm | 23 dBm | 23 dBm |
| Inter-site distance | 20 m | 20 m | 20 m |
| Number of antenna elements per TRxP | Up to 256 Tx/Rx | 32Tx, cross-polarized antenna (M,N,P,Mg,Ng) = (4,4,2,1,1) | 32Tx/Rx, (M,N,P,Mg,Ng) = (4,4,2,1,1) |
| Number of UE antenna elements | Up to 8 Tx/Rx | 4Rx with 0°,90° polarization (M,N,P,Mg,Ng) = (1,2,2,1,1) | 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1) |
| Device deployment | 100% indoor, Randomly and uniformly distributed over the area | 100% indoor, Randomly and uniformly distributed over the area | 100% indoor, Randomly and uniformly distributed over the area |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction | Aligned with reference | Aligned with reference |
| UE speeds of interest | 100% indoor, 3 km/h | 100% indoor, 3 km/h | 100% indoor, 3 km/h |
| Inter-site interference modeling | Explicitly modelled | Explicitly modelled | Explicitly modelled |
| BS noise figure | 5 dB | 5 dB | 5 dB |
| UE noise figure | 7 dB | 7 dB | 7 dB |
| BS antenna element gain | 5 dBi | 5 dBi | 5 dBi |
| UE antenna element gain | 0 dBi | 0 dBi | 0 dBi |
| Thermal noise level | ‒174 dBm/Hz | ‒174 dBm/Hz | ‒174 dBm/Hz |
| Traffic model | Full buffer | Full buffer | Full buffer |
| Simulation bandwidth | 20 MHz for TDD, 10 MHz+10 MHz for FDD | 10 MHz+10 MHz for FDD | 10 MHz+10 MHz for FDD |
| UE density | 10 UEs per TRxP, randomly and uniformly dropped throughout the geographical area | 10 UEs per TRxP, randomly and uniformly dropped throughout the geographical area | 10 UEs per TRxP, randomly and uniformly dropped throughout the geographical area |
| UE antenna height | 1.5 m | 1.5 m | 1.5 m |
| Channdel Model | InH_A, InH_B | InH_A | InH_A |
| Additional parameters |  |  |  |
| Subcarrier spacing |  | 15 kHz | 15 kHz |
| Symbols number per slot |  | 14 | 14 |
| Number of TXRU per TRxP |  | 32TXRU (Mp,Np,P,Mg,Ng) = (4,4,2,1,1) | 32TXRU (Mp,Np,P,Mg,Ng) = (4,4,2,1,1) |
| Number of TXRU per UE |  | 4TXRU, (1-to-1 mapping) (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) | 4TXRU, (1-to-1 mapping) (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) |
| TRxP number per site |  | 1 | 1 |
| Mechanic tilt |  | 180deg in GCS (pointing to the ground) | 180deg in GCS (pointing to the ground) |
| Electronic tilt |  | 90deg in LCS | 90deg in LCS |
| Scheduling |  | MU-PF | Round Robin |
| ACK/NACK delay |  | Next available UL slot | Next available UL slot |
| MIMO mode |  | MU-MIMO with rank 4 adaptation per user; Maximum MU layer = 12 | MU-MIMO |
| Guard band ratio |  | FDD: 6.4% (for 10 MHz) | FDD: 6.4% (for 10 MHz) |
| BS receiver type |  | MMSE-IRC | MMSE-IRC |
| Precoder derivation |  | FDD: NR Type II codebook based | FDD: NR Type II codebook based |
| Channel estimation |  | Non-ideal | Non-ideal |
| Waveform |  | OFDM | OFDM |
| Polarized antenna model |  | Model-2 (TR36.873) | Model-2 (TR36.873) |
| Modulation |  | Up to 256QAM | Up to 256QAM |

Table 10. Assumptions and Configuration of Indoor Hotspot-eMBB (Uplink Case)

| Configuration A - Uplink |  |  |  |
| --- | --- | --- | --- |
|  | ITU-R M.2412 | MEDIATEK | INRS-EMT |
| Baseline configuration |  |  |  |
| Carrier frequency for evaluation | 4 GHz | 4 GHz | 4 GHz |
| BS antenna height | 3 m | 3 m | 3 m |
| Total transmit power per TRxP | 24 dBm for 20 MHz bandwidth 21 dBm for 10 MHz bandwidth | 21 dBm for 10 MHz bandwidth | 21 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm | 23 dBm | 23 dBm |
| Inter-site distance | 20 m | 20 m | 20 m |
| Number of antenna elements per TRxP | Up to 256 Tx/Rx | 32Rx cross-polarized antenna (M,N,P,Mg,Ng) = (4,4,2,1,1) | 32Rx cross-polarized antenna (M,N,P,Mg,Ng) = (4,4,2,1,1) |
| Number of UE antenna elements | Up to 8 Tx/Rx | 2Tx, (M,N,P,Mg,Ng) = (1,1,2,1,1) with 0°,90° polarization | 2Tx, (M,N,P,Mg,Ng) = (1,1,2,1,1) with 0°,90° polarization |
| Number of TXRU per UE |  | 2TXRU | 2TXRU |
| Device deployment | 100% indoor, Randomly and uniformly distributed over the area | 100% indoor, Randomly and uniformly distributed over the area | 100% indoor, Randomly and uniformly distributed over the area |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction | Aligned with reference | Aligned with reference |
| UE speeds of interest | 100% indoor, 3 km/h | 100% indoor, 3 km/h | 100% indoor, 3 km/h |
| Inter-site interference modeling | Explicitly modelled | Explicitly modelled | Explicitly modelled |
| BS noise figure | 5 dB | 5 dB | 5 dB |
| UE noise figure | 7 dB | 7 dB | 7 dB |
| BS antenna element gain | 5 dBi | 5 dBi | 5 dBi |
| UE antenna element gain | 0 dBi | 0 dBi | 0 dBi |
| Thermal noise level | ‒174 dBm/Hz | ‒174 dBm/Hz | ‒174 dBm/Hz |
| Traffic model | Full buffer | Full buffer | Full buffer |
| Simulation bandwidth | 20 MHz for TDD, 10 MHz+10 MHz for FDD | 10 MHz+10 MHz for FDD | 10 MHz+10 MHz for FDD |
| UE density | 10 UEs per TRxP randomly and uniformly dropped throughout the geographical area | 10 UEs per TRxP randomly and uniformly dropped throughout the geographical area | 10 UEs per TRxP randomly and uniformly dropped throughout the geographical area |
| UE antenna height | 1.5 m | 1.5 m | 1.5 m |
| Channdel Model | InH_A, InH_B | InH_A | InH_A |
| Additional parameters |  |  |  |
| Subcarrier spacing |  | 15 kHz | 15 kHz |
| Symbols number per slot |  | 14 | 14 |
| Number of TXRU per TRxP |  | 16TXRU, Vertical 1-to-2 (Mp,Np,P,Mg,Ng) = (2,4,2,1,1) | 16TXRU, Vertical 1-to-2 (Mp,Np,P,Mg,Ng) = (2,4,2,1,1) |
| Number of TXRU per UE |  | 2TXRU, (1-to-1 mapping) (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) | 2TXRU, (1-to-1 mapping) (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) |
| TRxP number per site |  | 1 | 1 |
| Mechanic tilt |  | 180deg in GCS (pointing to the ground) | 180deg in GCS (pointing to the ground) |
| Electronic tilt |  | 90deg in LCS | 90deg in LCS |
| Scheduling |  | SU-PF | Round Robin |
| ACK/NACK delay |  | Next available UL slot | Next available UL slot |
| MIMO mode |  | SU-MIMO with rank 2/4 adaptation | SU-MIMO |
| BS receiver type |  | MMSE-IRC | MMSE-IRC |
| UE precoder scheme |  | Codebook based | Codebook based |
| UL CSI derivation |  | Non-precoded SRS based, with delay | Non-precoded SRS based, with delay |
| Power control |  | α= 0.6, P0 =-60 dBm | α= 0.6, P0 =-60 dBm |
| RB allocation for Power backoff model |  | Continuous: follow TS 38.101; Non-continuous: additional 2 RB reduction | Continuous: follow TS 38.101; Non-continuous: additional 2 RB reduction |
| Channel estimation |  | Non-ideal | Non-ideal |
| Waveform |  | OFDM | OFDM |
| Polarized antenna model |  | Model-2 (TR36.873) | Model-2 (TR36.873) |
| Modulation |  | Up to 256QAM | Up to 256QAM |

- Dense Urban – eMBB

Table 11. Assumptions and Configuration of Dense Urban-eMBB (Downlink Case)

| Configuration A - Downlink |  |  |  |
| --- | --- | --- | --- |
|  | ITU-R M.2412 | MEDIATEK | INRS-EMT |
| Baseline configuration |  |  |  |
| Carrier frequency for evaluation | 1 layer (Macro) with 4 GHz | 4 GHz | 4 GHz |
| BS antenna height | 25 m | 25 m | 25 m |
| Total transmit power per TRxP | 44 dBm for 20 MHz bandwidth 41 dBm for 10 MHz bandwidth | 41 dBm for 10 MHz bandwidth | 41 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type | 20% high loss, 80% low loss | 20% high loss, 80% low loss | 20% high loss, 80% low loss |
| Inter-site distance | 200 m | 200 m | 200 m |
| Number of antenna elements per TRxP | Up to 256 Tx/Rx | 128Tx cross-polarized antenna (M,N,P,Mg,Ng) = (8,8,2,1,1) | 128Tx cross-polarized antenna (M,N,P,Mg,Ng) = (8,8,2,1,1) |
| Number of UE antenna elements | Up to 8 Tx/Rx | 4Rx (M,N,P,Mg,Ng) = (1,2,2,1,1) with 0°,90° polarization | 4Rx (M,N,P,Mg,Ng) = (1,2,2,1,1) with 0°,90° polarization |
| Device deployment | 80% indoor, 20% outdoor (in-car) Randomly and uniformly distributed over the area under Macro layer | Aligned with reference | Aligned with reference |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction. | Aligned with reference | Aligned with reference |
| UE speeds of interest | Indoor users: 3 km/h Outdoor users (in-car): 30 km/h | Aligned with reference | Aligned with reference |
| Inter-site interference modeling | Explicitly modelled | Explicitly modelled | Explicitly modelled |
| BS noise figure | 5 dB | 5 dB | 5 dB |
| UE noise figure | 7 dB | 7 dB | 7 dB |
| BS antenna element gain | 8 dBi | 8 dBi | 8 dBi |
| UE antenna element gain | 0 dBi | 0 dBi | 0 dBi |
| Thermal noise level | ‒174 dBm/Hz | ‒174 dBm/Hz | ‒174 dBm/Hz |
| Traffic model | Full buffer | Full buffer | Full buffer |
| Simulation bandwidth | 20 MHz for TDD, 10 MHz+10 MHz for FDD | 10 MHz+10 MHz for FDD | 10 MHz+10 MHz for FDD |
| UE density | 10 UEs per TRxP Randomly and uniformly distributed over the area under Macro layer | Aligned with reference | Aligned with reference |
| UE antenna height | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) | Aligned with reference | Aligned with reference |
| Channdel Model | Macro layer: UMa_A, UMa_B Micro layer: UMi_A, UMi_B | UMa_A | UMa_A |
| Additional parameters |  |  |  |
| Subcarrier spacing |  | 15 kHz | 15 kHz |
| Symbols number per slot |  | 14 | 14 |
| Number of TXRU per TRxP |  | 32TXRU, Vertical 2-to-8 (Mp,Np,P,Mg,Ng) = (2,8,2,1,1) | 32TXRU, Vertical 2-to-8 (Mp,Np,P,Mg,Ng) = (2,8,2,1,1) |
| TRxP number per site |  | 3 | 3 |
| Number of TXRU per UE |  | 4TXRU (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) (1-to-1 mapping) | 4TXRU (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) (1-to-1 mapping) |
| Mechanic tilt |  | 90deg in GCS (pointing to the horizontal direction) | 90deg in GCS (pointing to the horizontal direction) |
| Electronic tilt |  | 105deg in LCS | 105deg in LCS |
| Scheduling |  | MU-PF | Round Robin |
| ACK/NACK delay |  | Next available UL slot | Next available UL slot |
| MIMO mode |  | MU-MIMO with rank 4 adaptation per user Maximum MU layer = 12 | MU-MIMO |
| Guard band ratio |  | FDD: 6.4% (for 10 MHz) | FDD: 6.4% (for 10 MHz) |
| BS receiver type |  | MMSE-IRC | MMSE-IRC |
| Precoder derivation |  | FDD: NR Type II codebook based | FDD: NR Type II codebook based |
| Channel estimation |  | Non-ideal | Non-ideal |
| Waveform |  | OFDM | OFDM |
| Polarized antenna model |  | Model-2 (TR36.873) | Model-2 (TR36.873) |
| Modulation |  | Up to 256QAM | Up to 256QAM |

Table 12. Assumptions and Configuration of Dense Urban-eMBB (Uplink Case)

| Configuration A - Uplink |  |  |  |
| --- | --- | --- | --- |
|  | ITU-R M.2412 | MEDIATEK | INRS-EMT |
| Baseline configuration |  |  |  |
| Carrier frequency for evaluation | 1 layer (Macro) with 4 GHz | Aligned with reference | Aligned with reference |
| BS antenna height | 25 m | 25 m | 25 m |
| Total transmit power per TRxP | 44 dBm for 20 MHz bandwidth 41 dBm for 10 MHz bandwidth | 41 dBm for 10 MHz bandwidth | 41 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type | 20% high loss, 80% low loss | 20% high loss, 80% low loss | 20% high loss, 80% low loss |
| Inter-site distance | 200 m | 200 m | 200 m |
| Number of antenna elements per TRxP | Up to 256 Tx/Rx | 128Rx cross-polarized antenna (M,N,P,Mg,Ng) = (8,8,2,1,1) | 128Rx cross-polarized antenna (M,N,P,Mg,Ng) = (8,8,2,1,1) |
| Number of UE antenna elements | Up to 8 Tx/Rx | 2Tx (M,N,P,Mg,Ng) = (1,1,2,1,1) with 0°,90° polarization | 2Tx (M,N,P,Mg,Ng) = (1,1,2,1,1) with 0°,90° polarization |
| Device deployment | 80% indoor, 20% outdoor (in-car) Randomly and uniformly distributed over the area under Macro layer | Aligned with reference | Aligned with reference |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction. | Aligned with reference | Aligned with reference |
| UE speeds of interest | Indoor users: 3 km/h Outdoor users (in-car): 30 km/h | Aligned with reference | Aligned with reference |
| Inter-site interference modeling | Explicitly modelled | Explicitly modelled | Explicitly modelled |
| BS noise figure | 5 dB | 5 dB | 5 dB |
| UE noise figure | 7 dB | 7 dB | 7 dB |
| BS antenna element gain | 8 dBi | 8 dBi | 8 dBi |
| UE antenna element gain | 0 dBi | 0 dBi | 0 dBi |
| Thermal noise level | ‒174 dBm/Hz | ‒174 dBm/Hz | ‒174 dBm/Hz |
| Traffic model | Full buffer | Full buffer | Full buffer |
| Simulation bandwidth | 20 MHz for TDD, 10 MHz+10 MHz for FDD | 10 MHz+10 MHz for FDD | 10 MHz+10 MHz for FDD |
| UE density | 10 UEs per TRxP Randomly and uniformly distributed over the area under Macro layer | Aligned with reference | Aligned with reference |
| UE antenna height | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) | Aligned with reference | Aligned with reference |
| Channdel Model | UMa_A, UMa_B | UMa_A | UMa_A |
| Additional parameters |  |  |  |
| Subcarrier spacing |  | 15 kHz | 15 kHz |
| Symbols number per slot |  | 14 | 14 |
| Number of TXRU per TRxP |  | 16TXRU (Mp,Np,P,Mg,Ng) = (1,8,2,1,1) Vertical 1-to-8 | 16TXRU (Mp,Np,P,Mg,Ng) = (1,8,2,1,1) Vertical 1-to-8 |
| TRxP number per site |  | 3 | 3 |
| Number of TXRU per UE |  | 2TXRU (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) (1-to-1 mapping) | 2TXRU (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) (1-to-1 mapping) |
| Mechanic tilt |  | 90deg in GCS (pointing to the horizontal direction) | 90deg in GCS (pointing to the horizontal direction) |
| Electronic tilt |  | 105deg in LCS | 105deg in LCS |
| Scheduling |  | SU-PF | Robin Robin |
| ACK/NACK delay |  | Next available UL slot | Next available UL slot |
| MIMO mode |  | SU-MIMO with rank 2/4 adaptation | SU-MIMO |
| BS receiver type |  | MMSE-IRC | MMSE-IRC |
| UE precoder scheme |  | Codebook based | Codebook based |
| UL CSI derivation |  | Non-precoded SRS based, with delay | Non-precoded SRS based, with delay |
| Power control |  | α= 0.6, P0 =-60 dBm | α= 0.6, P0 =-60 dBm |
| RB allocation for Power backoff model |  | Continuous: follow TS 38.101; Non-continuous: additional 2 RB reduction | Continuous: follow TS 38.101; Non-continuous: additional 2 RB reduction |
| Channel estimation |  | Non-ideal | Non-ideal |
| Waveform |  | OFDM | OFDM |
| Polarized antenna model |  | Model-2 (TR36.873) | Model-2 (TR36.873) |
| Modulation |  | Up to 256QAM | Up to 256QAM |

- Rural – eMBB

Table 13. Assumptions and Configuration of Rural-eMBB (Downlink Case) – Configuration A

| Configuration A - Downlink |  |  |  |
| --- | --- | --- | --- |
|  | ITU-R M.2412 | MEDIATEK | INRS-EMT |
| Baseline configuration |  |  |  |
| Carrier frequency for evaluation | 700 MHz | 700 MHz | 700 MHz |
| BS antenna height | 35 m | 35 m | 35 m |
| Total transmit power per TRxP | 49 dBm for 20 MHz bandwidth 46 dBm for 10 MHz bandwidth | 46 dBm for 10 MHz bandwidth | 46 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type | 100% low loss | 100% low loss | 100% low loss |
| Inter-site distance | 1732 m | 1732 m | 1732 m |
| Number of antenna elements per TRxP | Up to 64 Tx/Rx | 64Tx cross-polarized antenna (M,N,P,Mg,Ng) = (8,4,2,1,1) (dH,dV) = (0.5, 0.8)λ | 64Tx cross-polarized antenna (M,N,P,Mg,Ng) = (8,4,2,1,1) (dH,dV) = (0.5, 0.8)λ |
| Number of UE antenna elements | Up to 4 Tx/Rx | 2Rx (M,N,P,Mg,Ng) = (1,1,2,1,1) 0°,90° polarization | 2Rx (M,N,P,Mg,Ng) = (1,1,2,1,1) 0°,90° polarization |
| Device deployment | 50% indoor, 50% outdoor (in-car) Randomly and uniformly distributed over the area | Aligned with reference | Aligned with reference |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction | Aligned with reference | Aligned with reference |
| UE speeds of interest | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h; 500 km/h for evaluation of mobility in high-speed case | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h; | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h; |
| Inter-site interference modeling | Explicitly modelled | Explicitly modelled | Explicitly modelled |
| BS noise figure | 5 dB | 5 dB | 5 dB |
| UE noise figure | 7 dB | 7 dB | 7 dB |
| BS antenna element gain | 8 dBi | 8 dBi | 8 dBi |
| UE antenna element gain | 0 dBi | 0 dBi | 0 dBi |
| Thermal noise level | ‒174 dBm/Hz | ‒174 dBm/Hz | ‒174 dBm/Hz |
| Traffic model | Full buffer | Full buffer | Full buffer |
| Simulation bandwidth | 20 MHz for TDD, 10 MHz+10 MHz for FDD | 10 MHz+10 MHz for FDD | 10 MHz+10 MHz for FDD |
| UE density | 10 UEs per TRxP Randomly and uniformly distributed over the area | Aligned with reference | Aligned with reference |
| UE antenna height | 1.5 m | 1.5 m | 1.5 m |
| Channdel Model | RMa_A, RMa_B | RMa_A | RMa_A |
| Additional parameters |  |  |  |
| Subcarrier spacing |  | 15 kHz | 15 kHz |
| Symbols number per slot |  | 14 | 14 |
| Number of TXRU per TRxP |  | 8TXRU (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) Vertical 1-to-8 | 8TXRU (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) Vertical 1-to-8 |
| TRxP number per site |  | 3 | 3 |
| Number of TXRU per UE |  | 2TXRU, (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) (1-to-1 mapping) | 2TXRU, (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) (1-to-1 mapping) |
| Mechanic tilt |  | 90deg in GCS (pointing to the horizontal direction) | 90deg in GCS (pointing to the horizontal direction) |
| Electronic tilt |  | 100deg in LCS | 100deg in LCS |
| Scheduling |  | MU-PF | Round Robin |
| ACK/NACK delay |  | Next available UL slot | Next available UL slot |
| MIMO mode |  | MU-MIMO with rank 2/4 adaptation per user | MU-MIMO |
| Guard band ratio |  | FDD: 6.4% (for 10 MHz) | FDD: 6.4% (for 10 MHz) |
| BS receiver type |  | MMSE-IRC | MMSE-IRC |
| Precoder derivation |  | FDD: NR Type II codebook based | FDD: NR Type II codebook based |
| Channel estimation |  | Non-ideal | Non-ideal |
| Waveform |  | OFDM | OFDM |
| Polarized antenna model |  | Model-2 (TR36.873) | Model-2 (TR36.873) |
| Modulation |  | Up to 256QAM | Up to 256QAM |

Table 14. Assumptions and Configuration of Rural-eMBB (Uplink Case) – Configuration A

| Configuration A - Uplink |  |  |  |
| --- | --- | --- | --- |
|  | ITU-R M.2412 | MEDIATEK | INRS-EMT |
| Baseline configuration |  |  |  |
| Carrier frequency for evaluation | 700 MHz | 700 MHz | 700 MHz |
| BS antenna height | 35 m | 35 m | 35 m |
| Total transmit power per TRxP | 49 dBm for 20 MHz bandwidth 46 dBm for 10 MHz bandwidth | 46 dBm for 10 MHz bandwidth | 46 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type | 100% low loss | 100% low loss | 100% low loss |
| Inter-site distance | 1732 m | 1732 m | 1732 m |
| Number of antenna elements per TRxP | Up to 256 Tx/Rx | 64Rx cross-polarized antenna (M,N,P,Mg,Ng) = (8,4,2,1,1) (dH,dV) = (0.5, 0.8)λ | 64Rx cross-polarized antenna (M,N,P,Mg,Ng) = (8,4,2,1,1) (dH,dV) = (0.5, 0.8)λ |
| Number of UE antenna elements | Up to 8 Tx/Rx | 1Tx (M,N,P,Mg,Ng) = (1,1,1,1,1) 0°,90° polarization | 1Tx (M,N,P,Mg,Ng) = (1,1,1,1,1) 0°,90° polarization |
| Device deployment | 100% indoor Randomly and uniformly distributed over the area | 50% indoor, 50% outdoor (in car) Randomly and uniformly distributed over the area | 50% indoor, 50% outdoor (in car) Randomly and uniformly distributed over the area |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction | Aligned with reference | Aligned with reference |
| UE speeds of interest | 100% indoor, 3 km/h | Aligned with reference | Aligned with reference |
| Inter-site interference modeling | Explicitly modelled | Explicitly modelled | Explicitly modelled |
| BS noise figure | 5 dB | 5 dB | 5 dB |
| UE noise figure | 7 dB | 7 dB | 7 dB |
| BS antenna element gain | 8 dBi | 8 dBi | 8 dBi |
| UE antenna element gain | 0 dBi | 0 dBi | 0 dBi |
| Thermal noise level | ‒174 dBm/Hz | ‒174 dBm/Hz | ‒174 dBm/Hz |
| Traffic model | Full buffer | Full buffer | Full buffer |
| Simulation bandwidth | 20 MHz for TDD, 10 MHz+10 MHz for FDD | 10 MHz+10 MHz for FDD | 10 MHz+10 MHz for FDD |
| UE density | 10 UEs per TRxP randomly and uniformly dropped throughout the geographical area | Aligned with reference | Aligned with reference |
| UE antenna height | 1.5 m | 1.5 m | 1.5 m |
| Channdel Model | UMa_A, UMa_B | UMa_A | UMa_A |
| Additional parameters |  |  |  |
| Subcarrier spacing |  | 15 kHz | 15 kHz |
| Symbols number per slot |  | 14 | 14 |
| Number of TXRU per TRxP |  | 8TXRU (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) Vertical 1-to-8 | 8TXRU (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) Vertical 1-to-8 |
| TRxP number per site |  | 3 | 3 |
| Number of TXRU per UE |  | 1TXRU, (Mp,Np,P,Mg,Ng) = (1,1,1,1,1) (1-to-1 mapping) | 1TXRU, (Mp,Np,P,Mg,Ng) = (1,1,1,1,1) (1-to-1 mapping) |
| Mechanic tilt |  | 90deg in GCS (pointing to the horizontal direction) | 90deg in GCS (pointing to the horizontal direction) |
| Electronic tilt |  | 100deg in LCS | 100deg in LCS |
| Scheduling |  | SU-PF | Round Robin |
| ACK/NACK delay |  | Next available UL slot | Next available UL slot |
| MIMO mode |  | SU-MIMO with rank 2/4 adaptation | SU-MIMO |
| BS receiver type |  | MMSE-IRC | MMSE-IRC |
| UE precoder scheme |  | Codebook based | Codebook based |
| UL CSI derivation |  | Non-precoded SRS based, with delay | Non-precoded SRS based, with delay |
| Power control |  | α= 0.8, P0 =-76 dBm | α= 0.8, P0 =-76 dBm |
| RB allocation for Power backoff model |  | Continuous: follow TS 38.101; Non-continuous: additional 2 RB reduction | Continuous: follow TS 38.101; Non-continuous: additional 2 RB reduction |
| Channel estimation |  | Non-ideal | Non-ideal |
| Waveform |  | OFDM | OFDM |
| Polarized antenna model |  | Model-2 (TR36.873) | Model-2 (TR36.873) |
| Modulation |  | Up to 256QAM | Up to 256QAM |

Table 15. Assumptions and Configuration of Rural-eMBB (Downlink Case) – Configuration B

| Configuration B - Downlink |  |  |  |
| --- | --- | --- | --- |
|  | ITU-R M.2412 | MEDIATEK | INRS-EMT |
| Baseline configuration |  |  |  |
| Carrier frequency for evaluation | 700 MHz | 4 GHz | 4 GHz |
| BS antenna height | 35 m | 35 m | 35 m |
| Total transmit power per TRxP | 49 dBm for 20 MHz bandwidth 46 dBm for 10 MHz bandwidth | 46 dBm for 10 MHz bandwidth | 46 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type | 100% low loss | 100% low loss | 100% low loss |
| Inter-site distance | 1732 m | 1732 m | 1732 m |
| Number of antenna elements per TRxP | Up to 64 Tx/Rx | 128Tx cross-polarized antenna (M,N,P,Mg,Ng) = (8,8,2,1,1) (dH,dV) = (0.5, 0.8)λ | 128Tx cross-polarized antenna (M,N,P,Mg,Ng) = (8,8,2,1,1) (dH,dV) = (0.5, 0.8)λ |
| Number of UE antenna elements | Up to 4 Tx/Rx | 4Rx (M,N,P,Mg,Ng) = (1,2,2,1,1) 0°,90° polarization | 4Rx (M,N,P,Mg,Ng) = (1,2,2,1,1) 0°,90° polarization |
| Device deployment | 50% indoor, 50% outdoor (in-car) Randomly and uniformly distributed over the area | Aligned with reference | Aligned with reference |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction | Aligned with reference | Aligned with reference |
| UE speeds of interest | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h; 500 km/h for evaluation of mobility in high-speed case | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h; | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h; |
| Inter-site interference modeling | Explicitly modelled | Explicitly modelled | Explicitly modelled |
| BS noise figure | 5 dB | 5 dB | 5 dB |
| UE noise figure | 7 dB | 7 dB | 7 dB |
| BS antenna element gain | 8 dBi | 8 dBi | 8 dBi |
| UE antenna element gain | 0 dBi | 0 dBi | 0 dBi |
| Thermal noise level | ‒174 dBm/Hz | ‒174 dBm/Hz | ‒174 dBm/Hz |
| Traffic model | Full buffer | Full buffer | Full buffer |
| Simulation bandwidth | 20 MHz for TDD, 10 MHz+10 MHz for FDD | 10 MHz+10 MHz for FDD | 10 MHz+10 MHz for FDD |
| UE density | 10 UEs per TRxP Randomly and uniformly distributed over the area | Aligned with reference | Aligned with reference |
| UE antenna height | 1.5 m | 1.5 m | 1.5 m |
| Channdel Model | RMa_A, RMa_B | RMa_A | RMa_A |
| Additional parameters |  |  |  |
| Subcarrier spacing |  | 15 kHz | 15 kHz |
| Symbols number per slot |  | 14 | 14 |
| Number of TXRU per TRxP |  | 32TXRU, (Mp,Np,P,Mg,Ng) = (2,8,2,1,1) | 32TXRU, (Mp,Np,P,Mg,Ng) = (2,8,2,1,1) |
| TRxP number per site |  | 3 | 3 |
| Number of TXRU per UE |  | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) (1-to-1 mapping) | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) (1-to-1 mapping) |
| Mechanic tilt |  | 90deg in GCS (pointing to the horizontal direction) | 90deg in GCS (pointing to the horizontal direction) |
| UT attachment |  | Based on RSRP (Eq. (8.1-1) in TR 36.873) from port 0 | Based on RSRP (Eq. (8.1-1) in TR 36.873) from port 0 |
| Scheduling |  | MU-PF | Round Robin |
| ACK/NACK delay |  | Next available UL slot | Next available UL slot |
| MIMO mode |  | MU-MIMO with rank 2/4 adaptation per user | MU-MIMO |
| Guard band ratio |  | FDD: 6.4% (for 10 MHz) | FDD: 6.4% (for 10 MHz) |
| BS receiver type |  | MMSE-IRC | MMSE-IRC |
| Precoder derivation |  | FDD: NR Type II codebook based | FDD: NR Type II codebook based |
| Channel estimation |  | Non-ideal | Non-ideal |
| Waveform |  | OFDM | OFDM |
| Polarized antenna model |  | Model-2 (TR36.873) | Model-2 (TR36.873) |
| Modulation |  | Up to 256QAM | Up to 256QAM |

Table 16. Assumptions and Configuration of Rural-eMBB (Uplink Case) – Configuration B

| Configuration B - Uplink |  |  |  |
| --- | --- | --- | --- |
|  | ITU-R M.2412 | MEDIATEK | ITRI |
| Baseline configuration |  |  |  |
| Carrier frequency for evaluation | 700 MHz | 4 GHz | 4 GHz |
| BS antenna height | 35 m | 35 m | 35 m |
| Total transmit power per TRxP | 49 dBm for 20 MHz bandwidth 46 dBm for 10 MHz bandwidth | 46 dBm for 10 MHz bandwidth | 46 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type | 100% low loss | 100% low loss | 100% low loss |
| Inter-site distance | 1732 m | 1732 m | 1732 m |
| Number of antenna elements per TRxP | Up to 256 Tx/Rx | 128Rx cross-polarized antenna (M,N,P,Mg,Ng) = (8,8,2,1,1) (dH,dV) = (0.5, 0.8)λ | 128Rx cross-polarized antenna (M,N,P,Mg,Ng) = (8,8,2,1,1) (dH,dV) = (0.5, 0.8)λ |
| Number of UE antenna elements | Up to 8 Tx/Rx | 1Tx (M,N,P,Mg,Ng) = (1,1,1,1,1) 0°,90° polarization | 1Tx (M,N,P,Mg,Ng) = (1,1,1,1,1) 0°,90° polarization |
| Device deployment | 50% indoor, 50% outdoor (in car) Randomly and uniformly distributed over the area | Aligned with reference | Aligned with reference |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction | Aligned with reference | Aligned with reference |
| UE speeds of interest | 100% indoor, 3 km/h | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h; | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h; |
| Inter-site interference modeling | Explicitly modelled | Explicitly modelled | Explicitly modelled |
| BS noise figure | 5 dB | 5 dB | 5 dB |
| UE noise figure | 7 dB | 7 dB | 7 dB |
| BS antenna element gain | 8 dBi | 8 dBi | 8 dBi |
| UE antenna element gain | 0 dBi | 0 dBi | 0 dBi |
| Thermal noise level | ‒174 dBm/Hz | ‒174 dBm/Hz | ‒174 dBm/Hz |
| Traffic model | Full buffer | Full buffer | Full buffer |
| Simulation bandwidth | 20 MHz for TDD, 10 MHz+10 MHz for FDD | 10 MHz+10 MHz for FDD | 10 MHz+10 MHz for FDD |
| UE density | 10 UEs per TRxP randomly and uniformly dropped throughout the geographical area | Aligned with reference | Aligned with reference |
| UE antenna height | 1.5 m | 1.5 m | 1.5 m |
| Channdel Model | UMa_A, UMa_B | UMa_A | UMa_A |
| Additional parameters |  |  |  |
| Subcarrier spacing |  | 15 kHz | 15 kHz |
| Symbols number per slot |  | 14 | 14 |
| Number of TXRU per TRxP |  | 32TXRU (Mp,Np,P,Mg,Ng) = (2,8,2,1,1) Vertical 1-to-4 | 32TXRU (Mp,Np,P,Mg,Ng) = (2,8,2,1,1) Vertical 1-to-4 |
| TRxP number per site |  | 3 | 3 |
| Number of TXRU per UE |  | 1TXRU, (Mp,Np,P,Mg,Ng) = (1,1,1,1,1) (1-to-1 mapping) | 1TXRU, (Mp,Np,P,Mg,Ng) = (1,1,1,1,1) (1-to-1 mapping) |
| Mechanic tilt |  | 90deg in GCS (pointing to the horizontal direction) | 90deg in GCS (pointing to the horizontal direction) |
| Electronic tilt |  | 100deg in LCS | 100deg in LCS |
| Scheduling |  | SU-PF | Round Robin |
| ACK/NACK delay |  | Next available UL slot | Next available UL slot |
| MIMO mode |  | SU-MIMO with rank 2/4 adaptation | SU-MIMO |
| BS receiver type |  | MMSE-IRC | MMSE-IRC |
| UE precoder scheme |  | Codebook based | Codebook based |
| UL CSI derivation |  | Non-precoded SRS based, with delay | Non-precoded SRS based, with delay |
| Power control |  | α= 0.6, P0 =-70 dBm | α= 0.6, P0 =-70 dBm |
| RB allocation for Power backoff model |  | Continuous: follow TS 38.101; Non-continuous: additional 2 RB reduction | Continuous: follow TS 38.101; Non-continuous: additional 2 RB reduction |
| Channel estimation |  | Non-ideal | Non-ideal |
| Waveform |  | OFDM | OFDM |
| Polarized antenna model |  | Model-2 (TR36.873) | Model-2 (TR36.873) |
| Modulation |  | Up to 256QAM | Up to 256QAM |

- Candidates configuration comparison

As it can be seen in Table.16, 3GPP, China, and TSDSI provide the same guidelines, and thus, our results can be generalized for the three candidates. On the other hand, Korea and ETSI have different requirements that need to be taken into account in the next report.

Table 17. Assumptions and Configuration of the proposed candidates

|  | 3GPP (radio interface technologies: RIT) | Korea | China | ETSI | TSDSI |
| --- | --- | --- | --- | --- | --- |
| Test environment | all the five test environments | It addresses two test environments under eMBB (Indoor and Dense Urban-) and Urban under URLLC. | All five environment | Indoor Hotspot-eMBB, Urban Macro–mMTC and Urban Macro–URLLC. Additional test environments may be added. | All five environment |
| Multiple access schemes | - DL/UL OFDMA: - Synchronous/scheduling-based - Mutually orthogonal frequency assignments for UE transmission. - One RB consist of 12 subcarriers. Multiple sub-carrier spacings (15kHz, 30kHz, 60kHz and 120kHz) are supported including for data - CP-OFDM for DL/UL and possibly DFT-spread in UL - Spectral confinement technique is transparent to the receiver. TDMA: - One slot consisting of (2,4,7 or 14) OFDM symbols. - The physical length of one slot [0.125 , 1] ms depending on the sub-carrier spacing (for more details on the frame structure, see Item 5.2.3.2.7 and the references therein). CDMA: • Inter-cell interference suppressed by processing gain of channel coding (for more details on channel-coding, see Item 5.2.3.2.2.3 and the reference therein). SDMA: • Possibility to transmit to/from multiple users using the same time/frequency resource (SDMA a.k.a. “multi-user MIMO (for more details on the advanced antenna capabilities, see Item 5.2.3.2.9 and the reference therein) • UL transmission scheme without scheduling grant is supported. •The above scheme is applied to eMBB and URLLC. | OFDMA: - Mutually orthogonal frequency assignments for UE transmission. - Multiple OFDM numerologies are supported. - One RB consist of 12 subcarriers. - DFT-spread used in UL. TDMA: - One slot duration consisting of 14 OFDM symbols. - less than slot duration with 2, 4, 7 symbol-duration for non-slot based operation. CDMA: Inter-cell interference suppressed by processing gain of channel coding SDMA: SDMA a.k.a. “multi-user MIMO | - DL/UL OFDMA: - Synchronous/scheduling-based - Mutually orthogonal frequency assignments for UE transmission. - One RB consist of 12 subcarriers. Multiple sub-carrier spacings (15kHz, 30kHz, 60kHz and 120kHz) are supported including for data - CP-OFDM for DL/UL and possibly DFT-spread in UL - Spectral confinement technique is transparent to the receiver. TDMA: - One slot consisting of (2,4,7 or 14) OFDM symbols. - The physical length of one slot [0.125 , 1] ms depending on the sub-carrier spacing (for more details on the frame structure, see Item 5.2.3.2.7 and the references therein). CDMA: Inter-cell interference suppressed by processing gain of channel coding SDMA: SDMA a.k.a. “multi-user MIMO • UL transmission scheme without scheduling grant is supported. •The above scheme is applied to eMBB and URLLC. | TDMA and FDMA - simultaneously use multiple frequency sub-channels - Within, UL and DL may occur. - Time overlapping transmit/receive is not supported - An FP device can support simultaneous links to multiple PP devices. - Nominal sub-carrier spacing is 27 kHz with a nominal time slot duration of 416.67μs. - Additional sub-carrier spacings up to 216 kHz are supported depending on deployment. | - DL/UL OFDMA: - Synchronous/scheduling-based - Mutually orthogonal frequency assignments for UE transmission. - One RB consist of 12 subcarriers. Multiple sub-carrier spacings (15kHz, 30kHz, 60kHz and 120kHz) are supported including for data - CP-OFDM for DL/UL and possibly DFT-spread in UL - Spectral confinement technique is transparent to the receiver. TDMA: - One slot consisting of (2,4,7 or 14) OFDM symbols. - The physical length of one slot [0.125 , 1] ms depending on the sub-carrier spacing (for more details on the frame structure, see Item 5.2.3.2.7 and the references therein). CDMA: Inter-cell interference suppressed by processing gain of channel coding SDMA: SDMA a.k.a. “multi-user MIMO • UL transmission scheme without scheduling grant is supported. •The above scheme is applied to eMBB and URLLC. -+ |
| baseband modulation scheme and symbol rate after modulation | DL - For data and higher-layer control information: QPSK, 16QAM, 64QAM and 256QAM - L1/L2 control: QPSK - Symbol rate: 1344ksymbols/s per 1440kHz RB - UL - For both data and higher-layer control information: π/2-BPSK (when precoding is enabled), QPSK, 16QAM, 64QAM and 256QAM - L1/L2 control: BPSK, π/2-BPSK, QPSK - Symbol rate: 1344ksymbols/s per 1440kHz RB - The above is at least applied to eMBB. | DL - For data and higher-layer control information: QPSK, 16QAM, 64QAM and 256QAM - L1/L2 control: QPSK for PDCCH - Symbol rate: 168 symbols per RB per slot - UL - For both data and higher-layer control information 𝜋⁄2-BPSK (uplink only when transform precoding enabled) ), QPSK, 16QAM, 64QAM and 256QAM - L1/L2 control: : BPSK and QPSK for PUCCH - Symbol rate: 168 symbols per RB per slot symbol rate per resource block depends on sub-carrier spacing | DL - For data and higher-layer control information: QPSK, 16QAM, 64QAM and 256QAM - L1/L2 control: QPSK - Symbol rate: 1344ksymbols/s per 1440kHz RB - UL - For both data and higher-layer control information: π/2-BPSK (when precoding is enabled), QPSK, 16QAM, 64QAM and 256QAM - L1/L2 control: BPSK, π/2-BPSK, QPSK - Symbol rate: 1344ksymbols/s per 1440kHz RB The above is at least applied to eMBB. | For data and control OFDM with 24 Khz nominal symbol rate used, we have one of the following configuration: (modulation, code rate, Allowed constellation error) - (BPSK,1/4,-4) - (BPSK,1/2,-5) - (QPSK,1/2,-10) - (QPSK,3/4,-13) - (16QAM,1/2,-16) - (16QAM,3/4,-19) - (64QAM ,2/3,-22) - (64QAM ,3/4,-25) - (64QAM ,5/6,-27) - (256QAM,3/4,-30) - (256QAM,5/6,-32) - (1024QAM ,3/4,-35) - (1024QAM ,5/6,-37) | DL - For data and higher-layer control information: QPSK, 16QAM, 64QAM and 256QAM - L1/L2 control: QPSK - Symbol rate: 1344ksymbols/s per 1440kHz RB - UL - For both data and higher-layer control information: π/2-BPSK (when precoding is enabled), QPSK, 16QAM, 64QAM and 256QAM - L1/L2 control: BPSK, π/2-BPSK with spectrum shaping, QPSK - Symbol rate: 1344ksymbols/s per 1440kHz RB The above is at least applied to eMBB. |
| average power ratio after baseband filtering (dB) | The PAPR depends on the waveform and the number of component carriers. The single component carrier transmission is assumed herein when providing the PAPR. For DFT-spread OFDM, PAPR would depend on modulation scheme as well. For uplink using DFT-spread OFDM, the cubic metric (CM) can also be used as one of the methods of predicting the power de-rating from signal modulation characteristics, if needed. DL - PAPR is 8.4dB (99.9%) - UL - For CP-OFDM PAPR is 8.4dB (99.9%) - DFT-spread PAPR (99.9%) is: 4.5 db for π/2-BPSK 5.8 dB for QPSK 6.5 dB for 16QAM 6.6 dB for 64QAM 6.7 dB for 256QAM CM (99.9%) is: 0.3 dB for π/2-BPSK 1.2 dB for QPSK 2.1 dB for 16QAM 2.3 dB for 64QAM 2.4 dB for 256QAM | UL/DL OFDMA - PAPR is 8.4dB (99.9%) UL when DFT-OFDM is used PAPR and Cubic Metric (CM) depends on modulation scheme and number of component carrier. The CM is a method of predicting the power de-rating from signal modulation. PAPR (99.9%) is: 4.5 db for π/2-BPSK 5.8 dB for QPSK 6.5 dB for 16QAM 6.6 dB for 64QAM 6.7 dB for 256QAM CM (99.9%) is: 0.3 dB for π/2-BPSK 1.2 dB for QBSK 2.1 dB for 16QAM 2.3 dB for 64QAM 2.4 dB for 256QAM | The PAPR depends on the waveform and the number of component carriers. The single component carrier transmission is assumed herein when providing the PAPR. For DFT-spread OFDM, PAPR would depend on modulation scheme as well. For uplink using DFT-spread OFDM, the cubic metric (CM) can also be used as one of the methods of predicting the power de-rating from signal modulation characteristics, if needed. DL - PAPR is 8.4dB (99.9%) - UL - For CP-OFDM PAPR is 8.4dB (99.9%) - DFT-spread PAPR (99.9%) is: 4.5 db for π/2-BPSK 5.8 dB for QPSK 6.5 dB for 16QAM 6.6 dB for 64QAM 6.7 dB for 256QAM CM (99.9%) is: 0.3 dB for π/2-BPSK 1.2 dB for QPSK 2.1 dB for 16QAM 2.3 dB for 64QAM 2.4 dB for 256QAM | Expected PAPR range is [7, 12] dB. More details will be provided in later update. | The PAPR depends on the waveform and the number of component carriers. The single component carrier transmission is assumed herein when providing the PAPR. For DFT-spread OFDM, PAPR would depend on modulation scheme as well. For uplink using DFT-spread OFDM, the cubic metric (CM) can also be used as one of the methods of predicting the power de-rating from signal modulation characteristics, if needed. DL - PAPR is 8.4dB (99.9%) - UL - For CP-OFDM PAPR is 8.4dB (99.9%) - DFT-spread PAPR (99.9%) is: 4.5 db for π/2-BPSK 5.8 dB for QPSK 6.5 dB for 16QAM 6.6 dB for 64QAM 6.7 dB for 256QAM CM (99.9%) is: 0.3 dB for π/2-BPSK 1.2 dB for QPSK 2.1 dB for 16QAM 2.3 dB for 64QAM 2.4 dB for 256QAM |
| physical channel bit rate (M or Gbit/s) for supported bandwidths | The physical channel bit rate depends on the modulation scheme, number of spatial-multiplexing layer, number of RB in the channel bandwidth and the subcarrier spacing used. The physical channel bit rate per layer: Rlayer=Nmod*NRB*2μ*168 kps Where: Nmod is the number of bits per modulation symbol : 2 dB for QPSK 4 dB for 16QAM 6 dB for 64QAM 8 dB for 256QAM μ depends on the subcarrier spacing, Δ𝑓, given by Δf=2μ*15 kHz, μ = 0,1,…3 | The physical channel bit rate depends on the modulation scheme and number of spatial-multiplexing layers | The physical channel bit rate depends on the modulation scheme, number of spatial-multiplexing layer, number of RB in the channel bandwidth and the subcarrier spacing used. The physical channel bit rate per layer: Rlayer=Nmod*NRB*2μ*168 kps Where: Nmod is the number of bits per modulation symbol : 2 dB for QPSK 4 dB for 16QAM 6 dB for 64QAM 8 dB for 256QAM μ depends on the subcarrier spacing, Δ𝑓, given by Δf=2μ*15 kHz, μ = 0,1,…3 | physical channel bit rate (Mbps) depends on the Bandwidth (MHz), occupied bandwidth (MHz), subcarrier spacing (KHz): (BW,oc BW, Δ𝑓)=BR (0.864,0.648,27)= 28.8 (1.728,1.512,27)=74.88 (3.456,3.131,27)=155.52 (6.912,6.588,27)=336.96 (13.824,13.5,27)=673.92 (20.736,17.928,27)=915.84 (27.648,27,27)=1347.84 (55.296,52.704,216)=2695.68 (110.592,108,216)=5391.36 (165.888,146.424,216)=7326.72 (221.184,216,216)=10782.72 | The physical channel bit rate depends on the modulation scheme, number of spatial-multiplexing layer, number of RB in the channel bandwidth and the subcarrier spacing used. The physical channel bit rate per layer: Rlayer=Nmod*NRB*2μ*168 kps Where: Nmod is the number of bits per modulation symbol : 2 dB for QPSK 4 dB for 16QAM 6 dB for 64QAM 8 dB for 256QAM μ depends on the subcarrier spacing, Δ𝑓, given by Δf=2μ*15 kHz, μ = 0,1,…3 |
| Applications and services support with various bit rate requirements. | - For a given combination of modulation scheme, code rate, and number of spatial-multiplexing layers, the data rate available to a user can be controlled through assigning different number of RB for the transmission. - For multiple services, the available/assigned resource and ,thus, the available data rate is shared between the rvices. | - For a given combination of modulation scheme, code rate, and number of spatial-multiplexing layers, the data rate available to a user can be controlled through assigning different number of RB for the transmission. - For multiple services, the available/assigned resource and ,thus, the available data rate is shared between the services. - The 5G QoS model supports both (GBR QoS Flows) and (non-GBR QoS Flows). | - For a given combination of modulation scheme, code rate, and number of spatial-multiplexing layers, the data rate available to a user can be controlled through assigning different number of RB for the transmission. - For multiple services, the available/assigned resource and ,thus, the available data rate is shared between the services. | - For the nominal Δf=27 KHz ,the proposed RIT layer 1 supports 145 different data rates in [120 Kbps – 1.1 Gbps]. Lower data rates are associated with more robust transmission characteristics. Higher data rates require more favourable link conditions and tighter system specifications. - For the Δf=216 KHz data rates are up to 8.6 Gbps. | - For a given combination of modulation scheme, code rate, and number of spatial-multiplexing layers, the data rate available to a user can be controlled through assigning different number of RB for the transmission. - For multiple services, the available/assigned resource and ,thus, the available data rate is shared between the services. |
| Handover | - Inter-System HO: - supported between 5G Core Network and EPC. - HO between NR in 5GC and E-UTRA in EPC is supported via inter-RAT HO. - HO between E-UTRA in 5GC and E-UTRA in EPC is supported via intra- E-UTRA HO with change of CN type. - The source eNB/ng-eNB decides HO procedure to trigger. -UE has to know the target CN type from the HO command during intra-LTE inter-System HO, intra-LTE intra-System HO. - Intra-System HO: - Intra-NR HO: network controlled mobility applies to UE in RRC_Connected and is categorized in 2 types: - Cell level mobility: requires explicit RRC signaling to be triggered. - Beam level mobility: doesn’t require RRC signaling to be triggered (dealt at lower layers), RRC not required to know which beam is being used. Data forwarding, in-sequence delivery and duplication avoidance at handover can be guaranteed between target gNB and source gNB. - Intra-RAT HO: - Intra 5GC inter RAT mobility is supported between NR and E-UTRA. - Inter RAT measurements in NR are limited to E-UTRA and the source RAT should be able to support and configure Target RAT measurement and reporting. - The in-sequence and lossless handover is supported for the handover between gNB and ng-eNB. - Both Xn and NG based inter-RAT handover between NG-RAN nodes is supported. Whether the handover is over Xn or CN is transparent to the UE. -The target RAT receives the UE NG-C context information and based on this information configures the UE with a complete RRC message and Full configuration . - Measurement In RRC_CONNECTED, the UE measures multiple beams (N best beams above an absolute threshold) of a cell and average it to derive cell quality. Filtering takes place at two different levels: at the physical layer to derive beam quality and then at RRC level to derive cell quality from multiple beams. Cell quality from beam measurements is derived in the same way for the serving cell(s) and for the non-serving cell(s). Measurement reports may contain the measurement results of the X best beams if the UE is configured to do so by the gNB | - Network controlled handover mechanism is used for connected mode UEs when performing Inter-System and Intra-System handover. - Source base station initiates handover and target base station performs admission control. UE receives a handover command which is provided by source base station and moves to the target base station. - Handover mechanism uses intra-frequency, inter-frequency, and inter-RIT measurements which are performed by UEs. - UE measures reference signal strength and quality and reports them to base station for handover decision. - Time gap for measurements is needed for inter-frequency measurement - For Intra-System handover, the mechanisms are the same for handover within each RIT and between the two RITs. | - Inter-System HO: - supported between 5G Core Network and EPC. - HO between NR in 5GC and E-UTRA in EPC is supported via inter-RAT HO. - HO between E-UTRA in 5GC and E-UTRA in EPC is supported via intra- E-UTRA HO with change of CN type. - The source eNB/ng-eNB decides HO procedure to trigger. -UE has to know the target CN type from the HO command during intra-LTE inter-System HO, intra-LTE intra-System HO. - Intra-System HO: - Intra-NR HO: network controlled mobility applies to UE in RRC_Connected and is categorized in 2 types: - Cell level mobility: requires explicit RRC signaling to be triggered. - Beam level mobility: doesn’t require RRC signaling to be triggered (dealt at lower layers), RRC not required to know which beam is being used. Data forwarding, in-sequence delivery and duplication avoidance at handover can be guaranteed between target gNB and source gNB. - Intra-RAT HO: - Intra 5GC inter RAT mobility is supported between NR and E-UTRA. - Inter RAT measurements in NR are limited to E-UTRA and the source RAT should be able to support and configure Target RAT measurement and reporting. - The in-sequence and lossless handover is supported for the handover between gNB and ng-eNB. - Both Xn and NG based inter-RAT handover between NG-RAN nodes is supported. Whether the handover is over Xn or CN is transparent to the UE. -The target RAT receives the UE NG-C context information and based on this information configures the UE with a complete RRC message and Full configuration . - Measurement In RRC_CONNECTED, the UE measures multiple beams (N best beams above an absolute threshold) of a cell and average it to derive cell quality. Filtering takes place at two different levels: at the physical layer to derive beam quality and then at RRC level to derive cell quality from multiple beams. Cell quality from beam measurements is derived in the same way for the serving cell(s) and for the non-serving cell(s). Measurement reports may contain the measurement results of the X best beams if the UE is configured to do so by the gNB | - Intra-System handover - Intra-System handover may be intra-cell or inter cell. - Intra-cell handover may be controlled by either the PP or the FP and triggered when quality on allocated carrier-slot-combinations becomes poor and other free carrier-slot-combinations exist. - Detection of free carrier-slot-combinations is based on a spectrum sensing paradigm and takes into account the activity of other uncoordinated systems. Seamless handover is supported. The PP sends a handover-request to the FP on the selected random access channel. If the FP accepts the request, then it indicates the position of the new traffic channel and the data will be switched over. After that the old channel will be released. Inter-cell handover is generally controlled by the PP and triggered when quality on allocated carrier-slot combinations becomes poor and another suitable FP is becoming stronger. Seamless handover is supported. The PP sends a handover-request to the new FP on the selected random access channel. If the FP accepts the request, then it indicates the position of the new traffic channel and the data will be switched over. After that the old channel will be released. - Inter-System handover (for proposed IMT-2020 RIT): Inter-System handover is performed in the same way as inter-cell handover. Seamless handover is supported. Both systems should be interconnected by the proper network infrastructure. - Inter-System handover to other IMT systems (other than IMT-2020) Inter-System handover to IMT-2000 TDMA FDMA (DECT) is supported. Handover to IMT-2000 TDMA FDMA is controlled by the PP and triggered when another suitable FP becomes stronger. Seamless handover is supported. The PP sends a handover-request to the FP on the selected channel. After the new connection is confirmed by the FP, the data is switched over to the new connection and the old one is released. Both systems should be interconnected by the proper network infrastructure. | - Inter-System HO: - supported between 5G Core Network and EPC. - HO between NR in 5GC and E-UTRA in EPC is supported via inter-RAT HO. - HO between E-UTRA in 5GC and E-UTRA in EPC is supported via intra- E-UTRA HO with change of CN type. - The source eNB/ng-eNB decides HO procedure to trigger. -UE has to know the target CN type from the HO command during intra-LTE inter-System HO, intra-LTE intra-System HO. - Intra-System HO: - Intra-NR HO: network controlled mobility applies to UE in RRC_Connected and is categorized in 2 types: - Cell level mobility: requires explicit RRC signaling to be triggered. - Beam level mobility: doesn’t require RRC signaling to be triggered (dealt at lower layers), RRC not required to know which beam is being used. Data forwarding, in-sequence delivery and duplication avoidance at handover can be guaranteed between target gNB and source gNB. - Intra-RAT HO: - Intra 5GC inter RAT mobility is supported between NR and E-UTRA. - Inter RAT measurements in NR are limited to E-UTRA and the source RAT should be able to support and configure Target RAT measurement and reporting. - The in-sequence and lossless handover is supported for the handover between gNB and ng-eNB. - Both Xn and NG based inter-RAT handover between NG-RAN nodes is supported. Whether the handover is over Xn or CN is transparent to the UE. -The target RAT receives the UE NG-C context information and based on this information configures the UE with a complete RRC message and Full configuration . - Measurement In RRC_CONNECTED, the UE measures multiple beams (N best beams above an absolute threshold) of a cell and average it to derive cell quality. Filtering takes place at two different levels: at the physical layer to derive beam quality and then at RRC level to derive cell quality from multiple beams. Cell quality from beam measurements is derived in the same way for the serving cell(s) and for the non-serving cell(s). Measurement reports may contain the measurement results of the X best beams if the UE is configured to do so by the gNB |
| Radio resource management | NR performs radio resource management. RRM functions include: - Radio bearer control (RBC): the establishment, maintenance and release of radio bearer involves the configuration of radio resource. This is located in gNB/ng-eNB. - Radio Admission Control (RAC): RAC is to admit or reject the establishment of new radio bearer. It considers QoS requirement, the priority level, overall resource situation. This is located in gNB/ng-eNB. - Connection Mobility Control (CMC): it controls the number of UEs in idle mode and connected mode. In idle mode, cell reselection algorithm is controlled by parameter setting and in the connected mode, gNB controls UE mobility via handover and RRC connection release with redirection. Dynamic/flexible radio resource management NR supports dynamic and flexible radio resource management by packet scheduling that allocates and de-allocates resources to user and control plane packets. Load balancing(LB) Load balancing has the task to handle uneven distribution of the traffic load over multiple cells. The purpose of LB is thus to influence the load distribution for the higher resource utilization and QoS. LB is achieved in NR with hand-over, redirection or cell reselection. | - RRM functions are located in base station the functions include: radio bearer control (RBC), radio admission control (RAC), and radio resource allocation. - RRM performs packet scheduling to dynamically allocate radio resources to UEs. The scheduling is based on QoS and channel quality information - The periodicity of scheduling is one or multiple slots for slot-based scheduling and less than one slot for non-slot based scheduling. - In addition, RRM functions can flexibly configure/ modify/ release radio bearers and resources to support various service scenarios. - RRM provides load balancing between cells with handover, cell reselection mechanisms to efficiently distribute UEs and traffic among cells. - Inter-RIT interworking Describe the functional blocks and mechanisms for interworking between component RITs within a SRIT, if supported. - Tight Inter-RIT interworking is supported and RIT network interface supports the transparent transport of signaling for Inter-RIT interworking such as handover signaling, capability coordination. | NR performs radio resource management. RRM functions include: - Radio bearer control (RBC): the establishment, maintenance and release of radio bearer involves the configuration of radio resource. This is located in gNB/ng-eNB. - Radio Admission Control (RAC): RAC is to admit or reject the establishment of new radio bearer. It considers QoS requirement, the priority level, overall resource situation. This is located in gNB/ng-eNB. - Connection Mobility Control (CMC): it controls the number of UEs in idle mode and connected mode. In idle mode, cell reselection algorithm is controlled by parameter setting and in the connected mode, gNB controls UE mobility via handover and RRC connection release with redirection. Dynamic/flexible radio resource management NR supports dynamic and flexible radio resource management by packet scheduling that allocates and de-allocates resources to user and control plane packets. Load balancing(LB) Load balancing has the task to handle uneven distribution of the traffic load over multiple cells. The purpose of LB is thus to influence the load distribution for the higher resource utilization and QoS. LB is achieved in NR with hand-over, redirection or cell reselection. | - Radio resource management is based on the implementation of the cognitive radio, spectrum sensing paradigm and is able to take into account the activity of other systems –coordinated or uncoordinated operating in the same area. - Channels are automatically selected and allocated based on measurement of background RSSI. - The FP may also take into account carrier slot positions in order to allocate the most convenient resource blocks for system efficiency. | NR performs radio resource management. RRM functions include: - Radio bearer control (RBC): the establishment, maintenance and release of radio bearer involves the configuration of radio resource. This is located in gNB/ng-eNB. - Radio Admission Control (RAC): RAC is to admit or reject the establishment of new radio bearer. It considers QoS requirement, the priority level, overall resource situation. This is located in gNB/ng-eNB. - Connection Mobility Control (CMC): it controls the number of UEs in idle mode and connected mode. In idle mode, cell reselection algorithm is controlled by parameter setting and in the connected mode, gNB controls UE mobility via handover and RRC connection release with redirection. Dynamic/flexible radio resource management NR supports dynamic and flexible radio resource management by packet scheduling that allocates and de-allocates resources to user and control plane packets. Load balancing(LB) Load balancing has the task to handle uneven distribution of the traffic load over multiple cells. The purpose of LB is thus to influence the load distribution for the higher resource utilization and QoS. LB is achieved in NR with hand-over, redirection or cell reselection. |
| Frame structure | Frame length, sub-carrier spacing, and time slots: - One radio frame of length 10 ms consisting of 10 subframes, each of length 1 ms. - Each subframe consists of an OFDM sub-carrier spacing dependent number of slots. - Each slot consists of 14 OFDM symbols (twelve OFDM symbols in case of extended cyclic prefix) - 15 kHz SCS: 1 ms slot, 1 slot per sub-frame - 30 kHz SCS: 0.5 ms slot, 2 slots per sub-frame - 60 kHz SCS: 0.25 ms slot, 4 slots per sub-frame - 120 kHz SCS: 0.125 ms slot, 8 slots per sub-frame - 240 kHz SCS: 0.0625 ms slot (only used for synchronization, not for data) - Data transmissions can be scheduled on a slot basis, as well as on a partial slot basis, where the partial slot transmissions that may occur several times within one slot. The supported partial slot allocations and scheduling intervals are 2, 4 and 7 symbols for normal cyclic prefix, and 2, 4 and 6 symbols for extended cyclic prefix. - The slot structure supports zero, one or two DL/UL switches per slot, and dynamic selection of the link direction for each slot independently. Typically one symbol would be allocated as guard, but different number of symbols, or even full slot could be allocated as guard. Downlink control channel structure: Downlink control signaling is time and frequency multiplexed with data on a scheduling interval basis. The control region can span over 1-3 OFDM symbols in the beginning of the allocation, flexibly allocating 1-14 symbols for data transmission, including the time and frequency part of the control region that was not used for control signaling. Uplink control channel structure: Uplink control signaling can be both time-multiplexed with the data of the same UE and time and frequency multiplexed with control and data of other UEs when the UE has no data to be transmitted. Uplink control signaling is piggy-backed with data i.e. transmitted with data on the PUSCH when the UE has data to be transmitted. Power control bit rate: No specific power-control rate is defined, but a power control command can be sent at any slot, leading to a sub-carrier spacing specific maximum power control rate of 1/2/4/8 kHz for SCS of 15/30/60/120 kHz respectivel | One radio frame of length 10 ms consists of 10 subframes, each of length 1 ms. Each subframe consists of various number of slots depending on subcarrier spacing, respectively. Each slot consists of 14 OFDM symbols (12 OFDM symbols in case of extended cyclic prefix). In TDD, per each slot, up to 2 DL-to-UL switching points are allowed. One or more OFDM symbols serve as a guard time between DL and UL. Downlink control channel region is flexibly allocated in time and frequency. A downlink control region spans 1~3 OFDM symbols in time, and is localized or distributed in frequency. A downlink control channel can be time and/or frequency multiplexed with a downlink data channel on a slot or mini-slot basis. Uplink control channel region is flexibly allocated in time and frequency. An uplink control region spans 4~14 OFDM symbols for long duration formats and 1~2 OFDM symbols for short duration formats, and is localized in frequency. Intra-slot or interslot frequency hopping is supported. An uplink control channel can be time and/or frequency multiplexed with an uplink data channel on a slot or mini-slot basis. Uplink control information can be piggy-backed with data, i.e. transmitted with data on PUSCH when UE has uplink data to be transmitted. Up to one or multiple power control commands per slot are supported. The power control rate depends on the slot duration and the maximum allowed number of power control commands per slot. | - One radio frame of length 10 ms consisting of 10 subframes, each of length 1 ms. - Each subframe consists of an OFDM sub-carrier spacing dependent number of slots. - Each slot consists of 14 OFDM symbols (twelve OFDM symbols in case of extended cyclic prefix) - 15 kHz SCS: 1 ms slot, 1 slot per sub-frame - 30 kHz SCS: 0.5 ms slot, 2 slots per sub-frame - 60 kHz SCS: 0.25 ms slot, 4 slots per sub-frame - 120 kHz SCS: 0.125 ms slot, 8 slots per sub-frame - 240 kHz SCS: 0.0625 ms slot (only used for synchronization, not for data) - Data transmissions can be scheduled on a slot basis, as well as on a partial slot basis, where the partial slot transmissions that may occur several times within one slot. The supported partial slot allocations and scheduling intervals are 2, 4 and 7 symbols for normal cyclic prefix, and 2, 4 and 6 symbols for extended cyclic prefix. - The slot structure supports zero, one or two DL/UL switches per slot, and dynamic selection of the link direction for each slot independently. Typically one symbol would be allocated as guard, but different number of symbols, or even full slot could be allocated as guard. Downlink control channel structure: Downlink control signaling is time and frequency multiplexed with data on a scheduling interval basis. The control region can span over 1-3 OFDM symbols in the beginning of the allocation, flexibly allocating 1-14 symbols for data transmission, including the time and frequency part of the control region that was not used for control signaling. Uplink control channel structure: Uplink control signaling can be both time-multiplexed with the data of the same UE and time and frequency multiplexed with control and data of other UEs when the UE has no data to be transmitted. Uplink control signaling is piggy-backed with data i.e. transmitted with data on the PUSCH when the UE has data to be transmitted. Power control bit rate: No specific power-control rate is defined, but a power control command can be sent at any slot, leading to a sub-carrier spacing specific maximum power control rate of 1/2/4/8 kHz for SCS of 15/30/60/120 kHz respectively. | The basic frame structure consists of 24 time slots in 10 ms (for 27 kHz sub-carrier spacing). Half slots can be used for some services. Slots can be concatenated to form multi-slot transmissions. Any slot can be used for uplink or downlink transmission (i.e. there is no pre-set direction for the slot). When there is a change in the source of the transmission, then a guard space is used. Several transmissions to/from different PPs may occur simultaneously on non-overlapping channels. The transmissions are scheduled in such a way that none of the devices is required to transmit and receive simultaneously. For higher sub-carrier spacing the number of slots per frame can vary accordingly. The frame structure parameters for 27 kHz sub-carrier spacing are summarized in Table page 8. | The frame structure related information is as follows: ‐ Frame length, sub-carrier spacing, and time slots: One radio frame of length 10 ms consisting of 10 subframes, each of length 1 ms. Each subframe consists of an OFDM sub-carrier spacing dependent number of slots. Each slot consists of 14 OFDM symbols (twelve OFDM symbols in case of extended cyclic prefix) - 15 kHz SCS: 1 ms slot, 1 slot per sub-frame - 30 kHz SCS: 0.5 ms slot, 2 slots per sub-frame - 60 kHz SCS: 0.25 ms slot, 4 slots per sub-frame - 120 kHz SCS: 0.125 ms slot, 8 slots per sub-frame - 240 kHz SCS: 0.0625 ms slot (only used for synchronization, not for data) Data transmissions can be scheduled on a slot basis, as well as on a partial slot basis, where the partial slot transmissions that may occur several times within one slot. The supported partial slot allocations and scheduling intervals are 2, 4 and 7 symbols for normal cyclic prefix, and 2, 4 and 6 symbols for extended cyclic prefix. The slot structure supports zero, one or two DL/UL switches per slot, and dynamic selection of the link direction for each slot independently. Typically, one symbol would be allocated as guard, but different number of symbols, or even full slot could be allocated as guard. ‐ Downlink control channel structure: Downlink control signaling is time and frequency multiplexed with data on a scheduling interval basis. The control region can span over 1-3 OFDM symbols in the beginning of the allocation, flexibly allocating 1-14 symbols for data transmission, including the time and frequency part of the control region that was not used for control signaling. ‐ Uplink control channel structure: Uplink control signaling can be both time-multiplexed with the data of the same UE and time and frequency multiplexed with control and data of other UEs when the UE has no data to be transmitted. Uplink control signaling is piggy-backed with data i.e. transmitted with data on the PUSCH when the UE has data to be transmitted. ‐ Power control bit rate: No specific power-control rate is defined, but a power control command can be sent at any slot, leading to a sub-carrier spacing specific maximum power control rate of 1/2/4/8 kHz for SCS of 15/30/60/120 kHz respectively. |
| Frequency bands supported by the RIT/SRIT | - 450-6000Mhz: Band number n1: - UL band: 1920 – 1980 MHz - DL band: 2110 – 2170 MHz Band number n2: - UL band: 1850 – 1910 MHz - DL band: 1930 – 1990 MHz Band number n3: - UL band: 1710 – 1785 MHz - DL band: 1805 – 1880 MHz Band number n5: - UL band: 824 – 849 MHz - DL band: 869 – 894 MHz Band number n7: - UL band: 2500 – 2570 MHz - DL band: 2620 – 2690 MHz Band number n8: - UL band: 880 – 915 MHz - DL band: 925 – 960 MHz Band number n20: - UL band: 832 – 862 MHz - DL band: 791 – 821 MHz Band number n28: - UL band: 703 – 748 MHz - DL band: 758 – 803 MHz Band number n38: - UL band: 2570 – 2620 MHz - DL band: 2570 – 2620 MHz Band number n41: - UL band: 2496 – 2690 MHz - DL band: 2496 – 2690 MHz Band number n50: - UL band: 1432 – 1517 MHz - DL band: 1432 – 1517 MHz Band number n51: - UL band: 1427 – 1432 MHz - DL band: 1427 – 1432 MHz Band number n66: - UL band: 1710 – 1780 MHz - DL band: 2110 – 2200 MHz Band number n70: - UL band: 1695 – 1710 MHz - DL band: 1995 – 2020 MHz Band number n71: - UL band: 663 – 698 MHz - DL band: 617 – 652 MHz Band number n74: - UL band: 1427 – 1470 MHz - DL band: 1475 – 1518 MHz Band number n75: - UL band: NA - DL band: 1432 – 1517 MHz Band number n76: - UL band: NA - DL band: 1427 – 1432 MHz Band number n77: - UL band: 3.3 – 4.2 GHz - DL band: 3.3 – 4.2 GHz Band number n78: - UL band: 3.3 – 3.8 GHz - DL band: 3.3 – 3.8 GHz Band number n79: - UL band: 4.4 – 5 GHz - DL band: 4.4 – 5 GHz Band number n80: - UL band: 1710 – 1785 MHz - DL band: NA Band number n81: - UL band: 880 – 915 MHz - DL band: NA Band number n82: - UL band: 832 – 862 MHz - DL band: NA Band number n83: - UL band: 703 – 748 MHz - DL band: NA Band number n84: - UL band: 1920 – 1980 MHz - DL band: NA Band number n85: - UL band: 2496 – 2960 MHz - DL band: NA - 24250-52600Mhz: Band number n257: - UL band: 26.5 – 29.5 GHz - DL band: 26.5 – 29.5 GHz Band number n258: - UL band: 24.25 – 27.5 GHz - DL band: 24.25 – 27.5 GHz Band number n260: - UL band: 37 – 40 GHz - DL band: 37 – 40 GHz | - High frequency (above 6Ghz): Band number n257: - UL band: 26.5 – 29.5 GHz - DL band: 26.5 – 29.5 GHz Band number n258: - UL band: 24.25 – 27.5 GHz - DL band: 24.25 – 27.5 GHz - Low frequency (below 6Ghz): Band number n78: - UL band: 3.3 – 3.8 GHz - DL band: 3.3 – 3.8 GHz | - 450-6000Mhz: Band number n1: - UL band: 1920 – 1980 MHz - DL band: 2110 – 2170 MHz Band number n2: - UL band: 1850 – 1910 MHz - DL band: 1930 – 1990 MHz Band number n3: - UL band: 1710 – 1785 MHz - DL band: 1805 – 1880 MHz Band number n5: - UL band: 824 – 849 MHz - DL band: 869 – 894 MHz Band number n7: - UL band: 2500 – 2570 MHz - DL band: 2620 – 2690 MHz Band number n8: - UL band: 880 – 915 MHz - DL band: 925 – 960 MHz Band number n20: - UL band: 832 – 862 MHz - DL band: 791 – 821 MHz Band number n28: - UL band: 703 – 748 MHz - DL band: 758 – 803 MHz Band number n38: - UL band: 2570 – 2620 MHz - DL band: 2570 – 2620 MHz Band number n41: - UL band: 2496 – 2690 MHz - DL band: 2496 – 2690 MHz Band number n50: - UL band: 1432 – 1517 MHz - DL band: 1432 – 1517 MHz Band number n51: - UL band: 1427 – 1432 MHz - DL band: 1427 – 1432 MHz Band number n66: - UL band: 1710 – 1780 MHz - DL band: 2110 – 2200 MHz Band number n70: - UL band: 1695 – 1710 MHz - DL band: 1995 – 2020 MHz Band number n71: - UL band: 663 – 698 MHz - DL band: 617 – 652 MHz Band number n74: - UL band: 1427 – 1470 MHz - DL band: 1475 – 1518 MHz Band number n75: - UL band: NA - DL band: 1432 – 1517 MHz Band number n76: - UL band: NA - DL band: 1427 – 1432 MHz Band number n77: - UL band: 3.3 – 4.2 GHz - DL band: 3.3 – 4.2 GHz Band number n78: - UL band: 3.3 – 3.8 GHz - DL band: 3.3 – 3.8 GHz Band number n79: - UL band: 4.4 – 5 GHz - DL band: 4.4 – 5 GHz Band number n80: - UL band: 1710 – 1785 MHz - DL band: NA Band number n81: - UL band: 880 – 915 MHz - DL band: NA Band number n82: - UL band: 832 – 862 MHz - DL band: NA Band number n83: - UL band: 703 – 748 MHz - DL band: NA Band number n84: - UL band: 1920 – 1980 MHz - DL band: NA Band number n85: - UL band: 2496 – 2960 MHz - DL band: NA - 24250-52600Mhz: Band number n257: - UL band: 26.5 – 29.5 GHz - DL band: 26.5 – 29.5 GHz Band number n258: - UL band: 24.25 – 27.5 GHz - DL band: 24.25 – 27.5 GHz Band number n260: - UL band: 37 – 40 GHz - DL band: 37 – 40 GHz | For a DECT carrier the center frequency is a multiple of 1728 MHz. Carriers are specified for the frequency range 1 880 MHz to 2 025 MHz, 902 MHz to 928 MHz and 2 400 MHz to 2 483.5 MHz. Additional frequency bands at higher frequencies have been identified and can be defined as needed. | - 450-6000Mhz: Band number n1: - UL band: 1920 – 1980 MHz - DL band: 2110 – 2170 MHz Band number n2: - UL band: 1850 – 1910 MHz - DL band: 1930 – 1990 MHz Band number n3: - UL band: 1710 – 1785 MHz - DL band: 1805 – 1880 MHz Band number n5: - UL band: 824 – 849 MHz - DL band: 869 – 894 MHz Band number n7: - UL band: 2500 – 2570 MHz - DL band: 2620 – 2690 MHz Band number n8: - UL band: 880 – 915 MHz - DL band: 925 – 960 MHz Band number n20: - UL band: 832 – 862 MHz - DL band: 791 – 821 MHz Band number n28: - UL band: 703 – 748 MHz - DL band: 758 – 803 MHz Band number n38: - UL band: 2570 – 2620 MHz - DL band: 2570 – 2620 MHz Band number n40: - UL band: 2300 – 2400 MHz - DL band: 2300 – 2400 MHz Band number n41: - UL band: 2496 – 2690 MHz - DL band: 2496 – 2690 MHz Band number n50: - UL band: 1432 – 1517 MHz - DL band: 1432 – 1517 MHz Band number n51: - UL band: 1427 – 1432 MHz - DL band: 1427 – 1432 MHz Band number n66: - UL band: 1710 – 1780 MHz - DL band: 2110 – 2200 MHz Band number n70: - UL band: 1695 – 1710 MHz - DL band: 1995 – 2020 MHz Band number n71: - UL band: 663 – 698 MHz - DL band: 617 – 652 MHz Band number n74: - UL band: 1427 – 1470 MHz - DL band: 1475 – 1518 MHz Band number n75: - UL band: NA - DL band: 1432 – 1517 MHz Band number n76: - UL band: NA - DL band: 1427 – 1432 MHz Band number n77: - UL band: 3.3 – 4.2 GHz - DL band: 3.3 – 4.2 GHz Band number n78: - UL band: 3.3 – 3.8 GHz - DL band: 3.3 – 3.8 GHz Band number n79: - UL band: 4.4 – 5 GHz - DL band: 4.4 – 5 GHz Band number n80: - UL band: 1710 – 1785 MHz - DL band: NA Band number n81: - UL band: 880 – 915 MHz - DL band: NA Band number n82: - UL band: 832 – 862 MHz - DL band: NA Band number n83: - UL band: 703 – 748 MHz - DL band: NA Band number n84: - UL band: 1920 – 1980 MHz - DL band: NA Band number n85: - UL band: 2496 – 2960 MHz - DL band: NA - 24250-52600Mhz: Band number n257: - UL band: 26.5 – 29.5 GHz - DL band: 26.5 – 29.5 GHz Band number n258: - UL band: 24.25 – 27.5 GHz - DL band: 24.25 – 27.5 GHz Band number n260: - UL band: 37 – 40 GHz - DL band: 37 – 40 GHz |
| Minimum amount of spectrum required to deploy a contiguous network, including guardbands (MHz)? | - The minimum amount of paired spectrum is 2 x 5 MHz. - The minimum amount of unpaired spectrum is 5 MHz. | - The minimum amount of spectrum to support contiguous carrier aggregation for: - low frequency: 10 (2*5) MHz - high frequency : 100 (2*50) MHz. - Guard band between two component carriers is not required. | - The minimum amount of paired spectrum is 2 x 5 MHz. The minimum amount of unpaired spectrum is 5 MHz. | - Minimum practical spectrum for a contiguous network is assumed to be 10 MHz. - Operation over 5 MHz may be possible with certain restrictions. - In EU region, 20 MHz are assumed to be available as minimum spectrum | - The minimum amount of paired spectrum is 2 x 5 MHz. The minimum amount of unpaired spectrum is 5 MHz.. |
| Minimum and maximum transmission bandwidth (MHz) measured at the 3 dB down points | - The 3 dB bandwidth is not part of the specifications, however: - The minimum 99% channel bandwidth (occupied bandwidth of single component carrier) Is: - 5 MHz for frequency range 450 – 6000 MHz; - 50 MHz for frequency range 24250 – 52600 MHz - The maximum 99% channel bandwidth (occupied bandwidth of single component carrier) is: -100 MHz for frequency range 450 – 6000 MHz. - 400 MHz for frequency range 24250 – 52600 MHz. - Multiple component carriers can be aggregated to achieve up to 6.4 GHz of transmission bandwidth | - Assuming the 3 dB bandwidth roughly equals the 99% channel bandwidth occupied by a single component carrier, the minimum 3 dB bandwidth is : - low frequency: roughly 5 or 10 MHz - for high frequency: 50 MHz - the maximum 3 dB bandwidth is: - low frequency: roughly 100 MHz - for high frequency: 400 MHz | - The 3 dB bandwidth is not part of the specifications, however: - The minimum 99% channel bandwidth (occupied bandwidth of single component carrier) Is: - 5 MHz for frequency range 450 – 6000 MHz; - 50 MHz for frequency range 24250 – 52600 MHz - The maximum 99% channel bandwidth (occupied bandwidth of single component carrier) is: -100 MHz for frequency range 450 – 6000 MHz. - 400 MHz for frequency range 24250 – 52600 MHz. Multiple component carriers can be aggregated to achieve up to 6.4 GHz of transmission bandwidth | - Minimum transmission bandwidth is about 1.512 MHz. - Maximum transmission bandwidth if : - sub-carrier spacing Δf=27 Khz then 27 MHz - Δf=216 kHz Khz about 216 MHz. | - The 3 dB bandwidth is not part of the specifications, however: - The minimum 99% channel bandwidth (occupied bandwidth of single component carrier) Is: - 5 MHz for frequency range 450 – 6000 MHz; - 50 MHz for frequency range 24250 – 52600 MHz - The maximum 99% channel bandwidth (occupied bandwidth of single component carrier) is: -100 MHz for frequency range 450 – 6000 MHz. - 400 MHz for frequency range 24250 – 52600 MHz. Multiple component carriers can be aggregated to achieve up to 6.4 GHz of transmission bandwidth |
| Duplexing schemes | - NR supports paired and unpaired spectrum and allows : - FDD operation on a paired spectrum, different transmission directions in either part of a paired spectrum. - TDD operation on an unpaired spectrum where the transmission direction of time resources is not dynamically changed, and TDD operation on an unpaired spectrum where the transmission direction of most time resources can be dynamically changing. - DL and UL transmission directions for data can be dynamically assigned on a per-slot basis. - For FDD operation, it supports full-duplex FDD. - For both base station and terminal, a duplexer is needed for full-duplex FDD. - For full-duplex FDD, the required transmit/receive isolation is a UE function of: the Tx emission mask (emission level on the Rx frequency) , the TX-Rx frequency spacing , the Tx- Rx duplex filter isolation, the TX and RX configuration (RB location, RB power and RB allocation) and the required Rx desense criteria. - For the supported operating bands, the parameters including the minimum (up/down) Tx to Rx frequency separation and the minimum Tx-Rx band gap are being defined in 3GPP. - For different transmission directions in either part of a paired spectrum, a duplexer is needed for both base station and the terminal. The required frequency separation between the paired spectrum is the same as full-duplex FDD. The supported DL/UL resource assignment configurations for TDD can be applied. - For TDD operation, it supports variable DL/UL resource assignment ranging in a radio frame from 10/0 (ten downlink slots and no uplink slot) to 0/10 (no downlink slot and ten uplink slots). It also supports a slot with DL part and UL part. DL and UL transmission directions for data can be dynamically assigned on a per-slot basis. Adjacent cells using the same carrier frequency can use the same or different DL/UL resource assignment configuration. - For both the base station and the terminal, duplexer is not needed. - The TDD guard time is configurable to meet different deployment scenarios. | - The RIT supports full-duplex/half-duplex FDD, which operate on a paired spectrum - For the base station, a duplexer is needed for half-duplex/full-duplex FDD. - For the terminal, a duplexer is needed for full-duplex FDD only. - The RIT also supports TDD which operates on an unpaired spectrum. - For both the base station and the terminal, a RF switch is needed for TDD. - For FDD, the minimum Tx-Rx frequency separation is 41 MHz. - The minimum Tx-Rx band gap is 10 MHz. - For TDD, dynamic and semi-static DL/UL ratio adaptation with various symbol-level and slot-level DL/UL ratios is supported. Cross-link interference mitigation schemes are supported to enable different DL/UL configurations among adjacent cells. | - NR supports paired and unpaired spectrum and allows : - FDD operation on a paired spectrum, different transmission directions in either part of a paired spectrum. - TDD operation on an unpaired spectrum where the transmission direction of time resources is not dynamically changed, and TDD operation on an unpaired spectrum where the transmission direction of most time resources can be dynamically changing. - DL and UL transmission directions for data can be dynamically assigned on a per-slot basis. - For FDD operation, it supports full-duplex FDD. - For both base station and terminal, a duplexer is needed for full-duplex FDD. - For full-duplex FDD, the required transmit/receive isolation is a UE function of: the Tx emission mask (emission level on the Rx frequency) , the TX-Rx frequency spacing , the Tx- Rx duplex filter isolation, the TX and RX configuration (RB location, RB power and RB allocation) and the required Rx desense criteria. - For the supported operating bands, the parameters including the minimum (up/down) Tx to Rx frequency separation and the minimum Tx-Rx band gap are being defined in 3GPP. - For different transmission directions in either part of a paired spectrum, a duplexer is needed for both base station and the terminal. The required frequency separation between the paired spectrum is the same as full-duplex FDD. The supported DL/UL resource assignment configurations for TDD can be applied. For TDD operation, it supports variable DL/UL resource assignment ranging in a radio frame from 10/0 (ten downlink slots and no uplink slot) to 0/10 (no downlink slot and ten uplink slots). It also supports a slot with DL part and UL part. DL and UL transmission directions for data can be dynamically assigned on a per-slot basis. Adjacent cells using the same carrier frequency can use the same or different DL/UL resource assignment configuration. - For both the base station and the terminal, duplexer is not needed. - The TDD guard time is configurable to meet different deployment scenarios. | - The proposed RIT utilizes both TDD and FDD on contiguous or non-contiguous frequency segments. - Frequency segments and time slots are allocated to PP devices for DL and UL transmissions by the FP device. - FP devices can transmit (DL) or receive (UL) on contiguous or non-contiguous frequency segments. - PP devices can transmit (DL) or receive (UL) on contiguous frequency segments only. - At each slot time and across all allocated segments, the FP device can be either in UL mode or in DL mode but not both. | - NR supports paired and unpaired spectrum and allows : - FDD operation on a paired spectrum, different transmission directions in either part of a paired spectrum. - TDD operation on an unpaired spectrum where the transmission direction of time resources is not dynamically changed, and TDD operation on an unpaired spectrum where the transmission direction of most time resources can be dynamically changing. - DL and UL transmission directions for data can be dynamically assigned on a per-slot basis. - For FDD operation, it supports full-duplex FDD. - For both base station and terminal, a duplexer is needed for full-duplex FDD. - For full-duplex FDD, the required transmit/receive isolation is a UE function of: the Tx emission mask (emission level on the Rx frequency) , the TX-Rx frequency spacing , the Tx- Rx duplex filter isolation, the TX and RX configuration (RB location, RB power and RB allocation) and the required Rx desense criteria. - For the supported operating bands, the parameters including the minimum (up/down) Tx to Rx frequency separation and the minimum Tx-Rx band gap are being defined in 3GPP. - For different transmission directions in either part of a paired spectrum, a duplexer is needed for both base station and the terminal. The required frequency separation between the paired spectrum is the same as full-duplex FDD. The supported DL/UL resource assignment configurations for TDD can be applied. For TDD operation, it supports variable DL/UL resource assignment ranging in a radio frame from 10/0 (ten downlink slots and no uplink slot) to 0/10 (no downlink slot and ten uplink slots). It also supports a slot with DL part and UL part. DL and UL transmission directions for data can be dynamically assigned on a per-slot basis. Adjacent cells using the same carrier frequency can use the same or different DL/UL resource assignment configuration. - For both the base station and the terminal, duplexer is not needed. - The TDD guard time is configurable to meet different deployment scenarios. |
| Support of Advanced antenna capabilities and spatial multiplexing | - The multi-antenna systems in NR supports the following MIMO transmission schemes at both the UE and the base station: - Spatial multiplexing with DM-RS based closed loop, open loop and semi-open loop transmission schemes are supported. Both codebook and non-codebook based transmission is supported in DL and UL. - Spatial transmit diversity is supported by using specification transparent diversity schemes. - Hybrid beamforming including both digital and analog beamforming is supported. Beam management with periodic and aperiodic beam refinement is also supported. - NR supports -DL: 1,2,4,8,12,16,24,32 antenna port -UL: 1,2,4 antenna port - Base Station and UE support rectangular antenna arrays. - The rectangular panel array antenna can be described by the following tuple (Mg,Ng,M,N,P), where Mg is the number of panels in a column, Ng is the number of panels in row, 𝑀,𝑁 are the number of vertical, horizontal antenna elements within a panel and 𝑃 is number of polarizations per antenna element. (see figure in page 60) - The spacing in vertical and horizontal dimensions between the panels is specified by dg,V, dg,H and between antenna elements by dV, dH. - NR specification is flexible to support various antenna spacing, number of antenna elements, antenna port layouts and antenna virtualization approaches. - In NR, special multiplexing is supported: - in DL: Single codeword is supported for 1-4 layer transmissions and two codewords are supported for 5-8 layer transmissions. - in UL Only single codeword is supported for 1- 4 layer transmissions. - Both open and closed loop MIMO are supported in NR, where for demodulation of data, receiver does not require knowledge of the precoding matrix used at the transmitter. - Dynamic switching between different transmission schemes is also supported - Both single user and multi user MIMO are supported. - Single user MIMO transmissions: - DL: up to 8 orthogonal DM-RS ports are supported - UL : up to 4 orthogonal DMRS ports are supported. - Multi-user MIMO up to 12 orthogonal DM-RS ports with up to 4 orthogonal ports per UE are supported. - NR supports coordinated multipoint transmission/reception, which could be used to implement different forms of cooperative multi-antenna (MIMO) transmission schemes. | UL: Two transmission schemes are supported for PUSCH: - codebook based transmission: the UE determines its PUSCH transmission precoder based on SRI, TRI, and TPMI fields from the DCI where the TPMI is used to indicate the preferred precoder over the SRS ports in the selected SRS resource by the SRI when multiple SRS resources are configured. If a single SRS resource is configured it is used to indicate the preferred precoder over the SRS ports. The transmission precoder is selected from the uplink codebook, as defined in sub-clause 6.3.1.5 of 3GPP TS 38.211. - non-codebook based transmission: the UE determines its PUSCH precoder based on wideband SRI field from the DCI. The UE shall use multiple SRS resources with multiple SRS ports for SRS transmission. One SRS port per SRS resource is configured. DL: - For transmission scheme 1 of the PDSCH, the UE may assume that a gNB transmission on the PDSCH would be performed with up to 8 transmission layers on antenna ports as defined in sub-clause 7.3.1.4 of 3GPP TS 38.211. - For the multi-user MIMO transmission of the PDSCH, the UE may assume that a gNB transmission on the PDSCH would be performed with up to 4 layers on antenna ports as defined in subclause 7.3.1.4 of 3GPP TS 38.211. | - The multi-antenna systems in NR supports the following MIMO transmission schemes at both the UE and the base station: - Spatial multiplexing with DM-RS based closed loop, open loop and semi-open loop transmission schemes are supported. Both codebook and non-codebook based transmission is supported in DL and UL. - Spatial transmit diversity is supported by using specification transparent diversity schemes. - Hybrid beamforming including both digital and analog beamforming is supported. Beam management with periodic and aperiodic beam refinement is also supported. - NR supports -DL: 1,2,4,8,12,16,24,32 antenna port -UL: 1,2,4 antenna port - Base Station and UE support rectangular antenna arrays. - The rectangular panel array antenna can be described by the following tuple (Mg,Ng,M,N,P), where Mg is the number of panels in a column, Ng is the number of panels in row, 𝑀,𝑁 are the number of vertical, horizontal antenna elements within a panel and 𝑃 is number of polarizations per antenna element. (see figure in page 60) - The spacing in vertical and horizontal dimensions between the panels is specified by dg,V, dg,H and between antenna elements by dV, dH. - NR specification is flexible to support various antenna spacing, number of antenna elements, antenna port layouts and antenna virtualization approaches. - In NR, special multiplexing is supported: - in DL: Single codeword is supported for 1-4 layer transmissions and two codewords are supported for 5-8 layer transmissions. - in UL Only single codeword is supported for 1- 4 layer transmissions. - Both open and closed loop MIMO are supported in NR, where for demodulation of data, receiver does not require knowledge of the precoding matrix used at the transmitter. - Dynamic switching between different transmission schemes is also supported - Both single user and multi user MIMO are supported. - Single user MIMO transmissions: - DL: up to 8 orthogonal DM-RS ports are supported - UL : up to 4 orthogonal DMRS ports are supported. - Multi-user MIMO up to 12 orthogonal DM-RS ports with up to 4 orthogonal ports per UE are supported. NR supports coordinated multipoint transmission/reception, which could be used to implement different forms of cooperative multi-antenna (MIMO) transmission schemes. | - The proposed RIT supports multiplexing of NSTS ≤ 6 space-time streams with 6 × 6 antenna configuration - (NTX = NRX = 6). NTX> NSTS and/or NRX> NSTS can be configured to increase diversity and/or to enable digital beamforming. Space-time Block Coding (STBC) is also supported (2 × 1 configuration). - Both FP and PP can be equipped with NTX,NRX> NSTS antennas where NSTS ≤ 6. - Target antenna spacing is λ/4, where λ is the wavelength of the carrier signal. - Antenna range supported by test models is 1 ≤NTX ,NRX ≤ 6 - The proposed RIT supports: - Open and closed loop MIMO - Single-user MIMO and multi-user MIMO | - The multi-antenna systems in NR supports the following MIMO transmission schemes at both the UE and the base station: - Spatial multiplexing with DM-RS based closed loop, open loop and semi-open loop transmission schemes are supported. Both codebook and non-codebook based transmission is supported in DL and UL. - Spatial transmit diversity is supported by using specification transparent diversity schemes. - Hybrid beamforming including both digital and analog beamforming is supported. Beam management with periodic and aperiodic beam refinement is also supported. - NR supports -DL: 1,2,4,8,12,16,24,32 antenna port -UL: 1,2,4 antenna port - Base Station and UE support rectangular antenna arrays. - The rectangular panel array antenna can be described by the following tuple (Mg,Ng,M,N,P), where Mg is the number of panels in a column, Ng is the number of panels in row, 𝑀,𝑁 are the number of vertical, horizontal antenna elements within a panel and 𝑃 is number of polarizations per antenna element. (see figure in page 60) - The spacing in vertical and horizontal dimensions between the panels is specified by dg,V, dg,H and between antenna elements by dV, dH. - NR specification is flexible to support various antenna spacing, number of antenna elements, antenna port layouts and antenna virtualization approaches. - In NR, special multiplexing is supported: - in DL: Single codeword is supported for 1-4 layer transmissions and two codewords are supported for 5-8 layer transmissions. - in UL Only single codeword is supported for 1- 4 layer transmissions. - Both open and closed loop MIMO are supported in NR, where for demodulation of data, receiver does not require knowledge of the precoding matrix used at the transmitter. - Dynamic switching between different transmission schemes is also supported - Both single user and multi user MIMO are supported. - Single user MIMO transmissions: - DL: up to 8 orthogonal DM-RS ports are supported - UL : up to 4 orthogonal DMRS ports are supported. - Multi-user MIMO up to 12 orthogonal DM-RS ports with up to 4 orthogonal ports per UE are supported. NR supports coordinated multipoint transmission/reception, which could be used to implement different forms of cooperative multi-antenna (MIMO) transmission schemes. |
| Synchronization requirements | Common general aspects - Tight BS-to-BS synchronization not required. - Tight BS-to-network synchronization not required. - BS support a logical synchronization port for phase-, time- and/or frequency synchronization. (read more page 68) - Common SFN initialization time shall be provided for all BSs in synchronized TDD area. - A certain RAN-CN Hyper SFN synchronization is required in case of extended Idle mode DRX. Some accuracy requirements - BS transmit signals accuracy. - Cell phase synchronization accuracy: - The cell phase synchronization accuracy measured at BS antenna connectors shall be better than [3] μs. the synchronization mechanisms A UE receives the following synchronization signals (SS) in order to perform cell search: the primary synchronization signal (PSS) and secondary synchronization signal (SSS). - PSS used for initial symbol boundary, cyclic prefix, sub frame boundary, initial frequency synchronization to the cell. - SSS is used for radio frame boundary identification. - PSS and SSS together used for cell ID detection. Other synchronization mechanisms are defined see more info in [38.213] subclause 4 and [38.211] subclause 7.4.2 | - Tight BS-to-BS synchronization not required. However, in some scenarios, system performance may have gains from tight BS-to-BS synchronization. - BS-to-network synchronization not required. - Frequency and timing accuracy of base station transmit signal is within ±0.05 ppm observed over a period of one slot. the synchronization mechanisms The RIT may support different kind of methods and techniques to satisfy the synchronization requirements. A logical port at the base station is defined to support the reception of timing and/or frequency and/or phase inputs according to the synchronization method chosen. | Common general aspects - Tight BS-to-BS synchronization not required. - Tight BS-to-network synchronization not required. - BS support a logical synchronization port for phase-, time- and/or frequency synchronization. (read more page 68) - Common SFN initialization time shall be provided for all BSs in synchronized TDD area. - A certain RAN-CN Hyper SFN synchronization is required in case of extended Idle mode DRX. Some accuracy requirements - BS transmit signals accuracy. - Cell phase synchronization accuracy: - The cell phase synchronization accuracy measured at BS antenna connectors shall be better than [3] μs. the synchronization mechanisms A UE receives the following synchronization signals (SS) in order to perform cell search: the primary synchronization signal (PSS) and secondary synchronization signal (SSS). - PSS used for initial symbol boundary, cyclic prefix, sub frame boundary, initial frequency synchronization to the cell. - SSS is used for radio frame boundary identification. - PSS and SSS together used for cell ID detection. Other synchronization mechanisms are defined see more info in [38.213] subclause 4 and [38.211] subclause 7.4.2 | - Two synchronization mechanisms: Packet level synchronization and Frame synchronization. - Packet level synchronization: each packet contains a synchronization pattern. See figure 2 and 3 page 17 - Frame synchronization: see figure page 18 | Common general aspects - Tight BS-to-BS synchronization not required. - Tight BS-to-network synchronization not required. - BS support a logical synchronization port for phase-, time- and/or frequency synchronization. (read more page 68) - Common SFN initialization time shall be provided for all BSs in synchronized TDD area. - A certain RAN-CN Hyper SFN synchronization is required in case of extended Idle mode DRX. Some accuracy requirements - BS transmit signals accuracy. - Cell phase synchronization accuracy: - The cell phase synchronization accuracy measured at BS antenna connectors shall be better than [3] μs. the synchronization mechanisms A UE receives the following synchronization signals (SS) in order to perform cell search: the primary synchronization signal (PSS) and secondary synchronization signal (SSS). - PSS used for initial symbol boundary, cyclic prefix, sub frame boundary, initial frequency synchronization to the cell. - SSS is used for radio frame boundary identification. - PSS and SSS together used for cell ID detection. Other synchronization mechanisms are defined see more info in [38.213] subclause 4 and [38.211] subclause 7.4.2 |

- Conclusion

In this report, we evaluated 3GPP in terms of ASE and 5% USE for downlink. Some issues were found in the uplink simulations. Work is in progress to solve them and generate new mobility results.

We found that China and TSDSI candidates advocate exactly the very same configurations as 3GPP. Hence, their evaluation results by simulations should be exactly the same. However, we need some “double-check” for confirmation (?). On the other hand, work is in progress to finish adjustments of the simulator to account for different features specific to ETSI and Korea candidates.

- References

[1] ITU-R M.2412 IMT-2020.EVAL

[2] IMT-2020/ZZZ, Initial evaluation Report from TPCEG on the IMT-2020 proposal in Documents IMT-2020/3 (Rev. 1), TPCEG, September 09, 2018