IMT-2020: LMLC Link Budget analysis for TSDSI IMT-2020 submission (NR) – September 10th update Document for discussion CEG meeting Montreal, Canada September 12th, 2019 Source : Interdigital Inc.

# Abbreviations

3GPP: 3rd Generation Partnership Project

AP: Access point

BS: Base-station

BW: Bandwidth

CCE: Control channel element

CP: Control-plane or Cyclic Prefix

CSI: Channel state information

CSI-RS: Channel state information reference signal

DU: Dense Urban

DL: Downlink

DMRS: Demodulation reference signal

eMBB: enhanced mobile broadband

FDM: Frequency division multiplexing

gNB: g-NodeB

GoS: Grade-of-service

HARQ: Hybrid automatic repeat request

InH: Indoor Hotspot

ITU: International Telecommunication Union

LDPC: Low-density parity code

LMLC: low-mobility large-cell

LoS: Line-of-sight

MAC: Medium access control

MIMO: multiple-input multiple-output

mMTC: massive Machine-Type Communications

NB-IoT: Narrowband-Internet of Things

NLoS: non-Line-of-sight

NR: New Radio

OFDM: Orthogonal frequency division multiplexing

PDCCH: Physical downlink control channel, PDSCH: Physical downlink shared channel

PDCP: Packet data convergence protocol

PRB: Physical resource block

PT-RS: Phase tracking reference signal

PUCCH: Physical uplink control channel; PUSCH: Physical uplink shared channel

QoS: Quality-of-service

RAN: Radio access network

RB: Resource block

RE: Resource element

RIT: Radio-interface technology

RLC: Radio link control

RU: Rural

SCS: Sub-carrier spacing

SDU: Service data unit

SINR/SNR: Signal-to-interference noise ratio/Signal-to-noise ratio

SR: Scheduling request

SRS: Sounding reference symbol

SSB: Synchronization signal block

SCM: Stochastic channel model

TBS: Transport block size

TRxP: Transmission and reception point

TRS: Tracking reference signal

TTI: Transmission time interval

UE: User equipment

UL: Uplink

UP: User-plane

urLLC: ultra-reliable low-latency communications

# Introduction

In this contribution, we analyze the link budget template of the TSDSI submission in [1] received by WP5D in July meeting. The main delta with the 3GPP submission is the π/2-BPSK modulation used for the LMLC scenario. So, we will concentrate our analysis on this scenario.

# Discussion

The M.2412 document specify precisely how to evaluate the LMLC scenario in terms of deployment (ISD=6000m), UE speed, and then pathloss calculation, for both channel models A and B respectively.

We will take the 700MHz scenario DSUUD as an example without going any further in the link budget template.

It is useful to compare it with eMBB Rural in 700Mhz from 3GPP submission that we already validated and considered it correct in our previous evaluations.

Let’s analyse further the TSDSI LMLC differences and their validity.

## Spectrum Efficiency (SE), data channel power boosting and the proposed SNR value for UL data channel

TSDSI simply used the same SE=0.36 as 3GPP for the UL data channel and then they claim that PUSCH power can be boosted to 26dBm as the π/2 BPSK modulation is used.

Based on our link level simulation for SNR derivation/verification of the 3GPP evaluation of Rural eMBB, we found out the MCS index 3 must be used in order to reach SE=0.36. MCS index 3 imply that QPSK modulation must be used.

Observation1: the SE= 0.36 cannot be achieved with the current assumptions using

π/2-BPSK modulation.

Observation 2: We believe that a generic 23dBm output power shall be assumed for both control and data channels for the evaluation process, in order to have a consistent overall approach between all proponents. This has been fixed in the September update.

Observation 3: TSDSI LMLC link budget uses for the NLOS scenario a UE speed of 120km/h while LMLC description in M.2412 suggest a UE speed of 30km/h.

Observation 4: A correct SE value shall be derived using the appropriate simulations against a typical LMLC deployment as described in M.2412.

Observation 5: Since the SNR value suggested for the PUSCH correspond to the eMBB Rural where the UE speed is considered 120Km/h, we believe that a correct SNR value shall be derived using the appropriate channel delay spread specified for a 30km/h and the appropriate SE derived under LMLC scenario.

For convenience we are reproducing here the deployments description from M.2412:

| Parameters | Rural-eMBB |  |  |
| --- | --- | --- | --- |
|  | Spectral Efficiency and Mobility Evaluations |  | Average Spectral Efficiency Evaluation |
|  | Configuration A | Configuration B | Configuration C (LMLC) |
| Device deployment | 50% indoor, 50% outdoor (in-car) Randomly and uniformly distributed over the area | 50% indoor, 50% outdoor (in-car) Randomly and uniformly distributed over the area | 40% indoor, 40% outdoor (pedestrian), 20% outdoor (in-car) Randomly and uniformly distributed over the area |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction | Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction | Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction |
| UE speeds of interest | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h; 500 km/h for evaluation of mobility in high-speed case | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h; 500 km/h for evaluation of mobility in high-speed case | Indoor users: 3 km/h; Outdoor users (pedestrian): 3 km/h; Outdoor users (in-car): 30 km/h |
| Inter-site interference modeling | Explicitly modelled | Explicitly modelled | Explicitly modelled |
| BS noise figure | 5 dB | 5 dB | 5 dB |
| UE noise figure | 7 dB | 7 dB | 7 dB |
| BS antenna element gain | 8 dBi | 8 dBi | 8 dBi |
| UE antenna element gain | 0 dBi | 0 dBi | 0 dBi |
| Thermal noise level | ‒174 dBm/Hz | ‒174 dBm/Hz | ‒174 dBm/Hz |
| Traffic model | Full buffer | Full buffer | Full buffer |
| Simulation bandwidth | 20 MHz for TDD, 10 MHz+10 MHz for FDD | 20 MHz for TDD, 10 MHz+10 MHz for FDD | 20 MHz for TDD, 10 MHz+10 MHz for FDD |
| UE density | 10 UEs per TRxP Randomly and uniformly distributed over the area | 10 UEs per TRxP Randomly and uniformly distributed over the area | 10 UEs per TRxP Randomly and uniformly distributed over the area |
| UE antenna height | 1.5 m | 1.5 m | 1.5 m |

## Pathloss calculation, cell range and cell area availability

We noticed that for the data channel 12dB gain has been employed in the link budget template for LMLC as follows:

10^(3+(F63-161.04+12+7.1*LOG10(20)-7.5*LOG10(5)+(24.37-3.7*(5/F$5)^2)*LOG10(F$5)-20*LOG10(F$4)+(3.2*(LOG10(11.75*F$6))^2-4.97))/(43.42-3.1*LOG10(F$5)))

Also, a another 3dB has been added at the beginning compared to the 3GPP Rural case:

10^((F62-161.04+7.1*LOG10(20)-7.5*LOG10(5)+(24.37-3.7*(5/F$5)^2)*LOG10(F$5)-20*LOG10(F$4)+(3.2*(LOG10(11.75*F$6)^2)-4.97))/(43.42-3.1*LOG10(F$5))+3)

While a 12dB gain is possible, this may not be always the case. We would like to see both LOS and NLOS calculations and consequently the result, as suggested by M.2412 procedure for LMLC that we are highlighting in yellow for convenience for both channel model A and B respectively:

RMa_A. LOS. , , see Note 4 , dB , dB. NLOS. , PLRMa-NLOS = 161.04 – 7.1 log10 (W) + 7.5 log10 (h) – (24.37 – 3.7(h/hBS)2) log10 (hBS) + (43.42 – 3.1 log10 (hBS)) (log10 (d3D)-3) + 20 log10(fc) – (3.2 (log10 (11.75 hUT)) 2 - 4.97), dB for 10 m < d2D < 21 km. For LMLC. , , , The applicability ranges: ,, ,.

RMa_B. LOS. , see Note 4 , dB , dB. NLOS. , , for 10 m < d2D < 21 km ,dB For LMLC. , , , The applicability ranges: ,, , NOTE – the RMa pathloss model for >7 GHz is validated based on a single measurement campaign conducted at 24 GHz..

Observation 6: The pathloss calculation is not following the LMLC M.2412 suggested procedure.

We noticed also that the control channel PUCCH pathloss follows the Rural eMBB rule, resulting into at least 12dB difference compared to data channel in LMLC scenario. This has been fixed in the September update.

Observation 7: The control channel PUCCH pathloss calculation follows the Rural eMBB rule, resulting into at least 12dB worst compared to data channel. This is not anymore the case in the September update.

Observation 8: Based on the above multiple issues the cell range and the cell are coverage may be different as there are inconsistencies and probably wrong values for SE, SNR and possible pathloss.

# Summary and conclusion

In this contribution, it has been shown that the IMT-2020 submission from TSDSI contains some clear errors and inconsistencies for the LMLC link budget template, which lead to invalid link budget results.

References

- R15-WP5D-C-1231 WP-5D meeting, Brazil

- ITU-R M.2410 IMT-2020.TECH PERF REQ

- ITU-R M.2412 IMT-2020.EVAL