# Abbreviations

3GPP: 3rd Generation Partnership Project

BW: Bandwidth

CCE: Control channel element

CSI-RS: Channel state information reference signal

DL: Downlink

DM-RS: Demodulation reference signal

ITU: International Telecommunication Union

NR: New Radio

PDCCH: Physical downlink control channel

PRB: Physical resource block

PT-RS: Phase tracking reference signal

PUCCH: Physical uplink control channel

RAN: Radio access network

RE: Resource element

SCS: Sub-carrier spacing

SRS: Sounding reference symbol

SSB: Synchronization signal block

TRS: Tracking reference signal

UL: Uplink

# Introduction

In this paper, Ericsson demonstrates the evaluation results of NR with respect to peak data-rate, peak spectrum efficiency, and bandwidth. The ITU and 3GPP targets for these metrics as far as NR is concerned, are summarized in Table 1.

| Performance Measure | ITU | 3GPP |
| --- | --- | --- |
| Peak data rate | DL: 20 Gb/s UL: 10 Gb/s | DL: 20 Gb/s UL: 10 Gb/s |
| Peak spectral efficiency | DL: 30 bps/Hz UL: 15 bps/Hz | DL: 30 bps/Hz UL: 15 bps/Hz |
| Bandwidth | At least 100MHz, up to 1GHz for higher frequency bands | Reference to ITU |

Table 1: ITU and 3GPP targets on peak data rate, peak spectral efficiency, and bandwidth

Peak data-rate and peak spectral efficiency are to be evaluated through analysis, while bandwidth by inspection, as explained in § 6 of ITU-R Report M.2412 [1].

# Bandwidth

3GPP RAN4 agreed on the spectrum utilisation numbers listed in Table 2 and Table 3 for below and above 6 GHz, respectively [2].

|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 kHz | 25 | 52 | 79 | 106 | 133 | 216 | 270 | NA | NA | NA |
| 30 kHz | 11 | 24 | 38 | 51 | 65 | 106 | 133 | 162 | 217 | 273 |
| 60 kHz | NA | 11 | 18 | 24 | 31 | 51 | 65 | 79 | 107 | 135 |

SCS

BW

Table 2: Maximum spectrum utilization (in no. of PRBs) for various bandwidth/subcarrier spacing combinations for below 6 GHz (BW in MHz, SCS in kHz)

BW

| SCS | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 kHz | 66 | 132 | 264 | NA |
| 120 kHz | 32 | 66 | 132 | 264 |

Table 3: Maximum spectrum utilization (in no. of PRBs) for various bandwidth/subcarrier spacing combinations for above 6 GHz (BW in MHz, SCS in kHz)

One can see from Table 2 and Table 3 that several configurations support bandwidths of 100 MHz (and above). The largest component carrier bandwidth is 400 MHz. NR supports carrier aggregation of up to 16 component carriers in which case the supported NR carrier bandwidth exceeds 1 GHz.

For example, in Table 2, row 2 has 273 PRBs. Each resource block has 12 carriers and each carrier, in turn, is 30 kHz, yielding a carrier bandwidth of 273*12*30 = 98.28 MHz. In the same table, row 3 yields a carrier bandwidth of 135*12*60 = 97.20 MHz. (Note that guard-bands and one D.C. carrier take up the rest of the ‘space’).

In addition, NR can aggregate upto 16 such component carriers which means that other configurations could also potentially provide the requisite ITU bandwidth.

In Table 3, 264 PRBs at 60 kHz each yields a bandwidth of 264*12*60 = 190.08 MHz. Aggregating 6 such component carriers provides a bandwidth of 1140.48 MHz – which responds to the ITU requirement of 1 GHz at higher frequencies (above 6 GHz). Other configurations are possible, meeting or exceeding the 1 GHz limit.

Therefore, Ericsson concludes that the ITU requirement of 100 MHz bandwidth for lower frequencies (below 6 GHz) and 1 GHz bandwidth for higher frequencies (above 6 GHz) is met by NR.

# Peak data-rate and peak spectral efficiency

## Downlink

When assessing the downlink peak data-rate, the overheads due to SSB, TRS, PDCCH, DM-RS, PT-RS and CSI-RS have to be considered. These are shown in Table 4. To achieve peak data rates of 20 Gbits/s, bandwidths of the order of 400 MHz are required, so the evaluation focuses on frequencies above 6 GHz.

| Parameter | Setting |
| --- | --- |
| SSB (synchronization signal block) | 8 SSBs per 20 ms |
| TRS (tracking reference signal) | Min(52, BW in PRBs) PRB wide, occurs every 20 ms |
| PDCCH (physical downlink control channel) | 4 CCE in every slot |
| DM-RS (demodulation reference signal) | 2 complete symbols per slot |
| CSI-RS (channel-state information reference signal) | 8 RE per PRB, occurs every 10 ms |
| PT-RS (phase-tracking reference signal) | 1 subcarrier every 4th PRB, every symbol |
| Number of layers | 8 |
| Modulation format | 256QAM |
| Code rate | 0.93 |

Table 4: Evaluation assumptions for peak data-rate and peak spectral efficiency on the downlink

Using the assumptions from Table 4, the achievable DL peak data-rates are shown in Table 5. This was calculated using a Matlab script, which formulation is roughly:

DRdl = repmat(Nslots/s, Nrows, size(BWSC,2))*NRE/slot*(1-OHdl)*Nlayers*Modformat*CR

where

DRdl = date-rate on the DL

B = repmat(A,m,n) creates a large matrix B consisting of an m-by-n tiling of copies of A

s = size(A) returns a row vector whose elements contain the length of the corresponding dimension of A

NRE = no of resource elements

OHdl = overhead on the DL

For more details about the formula itself, the reader is referred to [4] and [5].

For a 400 MHz wide component carrier, the peak data rate is 17.49 Gbits/s. Aggregating two such component carriers consumes a bandwidth of 800 MHz and gives a peak data-rate of about 35 Gbits/s, well beyond the passing criterion of 20 Gbits/s.

SCS

BW

|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 kHz | 2.11 | 4.32 | 8.73 | NaN |
| 120 kHz | 1.98 | 4.25 | 8.66 | 17.49 |

Table 5: Downlink peak data-rates in Gbits/s

The peak spectral efficiency is obtained by normalizing the peak data rates shown in Table 5 by the bandwidth. The resulting peak spectral efficiency values are tabulated in Table 6. All values are well above the target of 30 bits/s/Hz.

| SCS | BW 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 kHz | 42.23 | 43.16 | 43.63 | NaN |
| 120 kHz | 39.56 | 42.50 | 43.31 | 43.72 |

Table 6: Downlink peak spectral efficiency in bps/Hz

## Uplink

The evaluation parameters for the uplink are listed in Table 7. The overheads due to DM-RS, PT-RS, SRS, and PUCCH are considered. The ITU peak data rate targets are fulfilled for carrier aggregation of two 400 MHz wide component carriers, see Table 8. All shown combinations exceed the peak spectral efficiency target of 15 bps/Hz, see Table 9.

| Parameter | Setting |
| --- | --- |
| DM-RS | 1 complete symbol per slot |
| PT-RS | 1 subcarrier every 4th PRB, every symbol |
| SRS | 1 complete symbol every 10 ms |
| PUCCH | Long PUCCH with 2 PRB over slot in every slot |
| Number of layers | 4 |
| Modulation format | 256QAM |
| Code rate | 0.93 |

Table 7: Evaluation assumptions for peak data rate and peak spectral efficiency on the uplink

|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 kHz | 1.16 | 2.35 | 4.74 | NaN |
| 120 kHz | 1.08 | 2.31 | 4.71 | 9.50 |

BW

SCS

Table 8: Uplink peak data rates in Gbits/s

SCS

BW

|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 kHz | 23.12 | 23.52 | 23.72 | NaN |
| 120 kHz | 21.61 | 23.14 | 23.54 | 23.74 |

Table 9: Uplink peak spectral efficiency in bits/s/Hz

# Conclusion

In this contribution, NR was evaluated with respect to peak data-rate, peak spectral efficiency as well as bandwidth and shown to fulfil all the ITU-R targets for these technical performance requirements.

# References

ITU-R Report M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”

R4-1806932, “TS 38.104 Combined updates (NSA) from RAN4 #86bis and RAN4 #87,” Ericsson

ITU-R Report M.2411, “Requirements, evaluation criteria and submission templates for the development of IMT-2020”

R1-1809934, Summary on discussion on IMT-2020 evaluation for peak data rate and peak spectral efficiency,” Huawei

R1-1805641, “Way Forward on NR Peak Data Rate Formula,” Intel, Samsung, MediaTek, Huawei, HiSilicon, Apple, Vivo, OPPO